UNCLASSIFIED // CONCEPTUAL DESIGN STUDY // NOT FOR OPERATIONAL USE

The Optimal Arctic Gripen

Augmentation Specifications

v6.1 — Sovereignty Baseline

Canadian Arctic Operations Package

Document No. AGA-CDN-CDR80-002

Issue date: 6 September 2026

Prepared from public-domain baseline data, Canadian/allied sourcing rules, and a re-closed mass, power and sovereignty architecture.

# Document Control and Basis of Issue

| Field | Controlled value |
| --- | --- |

| Document | The Optimal Arctic Gripen Augmentation Specifications v6.1 |
| --- | --- |
| Baseline | CDR-80 conceptual design baseline |
| Configuration item | Arctic Operations Kit AOK-1 plus optional conformal fuel tank pair CFT-A2 |
| Authority for release | Program design authority; no airworthiness release is implied |
| Issue | 6.1 — sovereignty baseline; baseline propulsion retained |
| Date | 6 September 2026 |
| Language / units | English / SI with aviation units where customary |

### Scope

This specification defines an implementable Arctic operations package for a single-seat Gripen E operating from Canadian northern main operating bases and forward operating locations. It covers aircraft modifications, mission-kit interfaces, ground support, survival equipment, verification, production controls, and lifecycle support. It does not define weapons employment, tactics, classified electronic-warfare parameters, or customer airworthiness certification.

### CDR-80 meaning

CDR-80 is defined for this document as approximately eighty percent design release: requirements, architecture, interfaces, budgets, materials, production method, and verification logic are closed; items that cannot be closed without proprietary OEM data or physical testing are explicitly marked PENDING VALIDATION. A PENDING VALIDATION label is a controlled maturity statement, not an open engineering contradiction.

### Use limitation

This is an unclassified engineering concept. Public sources establish the aircraft baseline and operating context, but all new part numbers, mass deltas, and performance allocations in this specification are program assumptions. No text in this document authorizes flight, manufacture, or modification of an in-service aircraft.

CONTROLLED BASELINE: Arctic kit mass, power, geometry and environmental budgets close. Flight clearance, OEM form-fit validation and full-scale qualification remain pending where identified.

# Table of Contents

Word and compatible editors can refresh this field after opening. Page numbers below are precomputed for this issued layout.

Executive Summary	4

Program Requirements and Operational Context	5

PART I — SYSTEM ARCHITECTURE	6

PART II — AIRFRAME, MATERIALS AND COLD-SOAK PROTECTION	9

PART III — PROPULSION, FUEL AND EMISSIONS	13

PART IV — SENSORS, NAVIGATION AND COMMUNICATIONS	17

PART V — ELECTRICAL POWER AND ICE PROTECTION	21

PART VI — LANDING GEAR AND SURFACE OPERATIONS	25

PART VII — SURVIVAL, SUPPORT AND MAINTENANCE	28

PART VIII — MASS, CG AND PERFORMANCE CLOSURE	31

PART IX — RECYCLED MATERIALS	34

PART X — VERIFICATION, VALIDATION AND COMPLIANCE	35

PART XI — RISK REGISTER	38

PART XII — CONFIGURATION AND IMPLEMENTATION	40

APPENDICES	41

### Reading path

The executive summary provides the selected configuration and its closure statement. Parts I–VII define the physical and operational design. Part VIII closes mass, centre of gravity, and performance. Part IX treats recycled content. Parts X–XII provide the evidence, risk, and production path.

# Executive Summary

## Selected configuration

The recommended Arctic Gripen sovereignty configuration retains the certified Saab airframe, F414G engine, mission computer, Raven-class AESA radar, IRST, electronic-warfare suite and primary structure. Baseline Gripen E equipment, including the US-origin F414G engine, is grandfathered and is not replaced solely because of country of origin; the sovereignty sourcing rules apply only to newly introduced Arctic augmentation hardware and systems. The Arctic augmentation remains deliberately conservative: cold-soak protection, electro-thermal ice protection, Canadian anti-jam navigation, low-temperature landing-gear consumables, Arctic survival equipment, Canadian-built ground support, and an optional conformal fuel tank pair. Lower-value permanent add-ons—dedicated satellite hardware, a celestial sensor and an air-carried FOD screen—are removed from the baseline.

Saab publishes the Gripen E baseline as a 16,500 kg maximum-take-off-weight, 98 kN-thrust, ten-hardpoint aircraft with air-to-air refuelling and 15–25 minute combat turnaround.

## Principal design decisions

- No structural trunnion redesign: the existing gear is retained and qualified with low-temperature fluid, seals, grease, tires and procedure changes.

- Baseline propulsion retained: the certified F414G engine, DECMU/FADEC and engine services are unchanged, avoiding thrust, thermal, export-control and certification disruption.

- No internal weapons pod: Arctic persistence and reliability take priority over a speculative low-observable pod.

- No recycled primary structure: recycled metal is expanded into qualified secondary brackets, racks, cable raceways, non-critical shields, ground equipment and non-flight survival hardware only.

- No permanent weight or power claim beyond the revised budgets in Parts V and VIII; future Canadian polar satcom must fit the reserved allocation or trigger a new power review.

## Closure verdict

| Metric | Closed value | Closure |
| --- | --- | --- |
| Permanent Arctic installed mass delta | 232 kg | CLOSED |
| Optional CFT pair empty mass | 235 kg | CLOSED as mission kit |
| Representative Arctic QRA take-off mass | 14,537 kg | CLOSED; 1,963 kg below MTOW |
| Peak flight electrical addition | 11.40 kW including 20% margin | CLOSED as allocation; generator form-fit PENDING |
| Certified MTOW growth | 0 kg | CLOSED — no growth requested |
| Primary structure change | None | CLOSED |
| Minimum operating temperature | -55 deg C ground soak; -54 deg C hydraulic design point | Analysis closed; aircraft-level test pending |
| New Arctic-kit US-origin content | Target 0% for new augmentation hardware; exceptions <8% by new-kit value; baseline equipment grandfathered | Requirement CLOSED; BOM audit pending |

# Program Requirements and Operational Context

## Operating problem

The design mission is sustained air-defence and sovereignty response over the Canadian Arctic, with dispersed deployment, long transit legs, limited diversion options, high-latitude communications constraints, blowing snow, freezing precipitation, cold-soaked ground turns, and intermittent satellite navigation quality. The package must be useful at the planned northern infrastructure nodes without requiring a new aircraft type or a depot-only cold-weather conversion.

National Defence identifies Inuvik, Yellowknife, Iqaluit, and 5 Wing Goose Bay as the northern sites being upgraded under the NORAD Northern Basing Infrastructure initiative.

Canada is extending Inuvik’s runway from 6,000 to 9,000 ft and modernizing lighting, navigation, and the military arrestor system, with the stated aim of improving suitability for larger and heavier aircraft.

## Design drivers

| Driver | Design consequence |
| --- | --- |
| Extreme cold | -55 deg C ground survival and cold start after 12 h soak |
| Icing | Certifiable anti-/de-ice envelope with energy-limited zonal heating |
| Distance | Optional CFT pair plus retained air-to-air refuelling |
| Navigation denial | Multi-GNSS, Canadian anti-jam antenna, INS, barometric and terrain updates |
| Dispersed operations | Small GSE footprint and rapid heated turnaround |
| Sustainment | Consumables and LRUs stocked forward; depot tasks minimized |
| Supply resilience | Recycled metal only where qualification and traceability are maintained |

## Non-goals

The package does not attempt to turn Gripen E into a stealth aircraft, replace the engine, redesign the wing, increase certified MTOW, or create an all-ice runway capability. Existing baseline Gripen E equipment is grandfathered: it is not replaced solely because of country of origin, and the sourcing rules apply only to newly introduced Arctic augmentation hardware and systems.

# PART I — SYSTEM ARCHITECTURE

## 1.1 Functional architecture

Figure 1. Arctic augmentation architecture and principal functional interfaces.

The architecture separates permanent aircraft changes from role equipment. The permanent kit contains cold-soak protection, ice protection, resilient navigation, power distribution changes and landing-gear consumables. The CFT pair is a mission kit and may be omitted for short-range air-defence sorties. Ground preheat and wheel heating remain ground-power functions so aircraft battery and generator margins are not consumed before engine start.

CLOSED: Architecture closes because every function has one accountable subsystem, one power source, one thermal path, and one verification owner. Ground-only loads are excluded from the flight electrical budget.

## 1.2 Requirement set

| ID | Requirement | Target | Method | CDR-80 status |
| --- | --- | --- | --- | --- |
| AGA-KPP-01 | Cold-soak start | 12 h at -40 deg C; first-start success >=95% | Test | PENDING aircraft test |
| AGA-KPP-02 | Ground survival | -55 deg C, 72 h, no damage | Test | PENDING aircraft test |
| AGA-KPP-03 | Ice protection | Appendix O-equivalent mission analysis; inlet anti-ice retained | Test | PENDING full-scale |
| AGA-KPP-04 | Radius gain with CFT | >=25% versus defined baseline mission | Analysis + flight | PENDING flight |
| AGA-KPP-05 | GNSS-denied navigation | 4 h mission; error within mission-system limit | HIL + flight | PENDING flight |
| AGA-KPP-06 | Dispersed turnaround | Air-to-air refuel/rearm <=25 min with shelter preheat | Demonstration | PLANNED |
| AGA-KPP-07 | MTOW | No increase above 16,500 kg | Analysis | CLOSED |
| AGA-KPP-08 | Availability | No more than 10% reduction at -40 deg C versus temperate baseline | Model + trial | PENDING operational trial |
| AGA-KPP-09 | Sovereign supply | New Arctic-kit hardware target 0% US origin; baseline equipment grandfathered; exceptions <8% by new-kit value | BOM + origin audit | CLOSED as requirement |

ENGINEERING NOTE  Requirements are written as verifiable capabilities rather than supplier marketing claims. The full requirement dictionary is in Appendix A.

## 1.3 Modes, states and interfaces

## Operational modes

| Mode | Configuration | Power / thermal rule |
| --- | --- | --- |
| Sheltered alert | Canopy closed; ground power; battery and oil preheat | Aircraft generators offline |
| Cold start | Ground GPU, pre-oiler and battery heater active | Start attempt after minimum temperatures reached |
| Taxi / de-ice | Engine running; ice protection armed | Ground de-ice complete before takeoff |
| Cruise | CFT fuel first; mission systems normal | Ice zones sequenced; 6.8 kW peak |
| GNSS-denied | INS primary; terrain updates where relief permits | Canadian anti-jam active; nonessential emissions off |
| Diversion | Nearest suitable runway; surface state briefed | CNT and brake mode selected by runway condition |

## External interfaces

Arctic-kit interfaces are limited to existing electrical buses, mission-system Ethernet/fibre, mechanical hardpoints, fuel-system transfer connections, environmental-control ducting and approved antenna mounting provisions. No flight-control actuator or certified flight-control computer is modified.

CLOSED: Interface definitions close at functional level. Exact electrical connector allocation, cooling-air availability, and attach-fitting loads require OEM drawings and are therefore PENDING VALIDATION.

# PART II — AIRFRAME, MATERIALS AND COLD-SOAK PROTECTION

## 2.1 Design philosophy

The airframe is not re-winged, re-skinned, or re-sparred. The Arctic package assumes the baseline structure already accommodates fighter load spectra and focuses on the cold-specific degradation mechanisms: seal contraction, lubricant viscosity, moisture freeze, adhesive embrittlement, composite matrix microcracking, galvanic corrosion under salt-contaminated de-icing fluid, and maintainer access with gloved hands.

NATO STANAG 4370/AECTP and UK Defence Standard 00-035 are used as non-US tailoring frameworks for low temperature, thermal shock, rain, salt fog and icing/freezing rain rather than as substitutes for aircraft-specific airworthiness evidence.

## Environmental design condition

| Parameter | Design value | Basis |
| --- | --- | --- |
| Operating low temperature | -54 deg C hydraulic system design point | NATO H-538 fluid capability; non-US qualified product |
| Ground survival | -55 deg C equipment survival | Arctic equipment category |
| Storage survival | -60 deg C non-operating, 72 h | Severe cold margin |
| Thermal transition | -55 deg C soak to full system operation | Start-up transient |
| Icing exposure | Mission-defined; not an all-known-icing clearance | Aircraft-specific certification |
| Salt/contamination | De-icing fluid, snow melt, coastal salt | Maintenance-driven |

CLOSED: Environmental envelope is internally consistent. Aircraft-level demonstration remains PENDING VALIDATION because public data do not establish the complete baseline qualification history.

## 2.2 Materials and fluid selections

| Application | Selected material / fluid | Reason | Recycled? |
| --- | --- | --- | --- |
| Hydraulic system | NYCO HydrauNycoil FH 42 or equal non-US NATO H-538 product | -54 to +200 deg C service range; fire-resistant synthetic hydrocarbon | No |
| Static seals | Low-temperature fluorocarbon or FFKM from a qualified Canadian or non-US NATO/EU source | Compression-set resistance; compatibility testing required | No |
| Dynamic seals | OEM-qualified low-temperature package from Canada or non-US NATO/EU source | Extrusion and wear control | No |
| Landing-gear grease | Approved low-temperature aviation grease from Canadian or non-US allied source | Torque and wear at cold start | No |
| Secondary brackets | Certified recycled Canadian 6061-T6 or 7075-T7351 | Non-primary, inspectable, replaceable | Yes |
| Brake heat shields | Certified recycled Canadian or EU 17-4 PH sheet | Non-structural thermal shield | Yes |
| GSE frames and shelters | Certified recycled Canadian structural steel or aluminium | Low criticality, high traceability and domestic fabrication | Yes |
| Equipment racks and cable raceways | Certified recycled 6061-T6 aluminium | Non-primary, replaceable and inspectable | Yes |
| Fasteners | New certified aerospace fasteners only | Traceability and fatigue risk dominate | No |

NYCO HydrauNycoil FH 42 is a French-sourced synthetic hydraulic fluid approved to NATO Code H-538 with a -54 deg C to +200 deg C operating range; an equivalent non-US NATO H-538 product may be substituted after OEM compatibility review.

US-origin AerMet 100 and other US-controlled alloys are not specified. Secondary metallic parts default to certified Canadian recycled-content feedstock where qualification is practical; Canada has publicly identified domestic steel and aluminium as industrial capabilities suitable for deeper aerospace supply-chain integration.

ENGINEERING NOTE  The prior draft’s use of MIL-PRF-83282 as a -55 deg C strut fluid was not closed: the current specification is rated only to -40 deg C. This baseline selects a non-US NATO H-538 product instead.

## 2.3 Airframe cold-soak measures

- Install removable close-fitting intake, exhaust, pitot/static, APU/drain-mast, and canopy covers with high-visibility remove-before-flight streamers.

- Add hydrophobic, erosion-resistant leading-edge coating qualified for rain, sand, de-icing fluid, and thermal cycling.

- Add low-power heated battery enclosure and avionics-rack thermal blankets fed from ground power before start.

- Replace selected external access-panel fasteners with captive, gloved-hand fasteners where aerodynamic flushness permits.

- Install improved door and panel drainage to prevent refreeze after snow melt or de-icing fluid ingestion.

- Apply corrosion-inhibiting compound at faying surfaces exposed to de-icing salt and meltwater.

### Structural closure

No primary structure is resized. Secondary brackets, equipment racks and cable raceways added for the CFT, navigation antennas and ice-protection controller are sized to 1.5 times limit load with material allowables from the applicable specification. Recycled parts receive the same dimensions as virgin-material parts; recycled content is not used to reduce section thickness.

CLOSED: Structural concept is CLOSED. Fastener pattern, fit-up, flutter influence, and fail-safe inspection intervals are PENDING OEM VALIDATION.

## 2.4 Manufacturing and inspection

All aircraft parts follow a controlled material route: qualified source, country-of-origin declaration, heat/lot record, recycled-content declaration, receiving inspection, machining, surface treatment, non-destructive inspection where required, part marking and configuration record. Recycled material is re-melted or re-certified into a standard wrought product before machining; salvaged aircraft parts are not machined directly into flight hardware. US-origin material is excluded by default from new augmentation parts and may only enter through a documented programme exception below the aggregate new-kit cap; this exclusion does not apply to baseline aircraft equipment.

IAQG 9100 provides the quality-management framework for aviation, space, and defence supply-chain traceability and reduction of organization-specific requirements.

| Gate | Evidence required |
| --- | --- |
| Material receipt | Melt certificate, chemistry, mechanical properties, recycled-content declaration |
| First article | Dimensional report, process coupons, surface treatment record, NDI report |
| Production | Lot traceability, torque records, bond-line witness coupons where applicable |
| Installation | Digital mass/properties update, electrical continuity, bonding, leak and functional test |
| In-service | Cold-season inspection plan and configuration-controlled repair limits |

CLOSED: Manufacturing route is CLOSED for secondary metallic parts. Bonded CNT heaters and CFT production first-article inspection remain PENDING VALIDATION.

# PART III — PROPULSION, FUEL AND EMISSIONS

## 3.1 Propulsion baseline

The Gripen E uses the GE F414G; Saab publishes 98 kN maximum thrust, and GE lists the F414 family at a 22,000 lb thrust class, 154 in length, 35 in maximum diameter, 170 lb/s airflow, and 30:1 pressure ratio.

GE states that more than 1,600 F414 engines have been delivered and that the family uses a six-module architecture intended to simplify maintenance.

The Arctic package does not change compressor geometry, turbine temperatures, afterburner hardware, nozzle hardware, or engine rating. This avoids introducing a new engine qualification program and preserves the published thrust basis. The F414G is baseline aircraft equipment and is grandfathered under the sourcing rules; no propulsion replacement is included. Arctic adaptation is limited to approved fluids, start logic, inlet protection, ground preheat, monitoring, and maintenance procedures.

### Engine changes explicitly excluded

- No new nozzle or plug geometry.

- No increased turbine temperature or thrust rating.

- No recycled rotating engine material.

- No uncertified fuel additive or oil.

CLOSED: Propulsion architecture is CLOSED at CDR-80 because the engine remains configuration-controlled and unchanged. Aircraft-level cold-start demonstration remains PENDING VALIDATION.

## 3.2 Cold-start system

| Function | Design | Control |
| --- | --- | --- |
| Battery conditioning | Heated enclosure; ground power until start | Thermal switch and cockpit indication |
| Oil preheat | External pre-oiler / oil heater cart | Ground interlock; automatic disconnect |
| Hydraulic readiness | H-538 fluid; reservoir temperature indication | Start inhibit below approved minimum |
| Fuel conditioning | Approved F-34/JP-8 with FSII as applicable | Water drain and fuel temperature check |
| FADEC start logic | OEM-approved cold-start schedule only | No field modification |
| Intake/exhaust protection | Covers installed until pre-start walkaround | Two-person removal check |

The start design uses ground energy because battery heating, oil heating, hydraulic warm-up and cockpit/avionics conditioning can exceed the practical capacity of an air-carried emergency battery. A 12 h cold-soak start at -40 deg C is achieved by preheating to the OEM start envelope, not by attempting to crank a fully cold engine with onboard energy.

| Start case | Ground support | Pass criterion |
| --- | --- | --- |
| Normal Arctic alert | GPU + pre-oiler + battery heater | Stable idle within OEM limit |
| Remote alert with shelter | Diesel-heated shelter + GPU | Stable idle within OEM limit |
| Emergency unsheltered start | Portable GPU and preheat cart | Start permitted only above OEM minimums |
| Generator failure | Abort preheat; preserve battery | No start attempt below limits |

## 3.3 Inlet and FOD protection

The primary inlet anti-ice function remains the approved engine/inlet system. The air-carried FOD screen is removed from the baseline because its qualification, pressure-loss, vibration and ice-bridging burden exceeds its Arctic value. FOD control is instead achieved through covers, runway sweeping, snow control, intake inspection and engine/inlet sensors supplied with the baseline engine installation.

| Case | Baseline control | Protection logic |
| --- | --- | --- |
| Parked | Covers installed | Exclude snow, ice and tools |
| Engine start / taxi | Covers removed after runway and intake check | FOD prevention by procedure and surface control |
| Takeoff roll | Approved inlet configuration only | Takeoff rejected on blockage or abnormal inlet indication |
| Flight | No air-carried screen | No supplemental pressure-loss penalty |
| Icing | Approved inlet anti-ice active | CNT does not replace engine inlet protection |

ENGINEERING NOTE  Deleting the FOD screen removes a low-value optional mission kit and its qualification burden. Runway sweeping, snow control, covers and intake inspection remain mandatory operational controls.

CLOSED: Inlet protection is CLOSED only because the approved engine inlet system remains primary and no separate air-carried screen is procured.

## 3.4 Fuel and conformal tank option

The optional CFT-A2 consists of two shoulder-mounted tanks, each sized for 950 L usable F-34/JP-8. At a planning density of 0.80 kg/L, the pair carries 1,520 kg usable fuel. The empty pair, including attach fittings and transfer plumbing, is budgeted at 235 kg. The tanks feed first in cruise to restore centre-of-gravity and reduce structural inertia early in the mission.

| Parameter | Value | Closure |
| --- | --- | --- |
| Usable CFT fuel | 2 x 950 L = 1,900 L | CLOSED as design volume |
| Planning fuel mass | 1,520 kg | CLOSED at 0.80 kg/L |
| Empty pair mass | 235 kg | CLOSED budget |
| Representative TOW with CFT | 14,537 kg | CLOSED below 16,500 kg |
| Manoeuvre limit with CFT | To be established by flight clearance | PENDING VALIDATION |
| Emergency jettison | Two independent release circuits | PENDING separation test |
| Fuel transfer | CFT first, then internal | CLOSED control logic |

The CFT is deliberately optional. Short-range alert sorties retain the lighter, cleaner aircraft; long-range sovereignty and ferry missions install the pair. Air-to-air refuelling remains the operational reserve and is not replaced by the CFT.

CLOSED: Mass and fuel-volume closure is complete. Flutter, buffet, separation, crashworthiness, and range demonstration remain PENDING VALIDATION.

# PART IV — SENSORS, NAVIGATION AND COMMUNICATIONS

## 4.1 Sensor philosophy

The radar, IRST, electronic-warfare system, datalinks, and mission computer are retained. Arctic value comes from software modes, environmental models, sensor-health monitoring, and disciplined emissions control—not from speculative hardware replacement. New modes are treated as mission-system applications isolated from flight-critical software.

Saab states that Gripen E has an AESA radar, IRST, advanced electronic warfare and communications, and an avionics architecture separating mission-system functionality from flight-critical software.

| Mode | Function | Verification |
| --- | --- | --- |
| Snow/ice clutter | Terrain-type adaptive radar processing | Recorded-data regression + flight test |
| Auroral optical clutter | IRST false-track suppression and confidence gating | Hardware-in-loop + flight trial |
| Cold-soak BIT | Extended start-up confidence checks | Environmental chamber |
| EMCON | Emission scheduler and passive-track priority | Mission-system test |
| Sea-ice navigation aid | Radar/INS cueing for featureless areas | Flight trial |

CLOSED: Mode architecture is CLOSED. Quantitative detection ranges and false-alarm performance are PENDING VALIDATION; unsupported test results from the draft have been removed.

## 4.2 Resilient navigation

Navigation uses a layered architecture: multi-constellation GNSS with Galileo-capable cross-checking as the normal source; Canadian anti-jam antenna protection; tightly coupled inertial navigation; barometric aiding; and terrain-aided updates where relief permits. The dedicated celestial sensor is deleted; a software-only celestial interface may be restored later if the mission computer already supports it. The pilot always has an explicit navigation-source display and a degrade/restore timeline.

Hexagon|NovAtel launched the Calgary-developed GAJT-AE3 airborne anti-jam antenna in March 2026; it provides multi-constellation, multi-frequency protection and up to seven nulls per band while feeding a protected RF signal to modern or legacy GNSS receivers.

| Layer | Equipment / method | Role |
| --- | --- | --- |
| Primary | Multi-GNSS receiver with Galileo cross-check; GPS not sole source | Normal navigation |
| Protection | Canadian GAJT-AE3-class multi-element anti-jam antenna | Jamming mitigation |
| Core backup | High-grade INS from non-US allied source with integrity monitor | GNSS-denied continuity |
| Aiding | Barometric and terrain where relief permits; no new celestial sensor | Bound drift |
| Procedural | Preplanned divert gates and fuel triggers | Human-system closure |

ENGINEERING NOTE  No numerical CEP is claimed for a full four-hour GNSS-denied sortie because the installed INS grade and error budget are not public. The requirement is an allocated error budget with flight-test validation.

## 4.3 Communications and polar connectivity

Tactical communications remain the approved military radios and datalinks. No permanent US-controlled commercial satellite terminal is installed. A provision is reserved for the Canadian Telesat Lightspeed / ESCP-P polar service for logistics, safety-of-flight messaging and search-and-rescue coordination when that service is fielded; until then, HF/UHF relay and preplanned procedures provide the fallback. The auxiliary path is not a replacement for secure tactical datalinks and does not carry flight-control commands.

Canada selected MDA Space and Telesat as strategic partners for ESCP-P Arctic military satellite communications. Telesat states that its expanded Lightspeed Mil-Ka service is planned from 2028 and will cover 65–90 deg N for the Canadian Armed Forces.

| Link | Use | Failure action |
| --- | --- | --- |
| Primary military voice/data | Mission command and control | Use preplanned lost-comms procedure |
| Tactical datalink | Cooperative track and command | Operate autonomous / passive |
| Canadian polar satcom provision | Future logistics, weather and SAR coordination | Use HF/UHF relay or return-to-base trigger |
| 406 MHz emergency beacon | Post-ejection location | Independent battery and antenna; non-US source required |
| Navigation database updates | Ground loading only | Signed data package and configuration check |

CLOSED: Communication architecture is CLOSED without a permanent US satcom dependency. Canadian service availability, antenna placement, electromagnetic compatibility and security accreditation remain PENDING VALIDATION.

## 4.4 Emissions and cyber control

Arctic operations may require long periods outside dense ground-based coverage. The aircraft therefore receives a mission-system emissions scheduler that coordinates radar, datalink, interrogator and future Canadian polar satcom use against the mission phase. Passive sensors and received tracks are preferred when the mission permits. The scheduler is advisory unless the pilot selects an EMCON state.

- Signed mission-data loads with rollback protection.

- Separate flight-critical and mission-system software domains.

- Hardware maintenance port physically disabled in flight.

- Zeroize and crypto procedures unchanged from customer baseline.

- No remote aircraft control or autonomous weapon-release function is added.

### Electromagnetic compatibility

CRPA, future polar satcom, heated mats and radar all introduce potential electromagnetic interactions. The design reserves frequency, cable-route, bonding and shielding allocations, then verifies by conducted/radiated emissions and susceptibility testing on a representative aircraft.

CLOSED: Cyber and EMCON architecture is CLOSED. Aircraft-level EMC qualification and mission-system security accreditation are PENDING VALIDATION.

# PART V — ELECTRICAL POWER AND ICE PROTECTION

## 5.1 Electrical architecture

The single-engine Gripen E cannot rely on a fictional twin-generator load sum. Deleting permanent satellite and celestial hardware reduces the allocated peak Arctic flight load to 11.40 kW, including a 20% design margin. Ground-only preheat and wheel-heating functions are supplied by GSE. If the OEM load survey shows insufficient baseline margin, the package includes an OEM-designed generator/power-distribution uprate rather than exceeding the existing bus.

Figure 2. Revised Arctic flight electrical allocation after removal of permanent satellite and celestial hardware; ground-only loads are excluded.

| Load | Peak kW | Duty |
| --- | --- | --- |
| CNT / electro-thermal zones | 6.80 | Sequenced, max 5 min |
| ECS recirculation and controls | 1.20 | Continuous in extreme cold |
| Navigation and anti-jam additions | 0.70 | Continuous |
| Battery recharge reserve | 0.80 | Post-start |
| Subtotal | 9.50 | — |
| 20% margin | 1.90 | Design reserve |
| Allocated peak | 11.40 | Flight budget |

CLOSED: Electrical load logic is CLOSED. Generator capacity, accessory-gearbox torque, cooling, and bus transient behaviour are PENDING OEM VALIDATION.

## 5.2 Electro-thermal ice protection

The electro-thermal system covers wing leading edges, canard leading edges, vertical-tail leading edge, selected pylons, and non-primary probes. It does not replace the certified engine-inlet anti-ice system. CNT or equivalent thin-film heaters are bonded beneath an erosion-resistant outer layer and arranged in 24 independently monitored zones.

University of Nottingham research reports CNT buckypaper integrated in fibre-reinforced composites as a promising self-heating and aircraft de-icing approach.

The National Research Council Canada operates icing wind-tunnel and related facilities for icing formation, detection and mitigation work on fixed-wing aircraft; qualification is assigned to Canadian facilities where capacity permits, with allied facilities as backup.

| Design parameter | Value |
| --- | --- |
| Zones | 24 independently switched zones |
| Peak simultaneous draw | 6.8 kW |
| Heater areal power | 2.0–4.5 kW/m2 by zone |
| Ice trigger | Ice detector plus pilot selection; no automatic takeoff credit |
| Failure tolerance | One failed zone does not inhibit engine inlet anti-ice |
| Monitoring | Resistance trend, current imbalance, over-temperature, open/short detection |
| Lightning | Bonded metallic edge and transient suppression at controller |
| Repair | Panel-level replacement; no structural repair credit |

CLOSED: Concept and power closure are complete. Icing-tunnel, lightning, fatigue, hail, bird, and flight qualification are PENDING VALIDATION.

## 5.3 Thermal management and ECS

The ECS retains the baseline architecture and adds a controlled recirculation mode for extreme cold, insulated avionics-rack blankets, temperature sensors at cold-soak-critical LRUs, and revised ground conditioning logic. Waste heat is reused where possible; no fuel is heated beyond its approved temperature limit, and the fuel thermal-management authority remains with the OEM.

| Zone | Ground rule | Flight rule |
| --- | --- | --- |
| Cockpit | Preheat to safe display and canopy temperature | 18–24 deg C target when mission permits |
| Mission avionics | Blankets and ground-conditioned air | Normal ECS cooling after start |
| Battery | Ground heater until minimum start temperature | Heater off; monitored discharge |
| Hydraulic reservoir | Temperature indication; minimum start limit | H-538 fluid and normal circulation |
| Wheel bearings | Ground-powered preheat through GSE only; no onboard heater kit | No flight heater load |
| Fuel | No unauthorized fuel heating | OEM-approved thermal management only |

ENGINEERING NOTE  The prior draft’s separate glycol-loop capacity claims were removed because the public baseline does not substantiate them. This design allocates functions and interfaces without inventing an installed heat-exchanger capacity.

## 5.4 Battery and emergency power

The emergency battery is sized as a separate requirement from propulsion start. The selected battery is a heated 28 V-class, 45 Ah aerospace-approved lithium iron phosphate (LiFePO4) unit with cells, pack and battery-management system sourced from Canada or a non-US NATO/EU supplier; a single chemistry is committed at this issue to bound qualification and integration effort. At 80% low-temperature usable capacity it provides approximately 1.01 kWh. A 1.3 kW shed load yields approximately 46 minutes theoretical endurance; the design requirement is 30 minutes with at least 25% reserve after cold derating.

| Design case | Calculation | Result |
| --- | --- | --- |
| Usable energy | 28 V x 45 Ah x 0.80 | 1.008 kWh |
| Maximum shed load | 1.30 kW | 0.775 h theoretical |
| Required endurance | 0.50 h | 1.55 energy margin factor |
| End-of-life allowance | 20% capacity loss | Requirement retained |
| Safety | Cell monitoring, venting, isolation, thermal-runaway containment | PENDING certification |

CLOSED: Battery energy closure is complete and chemistry selection is CLOSED with the committed LiFePO4 unit. Installation safety, thermal-runaway certification and the aircraft emergency-load survey remain PENDING VALIDATION.

# PART VI — LANDING GEAR AND SURFACE OPERATIONS

## 6.1 Gear, wheels and brakes

The baseline landing gear geometry, shock strut, retraction system, wheel bay, brake control unit and primary fittings are retained. The Arctic package changes approved consumables and monitoring: non-US NATO H-538 hydraulic fluid where authorized, low-temperature grease, low-compression-set seals, wheel-bearing temperature indication, snow-rated tires and a low-friction anti-skid schedule. Canadian landing-gear engineering, machining, test and overhaul capacity is preferred where OEM licensing permits.

Michelin states that aircraft tires may be certified for in-service operation to -55 deg C; this confirms a credible low-temperature design point, while the actual Gripen tire remains subject to dimensional, load and speed approval.

| Item | Baseline action | Validation |
| --- | --- | --- |
| Main/nose struts | Retain; inspect cold seals and gas charge | Strut drop and service test |
| Wheels | Retain unless tire approval requires wheel change | Dimensional and thermal survey |
| Tires | Low-temperature, snow-rated tire within approved envelope | Dynamometer and field test |
| Brakes | Low-mu anti-skid schedule | Instrumented runway test |
| Bearings | Ground-powered preheat through GSE only; no onboard heater kit | Cold-start torque test |
| Gear doors | Drainage and ice-clearance inspection | Icing/freezing-rain test |

CLOSED: Gear architecture is CLOSED. Tire/flotation and low-mu stopping performance are PENDING VALIDATION.

## 6.2 Runway and surface limits

The package supports paved runways, compacted snow, and prepared frozen gravel when the surface strength and friction have been assessed. It does not claim routine blue-ice landing capability. Operations on ice require site-specific bearing-strength, thickness, temperature, crack, and friction assessment by the responsible authority.

The operating concept uses prepared Canadian northern facilities, mobile shelters and existing allied support arrangements. It does not depend on a US airfield, US-controlled satellite service, or unimproved ice operations.

| Surface | Operating rule |
| --- | --- |
| Paved runway, dry/wet | Within baseline flight manual limits |
| Paved runway, ice contamination | Use reported runway condition and reduced crosswind limit |
| Compacted snow | Prepared surface; braking action report required |
| Frozen gravel | Prepared, swept and inspected; intake inspection required |
| Blue ice | Taxi/park only after site engineering approval; no routine landing credit |
| Deep loose snow | >75 mm loose accumulation requires runway treatment or delay |

ENGINEERING NOTE  A numerical CBR clearance is not claimed because tire pressure, gear loads, and the operator’s surface assessment method must be reconciled. This avoids the prior draft’s unsupported CBR 4 claim.

## 6.3 Cold turnaround and de-icing

The fastest safe Arctic turnaround uses a sheltered or partially sheltered aircraft, ground power, preheated oil/battery, approved Type I/IV ground de-/anti-icing where required and a two-person contamination check. Ground de-icing is a procedural control; the airborne CNT system is not credited to make a contaminated wing safe for takeoff.

| Step | Target time | Critical control |
| --- | --- | --- |
| Receive and safe aircraft | 2 min | Chocks, intake/exhaust clear, covers logged |
| Ground power and preheat | Parallel | GPU and pre-oiler connected |
| Refuel | 8–12 min | Fuel temperature and water check |
| Rearm / role check | Parallel | Cold-rated handling equipment |
| De-/anti-ice | As required | Holdover time and clean-wing check |
| Start and BIT | 5–8 min | Minimum temperatures satisfied |
| Release | 1 min | Cover and tool accountability |

CLOSED: Turnaround concept is CLOSED. The 15–25 minute baseline turnaround published by Saab remains the benchmark; Arctic demonstration under cold-soak and snow conditions is PENDING VALIDATION.

# PART VII — SURVIVAL, SUPPORT AND MAINTENANCE

## 7.1 Pilot survival equipment

The pilot equipment set is designed for ejection or forced landing in cold air, snow, open water, and darkness. It combines immersion protection, insulated clothing, personal locator beacon, fire-starting equipment, emergency rations, shelter, signalling, first aid, and water procurement. A satellite messenger is added only when a Canadian or allied accredited service and hardware are fielded. The aircraft installation preserves ejection-seat clearances and does not obstruct emergency egress.

| Item | Minimum requirement |
| --- | --- |
| Anti-exposure garment | 24 h static protection target at -40 deg C after donning; actual rating by test |
| Personal locator beacon | 406 MHz with 121.5 MHz homing, -40 deg C operation and non-US source |
| SAR communications | 406 MHz PLB plus HF/UHF procedure; satellite messenger only if Canadian/allied service and hardware are accredited |
| Shelter | Two-person snow shelter; poles may use reclaimed composite offcuts |
| Fire and water | Ferrocerium starter, sealed tinder, snow-melting container |
| Rations | 24 h cold-stable, no-cook pack |
| Flotation | Immersion-compatible life preserver and raft where mission dictates |

CLOSED: Equipment list is CLOSED. Seat integration, ejection clearance, immersion testing, and human-factors assessment are PENDING VALIDATION.

## 7.2 Arctic ground support equipment

| GSE item | Requirement |
| --- | --- |
| Rapid-deploy shelter | Wind-rated, heated, lights, 48 h fuel autonomy |
| Ground power unit | Aircraft-compatible AC/DC output; -55 deg C survival |
| Pre-oiler / oil heater | Closed-loop filtered unit with aircraft quick disconnects |
| Battery heater cart | Temperature-limited and aircraft interlocked |
| De-icing vehicle | Approved fluid, heated boom, holdover documentation |
| Fuel equipment | Filter/water separator, low-temperature hoses, bonding reels |
| Hydraulic service unit | H-538 clean service and particle monitoring |
| CNT test set | Zone isolation, resistance trend, insulation resistance |
| Snow/FOD tools | Sweepers, brooms, covers, inlet inspection lights |
| Composite repair kit | Heated tent, vacuum bag, approved cure blankets |

GSE is sized for two-aircraft dispersed elements and scalable to a six-aircraft forward operating location. Every item has a cold-soak start procedure, low-temperature fluid chart, spare igniter/battery plan and enclosed maintenance workspace where required. Canadian fabrication is the default for shelters, frames, carts, covers, hose reels, battery trays and non-powered GSE; recycled Canadian steel and aluminium are used where traceability is maintained.

Canada’s published northern infrastructure program includes runway, hangar, warehousing, operations, and life-support improvements; the GSE plan is designed to use that infrastructure rather than duplicate it.

## 7.3 Maintenance concept

The maintenance concept is two-level. Forward locations perform inspection, servicing, LRU replacement, tire/brake changes, CNT zone checks, software loading, and minor non-structural repairs. Depot performs CFT structural work, bonded heater replacement, landing-gear overhaul, engine module work, and any repair affecting primary structure or certified ice protection.

| Check | Trigger | Content |
| --- | --- | --- |
| Pre-flight | Every flight | Covers, contamination, tires, ice sensors, nav integrity |
| Cold turnaround | Each Arctic sortie | Fluid leaks, battery health, brake temperature, inlet inspection |
| 50 h / monthly | Earlier of | CNT resistance trend, heaters, connectors, drainage |
| 200 h / 12 months | Earlier of | Seal sampling, corrosion inspection, battery capacity, gear inspection |
| Depot | 1,200 h or 4 years | CFT, heater bonds, gear, engine modules, full corrosion survey |

- All special tools fit in two standard NATO air-cargo pallet positions per deployed detachment.

- Consumables are colour-coded by temperature rating to prevent mixed-fluid or wrong-seal installation.

- Maintenance data collect cold-start time, heater duty, tire cycles, brake energy, CNT resistance, and GNSS availability.

CLOSED: Maintenance allocation is CLOSED. Reliability-centred maintenance intervals require in-service data and are PENDING VALIDATION.

# PART VIII — MASS, CG AND PERFORMANCE CLOSURE

## 8.1 Mass budget

The public Gripen E empty mass is not issued as a certified mass statement. This specification therefore uses 8,000 kg as an open-source planning value and treats the result as an allocation, not a claim against an individual aircraft’s weighed empty mass.

Open-source comparisons place Gripen E empty mass at approximately 8.0 t and internal fuel approximately 40% above Gripen C; those values are used only as planning assumptions because Saab’s public page does not publish a detailed mass breakdown.

| Mass element | kg |
| --- | --- |
| Baseline empty planning mass | 8,000 |
| Permanent Arctic installed kit | 232 |
| Optional CFT pair empty | 235 |
| Crew and mission equipment | 100 |
| Representative QRA stores and pylons | 1,050 |
| Internal fuel planning value | 3,400 |
| CFT usable fuel | 1,520 |
| Representative take-off mass | 14,537 |
| Published MTOW | 16,500 |
| Unused MTOW margin | 1,963 |

CLOSED: Mass closes below MTOW with the CFT installed and without reducing fuel. Actual aircraft weighing and stores accounting remain PENDING VALIDATION.

## 8.2 Centre of gravity and structural loads

Permanent additions are distributed close to the existing centre of gravity. CFT tanks are positioned symmetrically about the longitudinal reference and are emptied from aft to forward within each tank so that the normal cruise CG travel remains bounded. The design target is no change to the certified flight-control laws and no extension of the existing CG envelope.

Figure 3. Representative Arctic QRA mass closure against the published 16,500 kg MTOW.

| Check | Criterion | Result |
| --- | --- | --- |
| Static mass | Representative TOW <= 16,500 kg | PASS — 14,537 kg |
| CFT limit load | 6 g mission limit assumed for sizing | Analysis closes at 1.5x limit |
| Attach-point ultimate load | Four-point load path; no single-point failure | Concept closes |
| CG envelope | Within baseline envelope in all fuel states | Analysis closes |
| Flutter / buffet | New external shape requires test | PENDING VALIDATION |
| Crash / jettison | No hazard to aircraft or crew | PENDING separation test |

## 8.3 Performance and integrated closure

The CFT pair increases available fuel by 1,520 kg. At a constant specific range this is approximately 45% of the 3,400 kg internal-fuel planning value. After a conservative drag and reserve allowance, the requirement remains a 25% mission-radius improvement for a defined high-altitude sovereignty profile; compliance is PENDING an aircraft-specific drag model and flight-test data.

| Integrated budget | Closure statement |
| --- | --- |
| Geometry | CFT and antenna envelopes close in concept; OEM clearance pending |
| Mass | Closes with 1,963 kg MTOW margin |
| Power | 11.40 kW allocated; OEM generator margin pending |
| Thermal | Ground preheat and ECS recirculation close functionally |
| Fuel | CFT volume and transfer sequence close |
| Structure | No primary airframe redesign; CFT fittings pending detail substantiation |
| Landing | No MTOW growth; tire/surface performance pending |
| Navigation | Layered architecture closes; end-to-end error budget pending |
| Ice | Power and zone logic close; aircraft icing qualification pending |

CONTROLLED: The Arctic kit has no internal contradiction after removal of low-value optional hardware. It is not yet a flight-release baseline because external-store, icing, EMC, cold-soak and generator substantiation require physical or OEM evidence.

# PART IX — RECYCLED MATERIALS

## 9.1 Recycled-content policy

Recycled metal is used only where it reduces supply risk without reducing traceability, fatigue performance, fracture control, electromagnetic performance or repairability. Material is converted back to a certified wrought product or casting feedstock; it is never used as unidentified salvaged aircraft structure. The phrase recycled content in this document therefore means certified reprocessed material with full chemistry, country-of-origin and mechanical-lot records.

| Use | Decision | Reason |
| --- | --- | --- |
| Secondary equipment brackets | ALLOWED | Inspectable, replaceable, non-primary |
| Brake heat shields | ALLOWED | Thermal shield only; not a load path |
| GSE and shelters | ALLOWED | High value and low airworthiness risk |
| Equipment racks and cable raceways | ALLOWED | Inspectable, replaceable and non-primary |
| Non-pressure CFT access fairings | ALLOWED | Non-structural and easily replaceable |
| Non-flight survival poles | ALLOWED | No aircraft structural function |
| Battery enclosure tray | ALLOWED | Non-structural, inspectable and replaceable enclosure |
| Primary wing/fuselage structure | EXCLUDED | Certification and fatigue risk |
| Landing-gear trunnions | EXCLUDED | Fracture-critical |
| Engine rotating parts | EXCLUDED | Unsafe and impractical |
| Critical fasteners | EXCLUDED | Traceability and fatigue risk dominate |
| EM-critical RAM mesh | EXCLUDED | Property variability could defeat performance |

CLOSED: Recycled-material policy closes with consistent allow-and-exclude rationale. Canadian recycled feedstock is preferred for all ALLOWED uses.

# PART X — VERIFICATION, VALIDATION AND COMPLIANCE

## 10.1 Verification logic

Verification proceeds from requirements review and analysis to component test, subsystem test, environmental qualification, hardware/software integration, ground test, taxi test, flight test, and operational evaluation. No marketing claim is accepted as verification. Every CLOSED item in this document means that the design logic and interface budget close; it does not mean that a physical aircraft has been tested.

EASA CS-25 icing provisions, including CS 25.1419, CS 25.1420 and Appendix O, are used as non-US public guidance for the structure of ice-protection compliance; military certification will still require the customer’s approved certification basis.

| Stage | Output | Gate |
| --- | --- | --- |
| SRR | Requirements verified as measurable and allocated | Complete for this baseline |
| PDR | Architecture and safety analyses | Assumed complete before CDR-80 |
| CDR-80 | Budgets, interfaces, drawings and V&V plan | This document |
| Qualification | Component and environmental evidence | Pending |
| Aircraft ground test | Integrated power, fuel, ECS, EMC, cold soak | Pending |
| Flight test | Performance, handling, icing, navigation, separation | Pending |
| Operational evaluation | Northern deployment and availability trial | Pending |

## 10.2 Qualification matrix

| Requirement | Method | Minimum evidence | Status |
| --- | --- | --- | --- |
| -55 deg C ground survival | Chamber / natural environment | No damage after 72 h; functional after return | PENDING |
| -40 deg C cold start | Chamber + field | 50 starts; >=95% first-attempt success | PENDING |
| Ice protection | Icing tunnel + flight | No hazardous accretion; inlet system independent | PENDING |
| CNT heater durability | Thermal/vibration/rain erosion | Resistance drift within limit and no unsafe failure | PENDING |
| Power | HIL + ground test | Transients and load shedding within bus limits | PENDING |
| Battery | Cell/pack/aircraft safety | 30 min emergency power after cold derating | PENDING |
| CFT | Structural, flutter, fuel, separation | Envelope and emergency jettison clearance | PENDING |
| Navigation | HIL + flight | 4 h GNSS-denied error within allocated budget | PENDING |
| Low-mu braking | Instrumented runway | No wheel lock/hydroplaning hazard; distance model | PENDING |
| GSE | NATO AECTP / DEF STAN 00-035 tailored | Operate at -40 deg C; survive -55 deg C | PENDING |

ENGINEERING NOTE  The qualification matrix intentionally has no fabricated completion dates. Dates are set only after aircraft availability and customer test-range scheduling are known.

## 10.3 Compliance matrix

| Spec para. | Requirement | Compliance evidence at CDR-80 | Status |
| --- | --- | --- | --- |
| 1.2 | All KPPs allocated | Requirement dictionary and V&V matrix | CLOSED |
| 2.2 | NATO H-538 hydraulic fluid | Non-US NYCO FH 42 or equivalent selection | CLOSED; installation pending |
| 2.3 | No primary structure redesign | Design decision and structural allocation | CLOSED |
| 3.1 | Engine unchanged | F414G baseline and exclusion list | CLOSED |
| 3.4 | CFT below MTOW | Mass closure, Part VIII | CLOSED |
| 4.2 | GNSS denial resilience | Layered navigation architecture | CLOSED; flight pending |
| 5.1 | Peak added load <=11.40 kW | Revised power allocation | CLOSED; OEM margin pending |
| 5.2 | Inlet anti-ice retained | System boundary | CLOSED |
| 6.2 | No unsupported blue-ice landing credit | Operating limits | CLOSED |
| 8.1 | Representative TOW <=16,500 kg | 14,537 kg calculation | CLOSED |
| 9.1 | No recycled primary structure | Materials policy | CLOSED |
| 10.1 | No false test completion | Qualification matrix labels evidence | CLOSED |

CLOSED: Compliance at CDR-80 is complete. Airworthiness compliance cannot be claimed until the customer approves a certification basis and test evidence.

# PART XI — RISK REGISTER

## 11.1 Programme and operational risks

The register records programme, supply, schedule and operational risks only. Engineering defects discovered during this reconciliation were corrected in the baseline and are not logged as open errors. Risk scores use Probability x Impact on a 1–5 scale; residual values assume the stated mitigation is funded.

| ID | Risk | P | I | Score | Mitigation | Residual |
| --- | --- | --- | --- | --- | --- | --- |
| R-001 | CFT flutter or separation test delays long-range capability | 3 | 4 | 12 | Keep CFT optional; retain baseline ferry tanks and AAR | M |
| R-002 | OEM electrical margin insufficient for added loads | 3 | 5 | 15 | Load survey before CDR closure; OEM generator uprate option | M |
| R-003 | CNT heater fails durability or lightning qualification | 3 | 4 | 12 | Retain inlet anti-ice; zone isolation; panel replacement | M |
| R-004 | GNSS denial exceeds modelled availability | 3 | 4 | 12 | Layered navigation and procedural divert gates | M |
| R-005 | Recycled feedstock lot fails traceability or chemistry audit | 2 | 3 | 6 | Use only approved lots; stop-use gate on audit failure | L |

## 11.2 Risk register continued

| ID | Risk | P | I | Score | Mitigation | Residual |
| --- | --- | --- | --- | --- | --- | --- |
| R-006 | Northern infrastructure schedule slips | 3 | 4 | 12 | Deploy mobile shelter/GSE; use existing hangars | M |
| R-007 | Low-temperature seal or fluid supply interruption | 3 | 3 | 9 | Dual-source QPL items; 24-month stock | L-M |
| R-008 | Canadian polar satcom service is not available before the planned 2028 service start | 3 | 3 | 9 | Use HF/UHF procedures; reserve but do not install permanent terminal | L-M |
| R-009 | Engine export-control or support interruption | 2 | 5 | 10 | Spares, in-country MRO plan, early government agreement | M |
| R-010 | Cold-weather maintainer error rate rises | 3 | 4 | 12 | Gloved-hand design, shelters, checklists, training | M |
| R-011 | Sovereign polar communications service or security accreditation is delayed | 3 | 3 | 9 | Keep provision uninstalled until accredited; HF/UHF procedures | L-M |
| R-012 | Operational availability model optimistic | 3 | 4 | 12 | 30-day northern trial before declaration | M |

### Risk posture

The dominant technical risk is no longer cold temperature alone; it is the interaction of added electrical loads, CFT flutter/separation and ice-protection qualification. The baseline controls these by retaining the engine and primary airframe, making the CFT optional, deleting low-value permanent add-ons, excluding ground preheat from flight loads and preserving approved inlet anti-ice as the primary protection.

CLOSED: No engineering-error entries remain in the register. All open evidence items are carried in Part X rather than disguised as risks.

# PART XII — CONFIGURATION AND IMPLEMENTATION

## 12.1 Configuration definition

| Configuration item | Identifier | Permanent / mission | Status |
| --- | --- | --- | --- |
| Arctic Operations Kit | AOK-1 | Permanent | CDR-80 |
| Cold-start support kit | CSK-1 | Ground equipment | CDR-80 |
| Electro-thermal ice-protection kit | EIPS-1 | Permanent | CDR-80; qualification pending |
| Resilient navigation kit | RNK-1 | Permanent | CDR-80; Canadian anti-jam flight pending |
| Landing-gear Arctic service kit | LGK-1 | Permanent/consumable | CDR-80; validation pending |
| Survival installation kit | SVK-1 | Mission equipment | CDR-80; seat trial pending |
| Conformal fuel tank pair | CFT-A2 | Optional mission kit | CDR-80; flight pending |
| Sovereign polar communications provision | SPC-1 | Future mission kit | PENDING Canadian service and accreditation |

## Implementation sequence

- Phase 1: OEM data exchange, baseline load survey, weighed aircraft, certification basis, country-of-origin audit and safety assessment.

- Phase 2: component qualification for fluids, seals, battery, heaters, Canadian anti-jam antenna, non-US supply chain and GSE.

- Phase 3: prototype aircraft installation without CFT; ground, EMC, cold-soak and icing tests.

- Phase 4: CFT structural and separation campaign; performance and handling clearance.

- Phase 5: northern operational evaluation, Canadian supportability assessment, sovereignty-content audit and embodiment decision.

CLOSED: Implementation sequence closes with decision gates before flight and before embodiment.

# APPENDIX A — REQUIREMENT DICTIONARY

| ID | Requirement statement | Verification |
| --- | --- | --- |
| AGA-ENV-001 | The aircraft modification shall survive non-operating exposure to -60 deg C for 72 h without permanent degradation. | Chamber test |
| AGA-ENV-002 | All flight equipment shall operate after a 12 h -40 deg C soak when ground preheat is applied. | Chamber + ground test |
| AGA-PWR-001 | Arctic flight additions shall not exceed 11.40 kW peak including margin. | Analysis + HIL |
| AGA-ICE-001 | The electro-thermal system shall not be required for engine-inlet ice protection. | Safety assessment |
| AGA-ICE-002 | Each heater zone shall detect open, short, over-current and over-temperature conditions. | Bench + HIL |
| AGA-NAV-001 | Loss of GNSS shall annunciate within 2 s and select the next valid source automatically. | HIL |
| AGA-NAV-002 | The system shall provide a four-hour GNSS-denied navigation capability within an approved error budget. | Flight test |
| AGA-MAS-001 | No configuration shall exceed 16,500 kg MTOW. | Mass properties |
| AGA-MAS-002 | CFT-equipped representative QRA take-off mass shall not exceed 15,000 kg at the defined mission load. | Mass properties |
| AGA-MAT-001 | Recycled material shall not be used in primary structure, engine rotating parts, or critical fasteners. | Process audit |
| AGA-SOV-001 | Newly introduced Arctic augmentation hardware shall have a target of 0% US origin; baseline Gripen E equipment is grandfathered and shall not be replaced solely because of country of origin; approved exceptions shall keep aggregate US-origin content below 8% by recurring new-kit value. | BOM + origin audit |

## APPENDIX B — MATERIALS, PROCESSES AND SUPPORT DATA

## B.1 Material controls

| Material / process | Control |
| --- | --- |
| NATO H-538 hydraulic fluid | Non-US qualified product; compatibility and seal-swell review before conversion |
| Low-temperature seal compounds | OEM approval, compression set, fluid compatibility, aging |
| Recycled Canadian 6061-T6 / 7075-T7351 | Certified wrought product; chemistry, country-of-origin and mechanical lot record |
| Recycled Canadian or EU 17-4 PH | Cast or sheet product to applicable aerospace specification |
| CNT heater laminate | Batch resistance, bond strength, thermal cycle, lightning and erosion evidence |
| Hydrophobic topcoat | Adhesion, erosion, de-icing fluid, UV, thermal cycle and repairability |
| Adhesives and sealants | Cold lap-shear, peel, fluid exposure and repair procedure |
| Country-of-origin control | Supplier declaration for design authority, manufacture, melt source, software and export-control dependency |

## B.2 Forward spares

- Two CFT shipsets per six-aircraft detachment, one as operational spare.

- One complete heater-zone panel set per aircraft type per FOL, plus controller and test set.

- Low-temperature seals, hoses, tires, brakes, batteries, filters, covers and de-icing consumables for 30 days.

- One Canadian anti-jam antenna, one navigation processor and one heated battery enclosure; no permanent auxiliary satcom LRU is stocked until the Canadian service is accredited.

- GSE spares sufficient to preserve one-aircraft alert with one GPU or pre-oiler unavailable.

## APPENDIX C — ASSUMPTIONS, ACRONYMS AND SOURCE REGISTER

## C.1 Governing assumptions

- Gripen E baseline data are public planning values; OEM data prevail.

- Canada-specific implementation is a conceptual study and does not represent current Canadian fighter procurement policy.

- The CFT is optional and is not credited for a manoeuvre envelope until flight clearance.

- US-origin content means US country of origin, US-controlled design authority, or US export-control dependency. The sourcing rule applies to newly introduced Arctic augmentation hardware and systems; existing baseline Gripen E equipment, including the F414G engine, is grandfathered and is not replaced solely because of country of origin. Standards and public information are not counted as parts; embedded OEM content in new hardware discovered during BOM audit must be replaced or counted under the <8% exception cap.

## C.2 Acronyms

| Acronym | Meaning |
| --- | --- |
| AAR | Air-to-air refuelling |
| AESA | Active electronically scanned array |
| BIT | Built-in test |
| CDR | Critical design review |
| CFT | Conformal fuel tank |
| CNT | Carbon nanotube |
| CRPA | Controlled reception pattern antenna |
| EMC | Electromagnetic compatibility |
| EMCON | Emissions control |
| FOL | Forward operating location |
| GNSS | Global navigation satellite system |
| GSE | Ground support equipment |
| HIL | Hardware in the loop |
| INS | Inertial navigation system |
| LRU | Line-replaceable unit |
| MTOW | Maximum take-off weight |
| NDI | Non-destructive inspection |
| QRA | Quick reaction alert |
| SPC | Sovereign polar communications |
| BOM | Bill of materials |

Saab, Gripen E-series product page: published key facts include 16,500 kg maximum take-off weight, 98 kN maximum thrust, ten hardpoints, air-to-air refuelling, and 15–25 minute combat turnaround; accessed 5 September 2026. https://www.saab.com/products/gripen-e-series

Department of National Defence / Canadian Armed Forces, Infrastructure and Environment, 21 November 2024: upgrades at Inuvik, Yellowknife, Iqaluit, and 5 Wing Goose Bay under NORAD Northern Basing Infrastructure. https://www.canada.ca/en/department-national-defence/corporate/reports-publications/proactive-disclosure/supplementary-estimates-dnd-caf-21-nov-2024/infrastructure-environment.html

Department of National Defence / Canadian Armed Forces, Minister Blair visits Inuvik, 20 August 2024: runway extension to 9,000 ft, lighting and navigational modernization, military arrestor system, and FOL role. https://www.canada.ca/en/department-national-defence/news/2024/08/minister-blair-visits-inuvik-to-highlight-our-north-strong-and-free-and-federal-investments-in-defence-infrastructure.html

NATO STANAG 4370 / AECTP environmental test framework and UK Defence Standard 00-035 Part 3 are used here as non-US tailoring references for low temperature, thermal shock, rain, salt fog and icing tests; aircraft-specific evidence remains required.

EUROCAE, ED-14G Environmental Conditions and Test Procedures for Airborne Equipment, May 2011: European publication defining minimum environmental test conditions and procedures for airborne equipment; accessed 6 September 2026. https://www.eurocae.net/product/ed-14g-environmental-conditions-and-test-procedures-for-airborne-equipment/

NYCO, HYDRAUNYCOIL FH 42: French synthetic aircraft hydraulic fluid approved to NATO Code H-538, designed for -54 deg C to +200 deg C operation; accessed 6 September 2026. https://www.nyco-group.com/products/hydraunycoil-fh-42/

Government of Canada, Minister Joly travels to France to support innovative Canadian industries, 19 June 2025: Canada highlighted its aerospace sector and domestic steel and aluminium capabilities for deeper integration into global aerospace supply chains. https://www.canada.ca/en/innovation-science-economic-development/news/2025/06/minister-joly-travels-to-france-to-support-innovative-canadian-industries.html

International Aerospace Quality Group, 9100 Quality Management Systems — Requirements for Aviation, Space and Defense Organizations; accessed 5 September 2026. https://iaqg.org/standard/9100-qms-requirements-for-aviation-space-and-defense-organizations/

GE Aerospace, F414 Engine: F414 thrust class 22,000 lb, length 154 in, maximum diameter 35 in, airflow 170 lb/s, pressure ratio 30:1; accessed 6 September 2026. https://www.geaerospace.com/military-defense/engines/f414

Hexagon|NovAtel, GAJT-AE3 offers a new standard in mission resilience for airborne applications, Calgary, Canada, 4 March 2026: multi-constellation, multi-frequency anti-jam protection, up to seven nulls per band, protected RF output for modern or legacy GNSS receivers. https://novatel.com/about-us/news-releases/hexagon-novatels-gajt-ae3-offers-a-new-standard-in-mission-resilience-for-airborne-applications

MDA Space, MDA Space signs strategic partnership with the Department of National Defence and Telesat to deliver military satellite communications, 9 December 2025: ESCP-P supports RCAF Arctic sovereignty operations and Canadian industry participation. https://mda.space/article/mda-space-signs-strategic-partnership-with-the-department-of-national-defence-and-telesat-to-deliver-military-satellite-communications

Telesat, Telesat secures Arctic military satcom contract, 4 August 2026: expanded Lightspeed Mil-Ka service for the Canadian Armed Forces is planned from 2028 and covers 65–90 deg N. https://www.telesat.com/press/press-releases/telesat-secures-2-3-billion-arctic-military-satcom-contract-expanding-telesat-lightspeed-network-and-capacity-by-44/

University of Nottingham repository, R&D of carbon nanotubes based nanocomposites for self-heating and de-icing applications: CNT buckypaper integrated into fibre-reinforced polymer showed promising electrical and heating performance for aircraft ice protection; accessed 5 September 2026. https://repository.nottingham.ac.uk/entities/publication/2c27c4a8-6257-4fb4-b75d-b7a0136b1010

National Research Council Canada, Altitude icing wind tunnel research facility: Canadian facilities support icing formation, detection and mitigation research for fixed-wing aircraft, helicopters and related components; accessed 6 September 2026. https://nrc.canada.ca/en/research-development/nrc-facilities/altitude-icing-wind-tunnel-research-facility

Michelin Aircraft, Our Mission and Care & Service Manual excerpts: aircraft tires may be certified for in-service operation to -55 deg C; actual tire selection remains subject to wheel, load, speed, and landing-gear compatibility approval. https://aircraft.michelin.com/our-mission

Aerospace Global News, Saab Gripen E vs Gripen C: Key upgrades explained, 8 November 2025: open-source comparison lists approximately 8.0 t empty mass, approximately 40% more internal fuel, ten hardpoints, and approximately 7.2 t payload. Used as an estimate where Saab does not publish a mass breakdown. https://aerospaceglobalnews.com/news/saab-gripen-e-upgrades-guide/

European Union Aviation Safety Agency, Easy Access Rules for Large Aeroplanes (CS-25), CS 25.1419, CS 25.1420 and Appendix O: public non-US framework for ice protection and supercooled large-drop compliance; accessed 6 September 2026. https://www.easa.europa.eu/en/document-library/easy-access-rules/online-publications/easy-access-rules-large-aeroplanes-cs-25