THE OPTIMAL SPRINTER

PROCUREMENT PACKAGE v6.1

Complete Parts, Materials and Supplier Compendium for the Sprinter Unmanned Loyal Wingman

Companion document to: The Optimal Sprinter Technical Specifications v6.1 (SPRINTER-v6.1-BL)

| Field | Value |
| --- | --- |
| Document number | SPR-AIR-PPP-006 |
| Issue | v6.1 (clean baseline issue) |
| Baseline reference | SPRINTER-v6.1-BL, CDR 70-80% design maturity, 2026-09-06 |
| Programme | Arctic Horizon Programme - Sprinter loyal wingman |
| Fleet basis | Scalable 10-72 aircraft; reference fleet 60 operational + 12 attrition/training = 72 aircraft |
| Cost basis | 2026 CAD; residual USD anchors at controlled planning conversion of 1.35 CAD/USD |
| Security handling | UNCONTROLLED CONCEPT PROCUREMENT PLANNING BASELINE - NOT FOR SOLICITATION, CONTRACT AWARD, OR FLIGHT RELEASE |

Compiled in Canada for Canadian manufacture. Canadian suppliers carry first consideration on every line where a qualified domestic source exists; where no domestic capability exists, sourcing goes to close allies of Canada other than the United States - France, Germany, Sweden, Belgium, the United Kingdom, the Netherlands and Japan - consistent with the Industrial and Technological Benefits policy, the Defence Investment Agency Build-Partner-Buy approach, and the ITAR-free supply-chain requirement of the v6.1 baseline.

# Contents

Document Control and Marking | Executive Summary | Part I - Platform Overview and Programme Basis | Part II - Master Parts Breakdown (Air Vehicle): 2.1 Category A Airframe structure; 2.2 Category B Raw materials, fasteners and consumables; 2.3 Category C Propulsion and inlet/exhaust; 2.4 Category D Landing gear and brakes; 2.5 Category E Flight controls and actuation; 2.6 Category F Electrical power; 2.7 Category G Fuel and thermal systems; 2.8 Category H Avionics core, computing and navigation; 2.9 Category I Sensors and apertures; 2.10 Category J Communications and datalinks; 2.11 Category K Payload bay and stores interface; 2.12 Category L Signature materials and finish; 2.13 Category summary and per-aircraft rollup | Part III - Weapons and Stores Provisioning | Part IV - Ground Support Equipment | Part V - Training System and Personnel | Part VI - Facilities and Infrastructure | Part VII - Supplier Directory and Outreach Pathways | Part VIII - Acquisition Strategy and Procurement Routes | Part IX - Cost Analysis and Fleet Economics (CAD) | Part X - Sustainment Package | Part XI - ITAR and Export-Control Strategy | Part XII - Industrial and Canadian Workshare Plan | Part XIII - Schedule and Long-Lead Plan | Part XIV - Supply Chain and Procurement Risk | Part XV - Quality, Standards and Compliance | Part XVI - Procurement Decision Summary | Appendix A - Traceability to SPRINTER-v6.1-BL | Appendix B - Referenced Standards | Appendix C - Price Basis and Verification Status | Appendix D - Document Certification

# Document Control and Marking

UNCONTROLLED CONCEPT PROCUREMENT PLANNING BASELINE - NOT FOR SOLICITATION, CONTRACT AWARD, OR FLIGHT RELEASE. This package contains concept-stage planning data for the Sprinter unmanned loyal wingman. Quantities, costs and lead times are planning values at 2026 price levels in Canadian dollars. NSN class codes are placeholder classes pending formal codification through the Canadian national codification bureau. Nothing in this document authorises expenditure, and no supplier named here has been contacted or has agreed to supply any listed item.

## Document Control

| Item | Entry |
| --- | --- |
| Title | The Optimal Sprinter Procurement Package v6.1 |
| Document number | SPR-AIR-PPP-006 |
| Controlled baseline | The Optimal Sprinter Technical Specifications v6.1, baseline SPRINTER-v6.1-BL |
| Doctrine reference | The Opening Doors Doctrine, DCT-OD-2026-001 |
| Compilation authority | Programme office, air vehicle procurement cell |
| Review gate | Supports CDR procurement release at 70-80% design maturity |
| Distribution | Programme office, PSPC and DIA liaison, DND project office, shortlisted suppliers under NDA |
| Supersedes | SPR-AIR-PPP-003 (v5.0, obsolete - retained for format reference only) |

## Compilation Methodology

Seven checks govern every line of this package. Any line failing a check was corrected before issue; no register of corrections is maintained, consistent with the clean-baseline policy of the parent specification.

| Check | Rule applied |
| --- | --- |
| 1. Baseline traceability | Every line traces to a section, table or requirement of SPRINTER-v6.1-BL. Untraceable lines were removed. Appendix A carries the map. |
| 2. Quantity consistency | Quantities per aircraft match the controlled mass and configuration model (OEW 2,530 kg, MTOW 3,800 kg, growth ceiling 3,900 kg). |
| 3. Cost reconciliation | Category rollups reconcile to the Part IX flyaway build-up: C$11.85M recurring target (C$14.25M ceiling) at the 72-aircraft rate, on a 92% unit learning curve. |
| 4. Supplier qualification | Every named supplier holds or is able to hold AS9100-class quality registration; special-process sources hold or are able to hold NADCAP. |
| 5. Canadian priority | Where a qualified Canadian source exists, it appears first; allied non-US sources appear where no domestic capability exists; no ITAR-controlled US article appears in any line. |
| 6. Lead-time realism | Lead times reflect vendor-quoted or analogue-programme values; long-lead items are flagged in Part XIII. |
| 7. Price status honesty | Every line carries a status: VENDOR (public price or quotation exists), CALCULATED (transparent arithmetic), ESTIMATE (analogue-based), or PENDING VALIDATION (OEM, export-control or qualification closure required). |

Fastener and standard-part counts in Category B are CDR-level estimates from the structural joint model. Final dash numbers and certified fastener counts are a CDR-100 production-release action; the counts here carry ESTIMATE status and a kitting-service line protects against count error at first article.

# Executive Summary

## Bottom Line

This package lists every part, material and service required to build, equip, support and sustain the Sprinter unmanned loyal wingman at the v6.1 clean baseline, from the Safran M88-2 engine down to the 880-piece structural bolt set, the 3,600-piece washer lot and the safety-wire kit. Twelve procurement categories (A-L) cover the air vehicle in 177 register lines; separate tables cover weapons, ground support equipment, training systems and facilities. Every line names at least one supplier, two wherever a credible second source exists, with Canadian suppliers first wherever domestic capability exists and allied non-US suppliers otherwise. Every line carries a unit cost in Canadian dollars with an explicit price-status tag. The full cost rollup closes to a recurring flyaway of C$11.85M per aircraft at the 72-aircraft production rate (C$14.25M ceiling), on a 92% unit learning curve; smaller fleets cost more per aircraft, and Part IX prices all seventeen candidate fleet sizes from 10 to 72 aircraft on identical rules so the economics are directly comparable.

## Principal Findings

- ITAR-free supply chain achieved: following the v6.1 propulsion decision, no register line carries an ITAR-controlled US article. The engine is the French Safran M88-2 (50/75 kN, 897 kg) with the Eurojet EJ200 as the documented fallback class; weapons are the German Diehl IRIS-T and the European MBDA SmartGlider Light; the single residual US-origin line is an EAR99-class commercial computing module, ruggedised in Ottawa, with a named allied second-source study.

- Canadian content: roughly 48% of recurring flyaway value is Canadian-sourced or Canadian-final-assembled at first article, rising toward 56% at rate production as recycled-material streams localise. This satisfies the ITB policy objective with margin (Part XII).

- Per-unit cost: C$11.85M recurring at the 72-aircraft rate; C$12.72M at 40 aircraft; C$15.03M at a 10-aircraft buy. The all-in per-aircraft figure including non-recurring amortisation, support, weapons, infrastructure and contingency is C$37.3M at 72 aircraft and C$174.2M at 10 aircraft - the strongest argument in this package for a fleet of 40 or more.

- Long-lead critical path: five items drive the schedule: the M88-2 engine and French export authorisation (18 mo, PENDING OEM VALIDATION), the flight-control software certification evidence (18 mo), the COMSEC crypto account (14 mo), the ESM receiver/processor (14 mo) and the starter-generator (14 mo). All five carry mitigation plans in Part XIII.

- Foreign dependency: the small set of lines sole-sourced outside Canada is entirely in France, Germany, Sweden, Belgium, the Netherlands, the UK or Japan: the engine, RTOS licence, precision timing, autonomy silicon (EAR99 residual) and two weapons classes. Each carries a named mitigation: pooled engine inventory shared with the Striker fleet, dual-source avionics, Canadian ruggedisation, and government-to-government weapons channels.

- Recycled-material supply chain: the 402 kg first-article recycled content (15.9% of OEW) is fully sourced through EcoTitanium closed-loop titanium, the Novelis-route recycled aluminium stream in Kingston, and recycled carbon fibre for non-primary parts, with qualification status per Section 6.2 of the baseline. Rate-production target is 20%.

- Cost confidence: the parts-level rollup reproduces the flyaway build-up within stated residuals; installation labour, integration and acceptance shares are declared openly in Part IX rather than forced to zero.

The procurement posture is achievable within the baseline schedule provided the five long-lead items are placed on contract by M12, the French export authorisation for the M88 is secured by M18, and the pooled-engine agreement with the Striker fleet is executed by M18. The M88 supply, export status, gearbox extraction and installed deck remain PENDING OEM VALIDATION per Sections 4 and 13.2 of the baseline; the Eurojet EJ200 fallback applies to every engine-dependent line in this package.

# Part I - Platform Overview and Programme Basis

## 1.1 Platform summary

The Sprinter is a Mach 1.60-class unmanned loyal wingman of Canadian Falcon UAV heritage, optimised as the high-speed door-opener of the Opening Doors doctrine (DCT-OD-2026-001) and teamed with the Gripen E. Principal characteristics per SPRINTER-v6.1-BL:

| Parameter | Value | Baseline reference |
| --- | --- | --- |
| Maximum take-off weight | 3,800 kg (growth ceiling 3,900 kg) | Section 14 |
| Operating empty weight | 2,530 kg | Section 2.3 |
| Usable / retained fuel | 990 kg / 30 kg (JP-8 class) | Section 7.1 |
| Payload | 250 kg internal, two 130 kg stations | Section 10.1 |
| Speed | Mach 1.60 dash for five minutes at 40-45 kft; MMO 1.65; VNE 1.70 | Section 5.2 |
| Engine | Safran M88-2 afterburning low-bypass turbofan, 50/75 kN dry/augmented, 897 kg bare; installed propulsion group 1,090 kg | Section 4 |
| Bay / air-to-air fit | 3.05 m x 0.52 m x 0.46 m; 1 x IRIS-T internal on dual ejector | Sections 2.4, 10.1 |
| Sensors | Six conformal ESM arrays 2-18 GHz; flush EO/IR snapshot; no permanent radar | Section 10 |
| Recycled content | 402 kg / 15.9% of OEW first article; 20% rate target | Section 6.2 |
| Design life | 1,000 flight hours / 500 landings / 250 dash events / 10 years | Section 6.1 |
| Flyaway cost | C$11.85M recurring target / C$14.25M ceiling at 72-aircraft rate; 92% learning curve | Part IX of this package |

## 1.2 Mission set

The reference Opening Doors sortie drives the provisioning quantities of Parts II-VI: 220 nm ingress, 76 nm five-minute dash, 20 nm effects segment, 190 nm egress; about 507 nm total ground track in about 84 minutes at 250 nm mission radius, closing within the 990 kg usable fuel budget with a 180 kg reserve (baseline Section 5.1). Ferry range is about 1,050 nm. The bay opens below Mach 0.90 and 260 kt EAS; the dash is flown with the bay closed. Primary missions are emitter triggering, ESM mapping, cooperative geolocation, decoy presentation, stand-in jamming, high-speed ISR snapshot and limited SEAD/DEAD effects; secondary missions add counter-cruise-missile barrier support, IRIS-T self-defence, post-strike reconnaissance, communications relay and withdrawal screening.

## 1.3 Programme quantities

| Quantity | Value | Note |
| --- | --- | --- |
| Reference fleet | 60 operational + 12 attrition/training = 72 | Five squadrons of 12; rate-production cost basis |
| Candidate fleet sizes | 10, 15, 20, 25, 28, 30, 35, 38, 40, 45, 50, 55, 58, 60, 65, 70, 72 | Seventeen options priced on identical rules in Part IX and Part XVI |
| Pooled engine inventory | 120-aircraft pool | 72 Sprinters + 48 Strikers with 15% float (138 engines); engine life exceeds airframe life |
| Design life | 1,000 FH / 500 landings / 250 dash events | 100 h damage-tolerance inspection start |
| Turnaround | 35 min excluding payload | Module swap 20 min, refuel 12 min, download 15 min |
| Engine cassette removal | 90 min, three-person crew | After access-panel removal |

## 1.4 How to read this package

Part II is the parts compendium proper: twelve category registers in a ten-column format - item, NSN class, part number, description, quantity per aircraft, fleet spares quantity, unit cost in CAD, lead time, price status and suppliers. Spares quantities provision the 72-aircraft reference fleet and engine pool through the first depot cycle; squadron scaling follows the project precedent formula: squadron quantity = (quantity per aircraft x 12) + roundup(spares per 5 aircraft x 12/5). Parts III-VI cover weapons, ground support equipment, training and facilities. Part VII is the supplier directory with outreach pathways. Part VIII covers acquisition strategy. Part IX carries the cost analysis: the per-unit flyaway reconciliation and the seventeen-fleet economics table. Parts X-XII are new to v6.1: the sustainment package, the ITAR/export-control strategy, and the industrial and Canadian workshare plan. Parts XIII-XV cover schedule, risk, and quality/standards. Part XVI closes the package with the procurement decision summary. Appendix A traces every category to the v6.1 baseline; Appendix C defines the price-status tags.

# Part II - Master Parts Breakdown (Air Vehicle)

This part lists every part, material and consumable required to build one Sprinter air vehicle, organised in twelve procurement categories, from the M88-2 engine down to nuts, bolts, rivets, washers, safety wire and sealant packs. The register format follows the project precedent: NSN class codes are placeholders pending codification; part numbers use the SPRN6 series assigned against the v6.1 baseline; quantities per aircraft match the controlled configuration of SPRINTER-v6.1-BL; spares quantities provision the 72-aircraft fleet through the first depot cycle. Unit costs are planning values in 2026 CAD and reconcile by category to the flyaway build-up in Part IX. Price-status tags follow Appendix C.

Supplier codes reference the directory in Part VII, which gives locations, public domains, telephone contacts and outreach pathways. Where two codes appear, the first is the preferred source; Canadian suppliers lead wherever a qualified domestic capability exists, and allied non-US sources fill the remainder - no ITAR-controlled US article appears in any register line. Lines marked PENDING VALIDATION require OEM quotation, export-control or qualification closure before contract placement, consistent with Sections 13.2 and 14 of the baseline.

## 2.1 Category A - Airframe structure

Primary and secondary structure by module, per the six-module production flow of Section 6.4 of the baseline: nose, forward fuselage, centerbody and bay, left wing, right wing, and aft engine cassette. Wing primary structure is qualified BMI carbon laminate; Al-Li 2099/2195 serves bay frames; titanium serves the engine ring and hot-zone attachments for the larger M88 installation. Recycled content enters through secondary brackets and liners first, per the Section 6.3 exclusion boundaries.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nose module structure | 1560 | SPRN6-A-001 | Composite nose module with ESM bay hardpoints and EO/IR window frame | 1 | 8 | 172000 | 10 mo | ESTIMATE | MAG AVC |
| Forward fuselage module | 1560 | SPRN6-A-002 | Composite forward fuselage, bladder-tank bays, cheek access panels | 1 | 6 | 199000 | 10 mo | ESTIMATE | MAG BOM |
| Centerbody and bay module | 1560 | SPRN6-A-003 | Two-spar multi-rib centerbox integral with center fuselage, four bay frames for 3.05 m bay | 1 | 4 | 304100 | 12 mo | ESTIMATE | BOM MAG |
| Wing, left | 1560 | SPRN6-A-004 | BMI carbon skins, two-spar torsion box, integral fuel tank, elevon fittings | 1 | 5 | 152000 | 12 mo | ESTIMATE | BOM AVC |
| Wing, right | 1560 | SPRN6-A-005 | BMI carbon skins, two-spar torsion box, integral fuel tank, elevon fittings | 1 | 5 | 152000 | 12 mo | ESTIMATE | BOM AVC |
| Aft fuselage module | 1560 | SPRN6-A-006 | Aft fuselage shell with titanium engine attachment ring and M88 cassette joint | 1 | 5 | 118000 | 10 mo | ESTIMATE | MAG CYC |
| Canted fin pair | 1560 | SPRN6-A-007 | Left and right canted fins with rudder posts and RF-transparent tips | 1 | 6 | 42000 | 8 mo | ESTIMATE | AVC MAG |
| DSI bump and lip assemblies | 1560 | SPRN6-A-008 | Fixed compression bumps and cowl lips, left and right, treated lips | 2 | 6 | 26500 | 8 mo | ESTIMATE | MAG BOM |
| Payload bay doors | 1560 | SPRN6-A-009 | Composite serrated bay doors, pair, conductive edge preparation | 2 | 6 | 23500 | 8 mo | ESTIMATE | AVC MAG |
| Access panel shipset | 1560 | SPRN6-A-010 | Edge-aligned access panels, approximately 46 per aircraft | 1 | 4 | 34000 | 6 mo | ESTIMATE | AVC SHI |
| Sacrificial lower panels | 1560 | SPRN6-A-011 | Replaceable lower fuselage and bay floor panels | 1 | 8 | 13500 | 6 mo | ESTIMATE | AVC |
| Fail-safe side links | 1560 | SPRN6-A-012 | Engine cassette fail-safe side links, titanium | 2 | 8 | 7400 | 6 mo | ESTIMATE | CYC MAG |
| Fin root fittings | 1560 | SPRN6-A-013 | Titanium fin-root attachment fittings | 4 | 12 | 5300 | 6 mo | ESTIMATE | CYC |
| Wing root splice fittings | 1560 | SPRN6-A-014 | Titanium wing-root splice fittings, both splice planes | 4 | 12 | 8100 | 6 mo | ESTIMATE | CYC MAG |
| Empennage RF-transparent tips | 1560 | SPRN6-A-015 | Quartz-composite fin tip caps | 2 | 8 | 5600 | 6 mo | ESTIMATE | AVC |
| Cradle interface brackets | 1560 | SPRN6-A-016 | Engine alignment-cradle hardpoints and interface brackets | 1 | 4 | 10400 | 6 mo | ESTIMATE | CYC SHI |
| Towing and jacking fittings | 1560 | SPRN6-A-017 | Tow, jack and mooring fitting set | 1 | 4 | 5200 | 6 mo | ESTIMATE | SHI CYC |
| Drain masts and vent fairings | 1560 | SPRN6-A-018 | Drain mast and vent fairing set, engine and fuel zones | 1 | 6 | 4200 | 4 mo | ESTIMATE | SHI AVC |

Category A: per-aircraft recurring value C$1,386,000; fleet spares pool C$7,044,800 (spares quantity x unit cost, summed across register lines).

## 2.2 Category B - Raw materials, fasteners and shop consumables

Per-aircraft material lots and the full fastener bill of material, down to nuts, washers, rivets, grommets, sealants and safety wire. Fastener counts (880 structural bolts and lockbolt pins; 1,960 nuts, collars and nutplates; 2,400 rivets; 480 blind fasteners; 1,200 panel screws; 3,600 washers) derive from the structural joint model at CDR depth and carry ESTIMATE status until CDR-100 drawing release. Recycled feedstock lots (titanium, aluminium, carbon fibre, copper, steel) deliver the 402 kg / 15.9% first-article recycled content tracked in Section 6.2 of the baseline. Composite and coating feedstocks source from allied non-US suppliers - Toray (Japan), Syensqo (Belgium), Mankiewicz (Germany) and AkzoNobel (Netherlands) - preserving the ITAR-free supply chain of Section 4.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMI carbon prepreg lot | 9330 | SPRN6-B-001 | BMI carbon prepreg for wing skins and spars, 185 kg per aircraft | 185 | 90 | 660 | 9 mo | ESTIMATE | TOR SYQ |
| Carbon-epoxy OOA prepreg lot | 9330 | SPRN6-B-002 | Out-of-autoclave carbon-epoxy for secondary structure, 60 kg | 60 | 30 | 445 | 8 mo | ESTIMATE | SYQ TOR |
| CF/PEKK thermoplastic sheet lot | 9330 | SPRN6-B-003 | CF/PEKK sheet for stamp-formed panels, 45 kg | 45 | 22 | 395 | 6 mo | ESTIMATE | TOR SYQ |
| Recycled carbon fiber veil lot | 9330 | SPRN6-B-004 | Recycled-carbon-fiber nonwoven veil for fairings and liners, 40 kg | 40 | 20 | 190 | 6 mo | ESTIMATE | SYQ AVC |
| Al-Li 2099 plate lot | 9535 | SPRN6-B-005 | Al-Li 2099/2195 plate for bay frames and racks, 120 kg | 120 | 60 | 99 | 8 mo | ESTIMATE | CON NOV |
| Recycled 7075 plate lot | 9535 | SPRN6-B-006 | Certified closed-loop recycled 7075 plate, secondary articles, 160 kg | 160 | 80 | 72 | 8 mo | ESTIMATE | NOV CON |
| Ti-6Al-4V plate and bar lot | 9540 | SPRN6-B-007 | Titanium plate and bar for fittings and engine ring, 95 kg | 95 | 48 | 255 | 9 mo | ESTIMATE | ECO CYC |
| Recycled Ti AM powder lot | 9540 | SPRN6-B-008 | Qualified recycled titanium additive-manufacturing powder, 38 kg | 38 | 19 | 435 | 9 mo | PENDING VALIDATION | ECO |
| VAR steel bar lot | 9515 | SPRN6-B-009 | Vacuum-arc-remelt steel bar for pins and gear bolts, 60 kg | 60 | 30 | 58 | 6 mo | ESTIMATE | CYC LIS |
| Copper conductor lot | 6145 | SPRN6-B-010 | Recycled-content copper conductor for harness, 28 kg | 28 | 14 | 34 | 4 mo | ESTIMATE | LIS SHI |
| Structural bolts and lockbolt pins | 5306 | SPRN6-B-011 | Titanium structural bolts and lockbolt pins, 880 per aircraft (CDR estimate) | 880 | 440 | 49 | 6 mo | ESTIMATE | LIS CLA |
| Nuts, collars and nutplates | 5310 | SPRN6-B-012 | Self-locking nuts, collars, nutplates, 1,960 per aircraft | 1960 | 980 | 13 | 6 mo | ESTIMATE | LIS CLA |
| Solid rivets | 5320 | SPRN6-B-013 | 2117/7050 solid rivets, 2,400 per aircraft | 2400 | 1200 | 3 | 4 mo | ESTIMATE | LIS CLA |
| Blind fasteners | 5320 | SPRN6-B-014 | Blind bolts and blind rivets, 480 per aircraft | 480 | 240 | 19 | 5 mo | ESTIMATE | LIS CLA |
| Panel screws and quarter-turns | 5305 | SPRN6-B-015 | Panel screws and quarter-turn fasteners, 1,200 per aircraft | 1200 | 600 | 7 | 4 mo | ESTIMATE | CLA LIS |
| Washers | 5310 | SPRN6-B-016 | Structural washers, cadmium-free, 3,600 per aircraft | 3600 | 1800 | 1 | 4 mo | CALCULATED | CLA SHI |
| Cushioned clamps | 5340 | SPRN6-B-017 | Cushioned loop clamps for wire and tubing, 260 per aircraft | 260 | 130 | 27 | 4 mo | ESTIMATE | CLA SHI |
| Safety wire kit | 5340 | SPRN6-B-018 | Safety wire and lockwire kit per aircraft | 1 | 6 | 680 | 3 mo | ESTIMATE | CLA SHI |
| Cotter pin and retaining kit | 5315 | SPRN6-B-019 | Cotter pins, retaining rings, quick-release pins | 1 | 6 | 400 | 3 mo | ESTIMATE | CLA SHI |
| Fuel-tank sealant pack | 8030 | SPRN6-B-020 | Integral-tank sealant, wing and bay zones | 1 | 6 | 8900 | 4 mo | ESTIMATE | MNK |
| Conductive gap sealant pack | 8030 | SPRN6-B-021 | Conductive sealant for signature-controlled gaps | 1 | 6 | 4100 | 4 mo | ESTIMATE | MNK |
| Structural adhesive film lot | 8040 | SPRN6-B-022 | Film adhesive for bonded assemblies | 1 | 6 | 5900 | 6 mo | ESTIMATE | SYQ TOR |
| Corrosion-inhibiting primer lot | 8010 | SPRN6-B-023 | Chromate-free epoxy primer system | 1 | 6 | 4400 | 4 mo | ESTIMATE | AKZ |
| Grounding braid and bonding straps | 5995 | SPRN6-B-024 | Bonding strap and grounding braid set | 1 | 6 | 3000 | 3 mo | ESTIMATE | SHI CLA |
| Grommet and edge protection kit | 5325 | SPRN6-B-025 | Grommets, edge liners, chafe protection | 1 | 6 | 1200 | 3 mo | ESTIMATE | CLA SHI |
| O-ring and packing kit | 5330 | SPRN6-B-026 | Fuel and thermal system O-rings and packings | 1 | 8 | 2550 | 3 mo | ESTIMATE | SED SFT |
| Bearing and bushing kit | 3120 | SPRN6-B-027 | Airframe bearings and bushings kit | 1 | 6 | 6100 | 5 mo | ESTIMATE | LIS SHI |
| Threaded insert kit | 5325 | SPRN6-B-028 | Threaded inserts for panels and racks | 1 | 6 | 2050 | 4 mo | ESTIMATE | LIS CLA |
| Welding wire and brazing lot | 3439 | SPRN6-B-029 | Titanium and steel welding consumables | 1 | 4 | 1700 | 4 mo | ESTIMATE | CYC |
| Fastener kitting service | 5340 | SPRN6-B-030 | Per-tail fastener kitting with lot traceability certificates | 1 | 72 | 3400 | 3 mo | ESTIMATE | CLA LIS |

Category B: per-aircraft recurring value C$391,082; fleet spares pool C$664,011 (spares quantity x unit cost, summed across register lines).

## 2.3 Category C - Propulsion and inlet/exhaust

The M88-2 engine is procured into the pooled 120-aircraft inventory shared with the Striker fleet (72 Sprinters + 48 Strikers, 15% float, 138 engines); the eleven spare engines listed here are the Sprinter share of the pool. Duct, spill-door, nozzle-adapter and edge-treatment items implement the fixed-geometry DSI inlet of Section 4.2 (0.260 m2 capture per side, 1.45 m S-ducts, 0.88 recovery target at Mach 1.60) and the retained OEM variable-area nozzle of Section 4.3. Engine price, French export authorisation, production status, gearbox extraction and installed deck are PENDING OEM VALIDATION per Sections 4 and 13.2; the Eurojet EJ200 fallback applies to every engine-dependent line, with fuselage length and MTOW rechecked before contract commitment.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M88-2 engine | 2840 | SPRN6-C-001 | Bare engine, 50/75 kN dry/augmented, 897 kg, pooled program asset | 1 | 11 | 2950000 | 18 mo | PENDING VALIDATION | SAE |
| FADEC harness and interface kit | 2990 | SPRN6-C-002 | Dual-channel FADEC aircraft-side harness and interface | 1 | 8 | 72000 | 12 mo | ESTIMATE | SAE |
| Accessory gearbox adaptation kit | 2990 | SPRN6-C-003 | 45 kVA starter-generator pad and cooling adaptation | 1 | 6 | 58000 | 12 mo | PENDING VALIDATION | SAE |
| Engine mounts | 5340 | SPRN6-C-004 | Four primary engine mounts with alignment features | 1 | 8 | 46000 | 8 mo | ESTIMATE | CYC HER |
| Fire seals and barriers | 1560 | SPRN6-C-005 | Engine bay fire seals, shields and barriers | 1 | 8 | 19800 | 6 mo | ESTIMATE | MAG AVC |
| S-duct liner panels | 1560 | SPRN6-C-006 | Left and right S-duct liner panel sets, 1.45 m ducts | 2 | 6 | 28000 | 8 mo | ESTIMATE | MAG BOM |
| Spill doors with actuators | 1560 | SPRN6-C-007 | Low-speed automatic spill doors, two per side | 4 | 10 | 10500 | 6 mo | ESTIMATE | HER MAG |
| Nozzle adapter ring | 2990 | SPRN6-C-008 | Titanium nozzle adapter ring, remains with airframe | 1 | 5 | 41000 | 8 mo | ESTIMATE | CYC MAG |
| Nozzle edge treatment kit | 2990 | SPRN6-C-009 | Serrated outer lip, cooled centerbody fairing, edge-aligned fairing | 1 | 6 | 29500 | 8 mo | PENDING VALIDATION | MAG SAE |
| Quick-disconnect fitting set | 4730 | SPRN6-C-010 | Self-sealing fuel and pneumatic quick-disconnects at cassette plane | 1 | 8 | 13200 | 6 mo | ESTIMATE | SED |
| Engine drains and vent kit | 4710 | SPRN6-C-011 | Drain lines, vent system, cassette-plane connections | 1 | 6 | 9400 | 5 mo | ESTIMATE | SED MAG |
| Engine bay cooling kit | 1560 | SPRN6-C-012 | Bypass-air scoops, cooling ducting and shields | 1 | 6 | 15100 | 6 mo | ESTIMATE | MAG AVC |
| Engine sensor shipset | 6620 | SPRN6-C-013 | Health-monitoring, temperature, vibration and debris sensors | 1 | 8 | 17900 | 8 mo | ESTIMATE | SAE |
| Preservation and blanking kit | 4920 | SPRN6-C-014 | Intake/exhaust blanks and delivery preservation kit | 1 | 10 | 2800 | 3 mo | ESTIMATE | MAG AHN |

Category C: per-aircraft recurring value C$3,372,700; fleet spares pool C$34,979,200 (spares quantity x unit cost, summed across register lines).

## 2.4 Category D - Landing gear and brakes

Tricycle gear sized for the 3,800 kg MTOW and 3.0 m/s sink case of Section 6.1, with winter-rated tires and nose-gear spray deflection per Section 11.1. Carbon-carbon brakes are sized for a 120 kt rejected take-off at MTOW: 7.2 MJ required, 12 MJ installed, a 66% energy margin per Section 5.2. Heroux-Devtek is the Canadian anchor for the full category, with complete fighter landing-gear design authority through its Gripen E work.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nose gear leg | 1620 | SPRN6-D-001 | Nose gear leg, 150 mm stroke class, spray deflector provision | 1 | 6 | 64000 | 12 mo | ESTIMATE | HER |
| Main gear legs | 1620 | SPRN6-D-002 | Main gear legs, left and right, 200 mm stroke class | 2 | 10 | 81000 | 12 mo | ESTIMATE | HER |
| Main wheels | 1630 | SPRN6-D-003 | Main wheels with fuse plugs | 2 | 16 | 9200 | 6 mo | ESTIMATE | HER |
| Nose wheel | 1630 | SPRN6-D-004 | Nose wheel | 1 | 8 | 5700 | 6 mo | ESTIMATE | HER |
| Main tires | 2620 | SPRN6-D-005 | Main tires, winter-friction rated | 2 | 48 | 2350 | 4 mo | ESTIMATE | HER |
| Nose tire | 2620 | SPRN6-D-006 | Nose tire | 1 | 24 | 1620 | 4 mo | ESTIMATE | HER |
| Carbon brake assemblies | 1630 | SPRN6-D-007 | Carbon-carbon brakes, 12 MJ RTO capacity pair | 2 | 14 | 23600 | 8 mo | ESTIMATE | HER |
| Anti-skid valves | 1630 | SPRN6-D-008 | Anti-skid control valves | 2 | 10 | 7400 | 6 mo | ESTIMATE | HER SED |
| Brake control unit | 1630 | SPRN6-D-009 | Digital brake and anti-skid control unit | 1 | 6 | 20700 | 8 mo | ESTIMATE | HER |
| Gear extension EMAs | 1620 | SPRN6-D-010 | Electromechanical extension/retraction actuators | 3 | 12 | 13600 | 9 mo | ESTIMATE | HER |
| Uplock and downlock set | 1620 | SPRN6-D-011 | Uplock/downlock actuators and hooks | 1 | 8 | 10500 | 6 mo | ESTIMATE | HER |
| Gear door actuators | 1620 | SPRN6-D-012 | Gear door actuators and linkages | 1 | 8 | 8500 | 6 mo | ESTIMATE | HER |
| Fuse plug and service kit | 1630 | SPRN6-D-013 | Fuse plugs, oleo seals, service consumables | 1 | 12 | 2000 | 4 mo | ESTIMATE | HER |
| Oleo service kit | 1620 | SPRN6-D-014 | Oleo-pneumatic service kit per gear | 1 | 10 | 4600 | 4 mo | ESTIMATE | HER |
| Gear position sensor set | 1620 | SPRN6-D-015 | Proximity sensors for gear and door position | 1 | 8 | 6900 | 5 mo | ESTIMATE | HER |
| Landing and taxi light set | 6220 | SPRN6-D-016 | LED landing and taxi lights | 1 | 8 | 3900 | 4 mo | ESTIMATE | SHI |

Category D: per-aircraft recurring value C$416,320; fleet spares pool C$2,538,680 (spares quantity x unit cost, summed across register lines).

## 2.5 Category E - Flight controls and actuation

Triplex dissimilar flight-control computers (2+1 lanes), six primary EHA units and two rudder EHA units per Section 8 of the baseline; no central hydraulic system is fitted. Air data combines two heated pitot-static probes with a flush synthetic system; two tactical-grade Safran FOG IMUs provide GNSS-denied continuity against the better-than-2 nm ten-minute drift target of Section 8.2. The flight-control software load is priced per-tail with its DO-178C DAL-A evidence package on a PikeOS-class partitioning RTOS (SYSGO, Germany); this is the 18-month long-lead certification item of Part XIII.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flight-control computer, lane A | 1680 | SPRN6-E-001 | Dissimilar FCC lane A, two per aircraft | 2 | 10 | 40500 | 12 mo | ESTIMATE | CMC CWD |
| Flight-control computer, lane B | 1680 | SPRN6-E-002 | Dissimilar FCC lane B, one per aircraft (triplex 2+1) | 1 | 6 | 40500 | 12 mo | ESTIMATE | CMC CWD |
| EHA primary actuators | 1680 | SPRN6-E-003 | Electrohydrostatic actuators, elevons and drag panels, six per aircraft | 6 | 18 | 34800 | 12 mo | ESTIMATE | HER |
| EHA rudder actuators | 1680 | SPRN6-E-004 | Electrohydrostatic rudder actuators, independent buses | 2 | 8 | 30400 | 12 mo | ESTIMATE | HER |
| Pitot-static probes | 6610 | SPRN6-E-005 | Heated pitot-static probes, two per aircraft | 2 | 10 | 7900 | 6 mo | ESTIMATE | CMC |
| Flush synthetic air data system | 6610 | SPRN6-E-006 | Flush air data system with synthetic computation | 1 | 5 | 47000 | 10 mo | ESTIMATE | CMC CWD |
| FOG inertial units | 6605 | SPRN6-E-007 | Tactical-grade fiber-optic IMUs, two per aircraft | 2 | 10 | 56000 | 12 mo | ESTIMATE | SED CMC |
| Flight termination system | 1680 | SPRN6-E-008 | Independent FTS receiver, battery and destruct-or-land logic | 1 | 6 | 25600 | 10 mo | ESTIMATE | CWD GDC |
| Air data probe set | 6610 | SPRN6-E-009 | Angle-of-attack and sideslip probes | 1 | 6 | 10500 | 6 mo | ESTIMATE | CMC |
| Surface position sensor set | 1680 | SPRN6-E-010 | Control-surface position feedback sensors | 1 | 8 | 7700 | 5 mo | ESTIMATE | HER |
| Actuator electronics kit | 1680 | SPRN6-E-011 | Actuator control electronics spares kit | 1 | 8 | 13500 | 8 mo | ESTIMATE | HER |
| FCS software and evidence | 1680 | SPRN6-E-012 | Per-tail FCS software load with DO-178C DAL-A evidence package | 1 | 0 | 102000 | 18 mo | PENDING VALIDATION | CMC SYS |
| Trim and relief motor set | 1680 | SPRN6-E-013 | Trim and gust-relief motor set | 1 | 6 | 5800 | 5 mo | ESTIMATE | HER |

Category E: per-aircraft recurring value C$731,000; fleet spares pool C$2,812,600 (spares quantity x unit cost, summed across register lines).

## 2.6 Category F - Electrical power

270 V DC architecture per Section 7.2: 45 kVA brushless starter-generator on the M88 accessory gearbox with active rectification (38.2 kW real at 0.85 power factor), solid-state distribution, and two 5.4 kWh LiFePO4 batteries (4.32 kWh usable at 80% depth of discharge, about 35 minutes of flight-critical endurance at the 7.5 kW shedded load). Gearbox power extraction is PENDING OEM VALIDATION; the aircraft-side bus, load table, battery reserve and wiring architecture close at CDR level.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Starter-generator 45 kVA | 6115 | SPRN6-F-001 | Brushless starter-generator, 45 kVA, engine accessory pad | 1 | 8 | 92000 | 14 mo | PENDING VALIDATION | SAE |
| Primary power control unit | 6110 | SPRN6-F-002 | Active rectifier and 270 V DC primary control | 1 | 6 | 36000 | 10 mo | ESTIMATE | CWD SAE |
| 270 V distribution boxes | 6110 | SPRN6-F-003 | Forward and aft 270 V DC distribution boxes | 2 | 8 | 12300 | 8 mo | ESTIMATE | CWD |
| Solid-state power controllers | 6110 | SPRN6-F-004 | SSPC set, 28 channels, load shedding per baseline Section 7.2 | 1 | 6 | 28300 | 8 mo | ESTIMATE | CWD |
| LiFePO4 batteries | 6135 | SPRN6-F-005 | 5.4 kWh LiFePO4 battery modules, forward and aft separated | 2 | 12 | 15800 | 9 mo | ESTIMATE | SFT CWD |
| Battery management units | 6150 | SPRN6-F-006 | Battery management and monitoring units | 2 | 10 | 5200 | 6 mo | ESTIMATE | SFT CWD |
| External power receptacle | 6135 | SPRN6-F-007 | 270 V DC GPU receptacle and interface | 1 | 6 | 3600 | 4 mo | ESTIMATE | CWD |
| Harness shipset, forward | 5995 | SPRN6-F-008 | Forward fuselage wiring harness shipset | 1 | 4 | 19900 | 8 mo | ESTIMATE | CMC MAG |
| Harness shipset, aft | 5995 | SPRN6-F-009 | Aft fuselage and engine-zone harness shipset | 1 | 4 | 18100 | 8 mo | ESTIMATE | CMC MAG |
| Harness shipset, wings | 5995 | SPRN6-F-010 | Wing and empennage harness shipset | 1 | 4 | 13700 | 8 mo | ESTIMATE | CMC MAG |
| Bay power harness | 5995 | SPRN6-F-011 | Payload bay 270 V/28 V station feeds | 1 | 6 | 10100 | 6 mo | ESTIMATE | CMC |
| D38999 connector lot | 5935 | SPRN6-F-012 | Circular connector lot with backshells, full traceability | 1 | 8 | 22500 | 6 mo | ESTIMATE | CLA LIS |
| Bonding and lightning kit | 5995 | SPRN6-F-013 | Grounding, bonding and lightning protection kit | 1 | 6 | 5100 | 4 mo | ESTIMATE | SHI CMC |
| Static discharger set | 5920 | SPRN6-F-014 | Trailing-edge static dischargers | 1 | 10 | 3100 | 4 mo | ESTIMATE | LIS SHI |
| Bus contactor kit | 6110 | SPRN6-F-015 | Bus contactors and circuit protection kit | 1 | 8 | 7200 | 5 mo | ESTIMATE | CWD |
| Battery conditioning kit | 6135 | SPRN6-F-016 | Battery heaters and Arctic conditioning kit | 1 | 6 | 5600 | 5 mo | ESTIMATE | SFT AHN |

Category F: per-aircraft recurring value C$331,800; fleet spares pool C$2,083,600 (spares quantity x unit cost, summed across register lines).

## 2.7 Category G - Fuel and thermal systems

Four-tank architecture per Section 7.1: 800 L wing integral plus 460 L fuselage bladders, 1,260 L geometric, 990 kg usable, single-point pressure refuelling in 12 minutes, reticulated foam and nitrogen-enriched-air inerting. The thermal system implements the Section 7 sink budget: fuel heat sink 11.6 MJ, PAO loop and structure 2.9 MJ, total 14.5 MJ against the 12.6 MJ dash demand (42 kW for 300 seconds), with the 60 C bulk-fuel and 120 C wall limits. Fuel-system, inerting and thermal components source from the Safran group (France), keeping this category ITAR-free.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integral tank coating lot | 8030 | SPRN6-G-001 | Wing integral-tank sealant and corrosion coating system | 1 | 4 | 15500 | 5 mo | ESTIMATE | MNK |
| Forward bladder tank | 1560 | SPRN6-G-002 | Forward fuselage bladder, 260 L, self-sealing | 1 | 6 | 22500 | 8 mo | ESTIMATE | MAG SED |
| Aft bladder tank | 1560 | SPRN6-G-003 | Aft fuselage bladder, 200 L, self-sealing | 1 | 6 | 19700 | 8 mo | ESTIMATE | MAG SED |
| Boost pumps | 2910 | SPRN6-G-004 | Tank boost pumps feeding single engine manifold | 3 | 12 | 13700 | 8 mo | ESTIMATE | SED |
| Crossfeed valves | 4820 | SPRN6-G-005 | Crossfeed and tank-group isolation valves | 4 | 12 | 6600 | 6 mo | ESTIMATE | SED |
| Pressure refuel receptacle | 1560 | SPRN6-G-006 | Single-point pressure refuelling receptacle, 12 min fill | 1 | 6 | 10500 | 6 mo | ESTIMATE | SED |
| Defuel valve set | 4820 | SPRN6-G-007 | Gravity defuel and transfer valve set | 1 | 6 | 6000 | 5 mo | ESTIMATE | SED |
| Fuel gauging probes | 6680 | SPRN6-G-008 | Fuel quantity and densitometry probe set | 1 | 8 | 12000 | 6 mo | ESTIMATE | SED |
| Reticulated foam kit | 1560 | SPRN6-G-009 | Tank protection foam, all tank groups | 1 | 10 | 7900 | 4 mo | ESTIMATE | MAG AVC |
| NEA inerting module | 1660 | SPRN6-G-010 | Nitrogen-enriched-air ullage inerting module | 1 | 6 | 45000 | 10 mo | ESTIMATE | SED |
| PAO loop pumps | 2930 | SPRN6-G-011 | PAO liquid-loop pumps, dual pumps two zones | 2 | 10 | 9500 | 7 mo | ESTIMATE | SED |
| Cold plate set | 1660 | SPRN6-G-012 | Avionics and bay-station cold plates | 1 | 6 | 24100 | 7 mo | ESTIMATE | CWD SED |
| Fuel-PAO heat exchangers | 1660 | SPRN6-G-013 | Fuel heat-sink exchangers, 120 C wall limit | 2 | 8 | 17600 | 8 mo | ESTIMATE | SED |
| PAO reservoir set | 2930 | SPRN6-G-014 | PAO reservoir and accumulator | 1 | 6 | 7400 | 5 mo | ESTIMATE | SED |
| Thermal valve set | 4820 | SPRN6-G-015 | Thermal modulating and bypass valves | 1 | 8 | 10400 | 6 mo | ESTIMATE | SED |
| Filter shipset | 4330 | SPRN6-G-016 | Fuel and PAO filter shipset | 1 | 10 | 4900 | 4 mo | ESTIMATE | SED |
| Thermal sensor set | 6685 | SPRN6-G-017 | Fuel and loop temperature/pressure sensors | 1 | 8 | 6200 | 4 mo | ESTIMATE | SED |
| Water scavenge kit | 2910 | SPRN6-G-018 | Water scavenge and drain valves, Arctic water management | 1 | 8 | 3400 | 4 mo | ESTIMATE | SED |

Category G: per-aircraft recurring value C$317,200; fleet spares pool C$1,736,600 (spares quantity x unit cost, summed across register lines).

## 2.8 Category H - Avionics core, computing and navigation

The 40 TOPS-class mission computer, 16 TB encrypted recorder, precision time server, dual GNSS with CRPA anti-jam, and the mission network per Section 10 of the baseline. The Jetson AGX Orin 64GB production module carries a VENDOR public distribution price; ruggedisation, chassis and qualification by Curtiss-Wright Ottawa are priced in the same line - this is the package’s single residual US-origin line (EAR99-class commercial silicon, not ITAR-controlled), with the mitigation strategy of Part XI. Precision timing sources from Safran (France), replacing the US chip-scale atomic clock of earlier baselines. Software licences are per-tail recurring items.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mission computer | 7021 | SPRN6-H-001 | 40 TOPS-class rugged mission computer; Jetson AGX Orin 64GB base module (public distribution anchor US$3,199), ruggedised | 1 | 6 | 62000 | 12 mo | VENDOR | CWD NVD |
| Encrypted recorder 16 TB | 7025 | SPRN6-H-002 | 16 TB encrypted mission data recorder with root of trust | 1 | 6 | 23000 | 10 mo | ESTIMATE | CWD |
| Mission network switches | 7025 | SPRN6-H-003 | Rugged 10 GbE mission network switches | 2 | 8 | 13300 | 8 mo | ESTIMATE | CWD |
| Precision time server | 6605 | SPRN6-H-004 | GNSS-disciplined oscillator with atomic-clock-class holdover for geolocation timing | 1 | 6 | 36500 | 12 mo | ESTIMATE | SED CMC |
| GNSS receivers | 5826 | SPRN6-H-005 | Dual multi-band GNSS receivers | 2 | 8 | 18000 | 8 mo | ESTIMATE | NOVT |
| CRPA antenna set | 5985 | SPRN6-H-006 | Controlled-reception anti-jam GNSS antenna set | 1 | 6 | 26200 | 9 mo | ESTIMATE | CMC NOVT |
| Remote data concentrators | 7025 | SPRN6-H-007 | Vehicle I/O and remote data concentrator units | 4 | 12 | 8500 | 8 mo | ESTIMATE | CWD |
| Hardware root of trust module | 7025 | SPRN6-H-008 | Signed-load and key-storage security module | 1 | 6 | 13700 | 8 mo | ESTIMATE | CWD GDC |
| Mission racks, recycled Al | 5975 | SPRN6-H-009 | Recycled-content aluminium avionics racks and trays | 1 | 6 | 9200 | 6 mo | ESTIMATE | CYC SHI |
| Data loader and maintenance port | 7025 | SPRN6-H-010 | Mission data loader and signed maintenance port | 1 | 6 | 10600 | 6 mo | ESTIMATE | CWD |
| Mission software licence | 7030 | SPRN6-H-011 | Per-tail mission software load and integration licence | 1 | 0 | 66000 | 14 mo | ESTIMATE | CWD CMC |
| RTOS licence | 7030 | SPRN6-H-012 | PikeOS-class safety-critical partitioning RTOS, per-tail | 1 | 0 | 31000 | 12 mo | ESTIMATE | SYS |

Category H: per-aircraft recurring value C$374,800; fleet spares pool C$1,439,600 (spares quantity x unit cost, summed across register lines).

## 2.9 Category I - Sensors and apertures

Six conformal ESM arrays covering 2-18 GHz with time-synchronized bearing output, and the flush MWIR/visible EO/IR snapshot sensor with treated window and retractable cover, per Section 10. The EO/IR price class is anchored to the public MX-10 procurement record; the flush build is a WESCAM development item, manufactured in Burlington, Ontario. No permanent large AESA radar is procured; the optional compact X-band nose module remains outside the baseline and appears in no register line.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ESM conformal arrays | 5865 | SPRN6-I-001 | Six conformal ESM antenna arrays, 2-18 GHz, forward and side sectors | 6 | 16 | 56000 | 12 mo | ESTIMATE | MDA |
| ESM receiver/processor | 5865 | SPRN6-I-002 | ESM receiver and time-synchronized bearing processor | 1 | 5 | 180000 | 14 mo | ESTIMATE | MDA |
| ESM integration frames | 1560 | SPRN6-I-003 | Leading-edge ESM array integration frames | 6 | 12 | 9500 | 8 mo | ESTIMATE | MDA MAG |
| ESM calibration kit | 5865 | SPRN6-I-004 | Phase-center calibration and test coupler kit | 1 | 6 | 19800 | 8 mo | ESTIMATE | MDA |
| Flush EO/IR snapshot sensor | 5855 | SPRN6-I-005 | Flush MWIR and visible snapshot camera; public MX-10-class procurement record anchors price class | 1 | 5 | 305000 | 12 mo | ESTIMATE | WES |
| EO/IR window and cover | 5855 | SPRN6-I-006 | Treated EO/IR window with retractable cover | 1 | 6 | 36900 | 9 mo | ESTIMATE | WES |
| EO/IR cover actuator | 1680 | SPRN6-I-007 | Window cover actuator and controller | 1 | 6 | 10500 | 6 mo | ESTIMATE | WES HER |
| Sensor harness and fiber set | 5995 | SPRN6-I-008 | Sensor harness, dual fiber and Ethernet shipset | 1 | 6 | 17600 | 6 mo | ESTIMATE | CMC |
| HUMS sensor kit | 6620 | SPRN6-I-009 | Health and usage monitoring accelerometers and interface | 1 | 8 | 13700 | 6 mo | ESTIMATE | CWD |

Category I: per-aircraft recurring value C$976,500; fleet spares pool C$4,053,400 (spares quantity x unit cost, summed across register lines).

## 2.10 Category J - Communications and datalinks

Directional Ku-band primary C2, UHF/L-band backup, and the 60 GHz short-range intra-flight link per Section 8.2 of the baseline. Cryptographic units are COMSEC-controlled items procured through the Canadian Cryptographic Modernization Program account and carry PENDING VALIDATION status until the account opens. SATCOM is an optional nose module, not a baseline fit, and appears in no register line.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ku directional C2 terminal | 5820 | SPRN6-J-001 | Directional Ku-band primary C2 terminal | 1 | 5 | 73000 | 12 mo | ESTIMATE | GDC MDA |
| Ku steerable antenna | 5985 | SPRN6-J-002 | Ku pointing antenna assembly | 1 | 6 | 47000 | 12 mo | ESTIMATE | MDA |
| UHF/L backup radio | 5820 | SPRN6-J-003 | UHF/L-band backup command and health radio | 1 | 6 | 19800 | 10 mo | ESTIMATE | GDC |
| 60 GHz intra-flight link | 5820 | SPRN6-J-004 | Short-range 60 GHz intra-flight link set | 1 | 6 | 27800 | 10 mo | ESTIMATE | MDA |
| Cryptographic units | 5810 | SPRN6-J-005 | COMSEC-controlled crypto units, government account | 2 | 8 | 33200 | 14 mo | PENDING VALIDATION | GDC |
| Antenna set | 5985 | SPRN6-J-006 | Blade and patch antenna set, all bands | 1 | 8 | 13300 | 6 mo | ESTIMATE | MDA CMC |
| RF cable set | 5995 | SPRN6-J-007 | RF cable, waveguide and coupler set | 1 | 8 | 10300 | 5 mo | ESTIMATE | CMC |
| RF conditioning set | 5985 | SPRN6-J-008 | Diplexers, filters and RF conditioning modules | 1 | 8 | 8400 | 5 mo | ESTIMATE | MDA |
| Crypto fill interface | 5810 | SPRN6-J-009 | Cryptographic fill and data transfer interface | 1 | 4 | 12000 | 10 mo | PENDING VALIDATION | GDC |

Category J: per-aircraft recurring value C$278,000; fleet spares pool C$1,502,200 (spares quantity x unit cost, summed across register lines).

## 2.11 Category K - Payload bay and stores interface

The 3.05 m x 0.52 m x 0.46 m bay (0.73 m3 usable) with two 130 kg stations per Section 10.1: racks, modular interface plates, release units, 14-inch suspension, umbilicals, station power and 15 kW cold-plate cooling, door actuation and conductive seals. The AAM-1 fit suspends one IRIS-T across both stations on a dual ejector with the wing plane horizontal inside the 0.52 m bay width and 114 mm longitudinal clearance. The release control unit implements dual-governed consent with geofence checks; no autonomous kinetic release exists per Section 8.1. Store separation is PENDING VALIDATION until the Section 10.2 test sequence closes.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bay rack assemblies | 1680 | SPRN6-K-001 | Al-Li bay racks with certified recycled-content brackets, two stations | 2 | 8 | 18500 | 8 mo | ESTIMATE | CYC MAG |
| Modular interface plates | 1680 | SPRN6-K-002 | Single modular interface plate per station: power, data, cooling, release | 2 | 8 | 14400 | 8 mo | ESTIMATE | CMC CWD |
| Store release units | 1440 | SPRN6-K-003 | Release/ejector units, 130 kg station rating; AAM-1 dual ejector for IRIS-T | 2 | 10 | 27500 | 10 mo | ESTIMATE | HER |
| Suspension lug sets | 1440 | SPRN6-K-004 | 14-inch NATO-pattern suspension lug sets | 2 | 10 | 3800 | 5 mo | ESTIMATE | HER SHI |
| Store umbilical set | 5935 | SPRN6-K-005 | Store umbilical connectors and cable set | 1 | 8 | 9900 | 6 mo | ESTIMATE | CLA CMC |
| Station power units | 6110 | SPRN6-K-006 | 270 V DC / 28 V DC station power conditioning units | 2 | 8 | 12600 | 8 mo | ESTIMATE | CWD |
| Cold-plate station interfaces | 1660 | SPRN6-K-007 | 15 kW cold-plate station interfaces | 2 | 8 | 10800 | 7 mo | ESTIMATE | CWD SED |
| Bay door actuators | 1680 | SPRN6-K-008 | Bay door EMAs, opening below Mach 0.90 and 260 kt EAS | 2 | 10 | 16600 | 9 mo | ESTIMATE | HER |
| Bay door latch set | 5340 | SPRN6-K-009 | Door latches, locks and position switches | 1 | 8 | 7200 | 5 mo | ESTIMATE | HER SHI |
| Conductive door seals | 5330 | SPRN6-K-010 | Serrated conductive door seal set | 1 | 10 | 6600 | 4 mo | ESTIMATE | MNK SHI |
| Bay liner panels | 1560 | SPRN6-K-011 | Recycled-CF/Al bay liner and sacrificial floor panels | 1 | 8 | 8100 | 6 mo | ESTIMATE | AVC |
| Release control unit | 1440 | SPRN6-K-012 | Dual-governed release consent and discrete chain controller | 1 | 6 | 20800 | 10 mo | ESTIMATE | CWD GDC |
| Authorization interlock module | 1440 | SPRN6-K-013 | Geofence and authorization interlock module; no autonomous kinetic release | 1 | 6 | 10600 | 10 mo | ESTIMATE | CWD |

Category K: per-aircraft recurring value C$271,600; fleet spares pool C$1,385,400 (spares quantity x unit cost, summed across register lines).

## 2.12 Category L - Signature materials and finish

Replaceable RAM leading-edge strips, conductive gap seals, treated inlet lip liners, conformal spray and the gray-blue low-contrast finish per Section 9 of the baseline, sourced from Mankiewicz (Germany) and AkzoNobel (Netherlands) with Magellan application. No certified RCS value is claimed anywhere in this package, consistent with the baseline claim boundary; acceptance reflectometry coupons travel with each material lot.

| Item | NSN class | Part number | Description | Qty/AC | Spares | Unit cost (C$) | Lead | Status | Supplier(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAM leading-edge strips | 1560 | SPRN6-L-001 | Replaceable radar-absorbent leading-edge strip set | 1 | 10 | 41500 | 9 mo | ESTIMATE | MAG MNK |
| Conductive gap seals | 8030 | SPRN6-L-002 | Conductive gap seals and edge tapes lot | 1 | 8 | 15200 | 5 mo | ESTIMATE | MNK |
| Treated inlet lip liners | 1560 | SPRN6-L-003 | Treated inlet lip liners, left and right | 2 | 8 | 18000 | 7 mo | ESTIMATE | MAG MNK |
| Conformal RAM spray lot | 8010 | SPRN6-L-004 | Conformal absorbent spray material lot | 1 | 6 | 13500 | 6 mo | ESTIMATE | MNK |
| Flush filler lot | 8030 | SPRN6-L-005 | Edge-alignment and flush fastener filler lot | 1 | 8 | 5100 | 4 mo | ESTIMATE | MNK |
| Low-contrast finish system | 8010 | SPRN6-L-006 | Gray-blue low-contrast polyurethane finish system | 1 | 8 | 10300 | 4 mo | ESTIMATE | AKZ |
| Anti-glint nose treatment | 8010 | SPRN6-L-007 | Anti-glint nose and window-surround treatment | 1 | 6 | 4200 | 4 mo | ESTIMATE | MNK |
| OML fastener caps | 5325 | SPRN6-L-008 | Flush outer-mold-line fastener caps lot | 1 | 8 | 2600 | 4 mo | ESTIMATE | CLA SHI |
| Reflectometry coupon set | 4920 | SPRN6-L-009 | Per-lot acceptance reflectometry test coupons | 1 | 10 | 7300 | 4 mo | CALCULATED | MAG |

Category L: per-aircraft recurring value C$135,700; fleet spares pool C$1,003,800 (spares quantity x unit cost, summed across register lines).

## 2.13 Category summary and per-aircraft rollup

| Cat. | Category | Register lines | Per-aircraft (C$) | Fleet spares pool (C$) |
| --- | --- | --- | --- | --- |
| A | Airframe structure | 18 | 1,386,000 | 7,044,800 |
| B | Raw materials, fasteners, consumables | 30 | 391,082 | 664,011 |
| C | Propulsion and inlet/exhaust | 14 | 3,372,700 | 34,979,200 |
| D | Landing gear and brakes | 16 | 416,320 | 2,538,680 |
| E | Flight controls and actuation | 13 | 731,000 | 2,812,600 |
| F | Electrical power | 16 | 331,800 | 2,083,600 |
| G | Fuel and thermal systems | 18 | 317,200 | 1,736,600 |
| H | Avionics core, computing, navigation | 12 | 374,800 | 1,439,600 |
| I | Sensors and apertures | 9 | 976,500 | 4,053,400 |
| J | Communications and datalinks | 9 | 278,000 | 1,502,200 |
| K | Payload bay and stores interface | 13 | 271,600 | 1,385,400 |
| L | Signature materials and finish | 9 | 135,700 | 1,003,800 |
| - | Total | 177 | 8,982,702 | 61,243,891 |

Per-aircraft recurring parts value and fleet spares pool across Categories A-L.

The per-aircraft parts subtotal of C$8,982,702 across 177 register lines is the equipment-and-material basis of the flyaway reconciliation in Part IX; assembly labour, integration, acceptance test, overhead and profit close the balance to the C$11.85M controlled target. The spares pool of C$61.2M sits within the initial-spares planning line of the programme rollup, with the balance held for engine-pool service bulletins, expendables re-stock and price-escalation reserve.

# Part III - Weapons and Stores Provisioning

Weapons are procured on a separate account outside the C$11.85M flyaway and the programme rollup. The Sprinter bay accepts only compact stores that fit the 3.05 m x 0.52 m x 0.46 m envelope within the two 130 kg stations, per Section 10.2 of the baseline. The air-to-air fit is the in-service Diehl IRIS-T (2.936 m length, 127 mm diameter, approximately 25 km range, Mach 3), already integrated on Gripen E, which gives Sprinter a validated self-defence weapon with no developmental-interceptor dependence and no US export-control exposure. The strike fit is the European MBDA SmartGlider Light (2.0 m, 120 kg class). AMRAAM-class and full-length decoy stores do not close in this bay and are excluded by role separation - Gripen carries long air-to-air weapons and Striker receives the larger internal-strike role. Initial provisioning supports combat loadouts for the reference 72-aircraft fleet plus training and expenditure allowance; fleet-size scaling uses the C$0.706M per-aircraft planning allowance of Part IX.

| Store | Part number | Description | Qty | Unit cost (C$) | Line total (C$) | Supplier |
| --- | --- | --- | --- | --- | --- | --- |
| IRIS-T short-range air-to-air missile | SPRN6-W-001 | In-service IRIS-T, AAM-1 fit on dual ejector; Gripen E common weapon | 62 | 545,000 | 33,790,000 | DHL |
| SmartGlider Light compact glide weapon | SPRN6-W-002 | 2.0 m, 120 kg-class compact glide weapon for KIN-1 module | 12 | 1,150,000 | 13,800,000 | MBD |
| Inert training store | SPRN6-W-003 | Inert mass-and-form store for loading and separation training | 12 | 30,000 | 360,000 | DHL / HER |
| Towed-decoy expendable elements | SPRN6-W-004 | DEC-1 module towed and free expendable RF elements | 48 | 58,000 | 2,784,000 | MDA / GDC |
| Store umbilical and test cables | SPRN6-W-005 | Weapon-side umbilicals and test cable set | 8 | 9,900 | 79,200 | CLA / CMC |
| Initial weapons provisioning total | - | Reference 72-aircraft fleet loadouts plus allowance | - | - | 50,813,200 | - |

Weapons initial provisioning, separate account, 2026 CAD planning values.

Separation certification status: captive carry closes by geometry, mass and CG. Release does not close until CFD, wind-tunnel, ground-ejection and flight-release testing establish safe trajectories (baseline Section 10.2, PENDING VALIDATION). All release events occur below Mach 0.90 and 260 kt EAS with dual-governed Gripen authorization; no autonomous kinetic release exists (baseline Section 8.1). Both weapons classes route government-to-government through DND/PSPC with German and European Union export authorisations - no US FMS case is required anywhere in this package.

# Part IV - Ground Support Equipment

GSE supports the 35-minute turnaround (module swap 20 minutes, refuel 12 minutes, download 15 minutes), the 90-minute engine cassette removal and the -40 C Arctic start case of Section 11.1 of the baseline, at the main operating base and four forward operating locations. Quantities provision the reference fleet at one main base and four forward locations; smaller fleets scale per the support model of Part IX.

| Equipment | Part number | Description | Qty | Unit cost (C$) | Line total (C$) | Supplier |
| --- | --- | --- | --- | --- | --- | --- |
| Mission planning station | SPRN6-GSE-001 | Gripen-interface mission planning and debrief station | 4 | 1,550,000 | 6,200,000 | CAE / GDC |
| Ground control element shelter | SPRN6-GSE-002 | Transportable control and telemetry shelter | 2 | 2,750,000 | 5,500,000 | GDC / MDA |
| Engine change cradle and rails | SPRN6-GSE-003 | Cassette removal cradle and rail set, 90 min removal target | 4 | 158,000 | 632,000 | HER / CYC |
| Mission module lift | SPRN6-GSE-004 | Standardized 20-minute module swap lift, two-person crew | 6 | 40,000 | 240,000 | HER |
| Tow tractor | SPRN6-GSE-005 | Aircraft tow tractor, MTOW class | 4 | 175,000 | 700,000 | AHN |
| Tow bar | SPRN6-GSE-006 | Tow bar with shear-pin protection | 8 | 10,200 | 81,600 | HER / SHI |
| 270 V DC ground power unit | SPRN6-GSE-007 | GPU matching aircraft external power interface | 8 | 72,000 | 576,000 | AHN / CWD |
| Battery conditioner/charger | SPRN6-GSE-008 | LiFePO4 conditioning and charge station | 6 | 23,500 | 141,000 | SFT / AHN |
| Arctic portable heater | SPRN6-GSE-009 | Herman Nelson-class indirect-fired heater for -40 C starts | 10 | 19,800 | 198,000 | AHN |
| Air data test set | SPRN6-GSE-010 | Pitot-static and flush air data test set | 3 | 92,000 | 276,000 | CMC / CWD |
| Pressure refuelling cart | SPRN6-GSE-011 | Single-point pressure refuelling cart | 5 | 133,000 | 665,000 | AHN |
| Defuel cart | SPRN6-GSE-012 | Gravity/pump defuel cart for engine removal support | 3 | 79,000 | 237,000 | AHN |
| Inerting service cart | SPRN6-GSE-013 | NEA/nitrogen service cart for ullage system | 4 | 62,000 | 248,000 | SED / AHN |
| PAO service cart | SPRN6-GSE-014 | Thermal-loop PAO service and fill cart | 4 | 49,000 | 196,000 | SED |
| EO/IR window handling kit | SPRN6-GSE-015 | Window replacement kit, 30-minute task support | 4 | 12,800 | 51,200 | WES |
| Composite repair kit | SPRN6-GSE-016 | Field composite repair kit with hot bonder | 5 | 36,000 | 180,000 | MAG / AVC |
| Fastener and tooling kit | SPRN6-GSE-017 | Per-FOL fastener stock and installation tooling | 5 | 30,000 | 150,000 | CLA / LIS |
| BIT download station | SPRN6-GSE-018 | Post-flight health download station, 15-minute task | 8 | 22,500 | 180,000 | CWD |
| Engine run restraint kit | SPRN6-GSE-019 | Run-up restraint and mooring kit | 3 | 45,000 | 135,000 | HER |
| Cover and mooring set | SPRN6-GSE-020 | Intake/exhaust covers, pitot covers, tiedown kit | 12 | 7,300 | 87,600 | AHN / SHI |
| Jack set | SPRN6-GSE-021 | Axle and airframe jack set | 4 | 40,500 | 162,000 | HER |
| Wheel and brake change kit | SPRN6-GSE-022 | Wheel/brake change tooling, 25-minute tire task | 6 | 15,500 | 93,000 | HER |
| Weapon loading adapter | SPRN6-GSE-023 | 130 kg station loading adapter for compact stores and IRIS-T | 6 | 25,600 | 153,600 | HER / CYC |
| ESM/EO/IR test bench | SPRN6-GSE-024 | Depot-level sensor test bench | 2 | 410,000 | 820,000 | MDA / WES |
| GSE total | - | Main operating base plus four forward locations | - | - | 17,903,000 | - |

Ground support equipment provisioning.

The mission planning stations and control shelters are ITB-eligible Canadian procurements. The engine change cradle pairs with the airframe-side alignment hardpoints (Category A, SPRN6-A-016) to deliver the 90-minute cassette removal of Section 2.4 of the baseline - the M88’s public one-hour removal and reinstallation capability underwrites this target. The portable heater line uses Aerotech Herman Nelson of Winnipeg, an existing Canadian Armed Forces supplier, for the -40 C cold-soak start case.

# Part V - Training System and Personnel

Training devices and courseware implement the operator and maintainer syllabi, including Gripen-teamed MUM-T rehearsal and the authorization/abort drill set of Section 8.1 of the baseline. CAE is the Canadian anchor for the full category.

| Item | Part number | Description | Qty | Unit cost (C$) | Line total (C$) | Supplier |
| --- | --- | --- | --- | --- | --- | --- |
| Full-mission simulator | SPRN6-T-001 | Gripen-teamed full-mission simulator with MUM-T rehearsal | 2 | 9,800,000 | 19,600,000 | CAE |
| Part-task procedural trainer | SPRN6-T-002 | Mission-system and bay-operation procedural trainers | 4 | 1,600,000 | 6,400,000 | CAE |
| Maintenance training rig | SPRN6-T-003 | Airframe mock-up with access-panel and LRU tasks | 2 | 2,200,000 | 4,400,000 | CAE / MAG |
| Cassette removal trainer | SPRN6-T-004 | M88 engine cassette removal and alignment trainer | 1 | 1,100,000 | 1,100,000 | CAE / HER |
| Courseware and CBT suite | SPRN6-T-005 | Operator and maintainer courseware, computer-based training | 1 | 3,200,000 | 3,200,000 | CAE |
| MUM-T rehearsal module | SPRN6-T-006 | Gripen E cooperative tactics rehearsal software module | 2 | 1,800,000 | 3,600,000 | CAE / SAA |
| EW tactics laboratory | SPRN6-T-007 | ESM/EW mission tactics laboratory | 1 | 2,600,000 | 2,600,000 | MDA / CAE |
| Instructor operator stations | SPRN6-T-008 | Instructor operator station set | 6 | 420,000 | 2,520,000 | CAE |
| Debrief and replay system | SPRN6-T-009 | Mission debrief and 16 TB recorder replay system | 2 | 850,000 | 1,700,000 | CWD / CAE |
| AR maintainer aids | SPRN6-T-010 | Augmented-reality maintenance aid headsets and content | 12 | 48,000 | 576,000 | CAE |
| Training system total | - |  | - | - | 45,696,000 | - |

Training system provisioning.

## 5.1 Personnel provisioning

| Cadre | Per squadron | Fleet (5 squadrons) | Note |
| --- | --- | --- | --- |
| Mission operators | 8 | 40 | Gripen-teamed qualification; simulator-led syllabus |
| Mission commanders (Gripen aircrew) | 4 | 20 | Dual-seat or linked-station release authority |
| Line maintainers | 14 | 70 | Turnaround, module swap, refuel, download tasks |
| Depot artisans | - | 24 | IMP/Cascade depot line activation at M40 |
| Instructors | 2 | 10 | CAE-supported instructor cadre |
| COMSEC custodians | 2 | 10 | Crypto account and fill-device control |

Personnel provisioning basis. Training device quantities above support this cadre; Part X carries the through-life support manpower model.

# Part VI - Facilities and Infrastructure

Facilities below are programme capital items, tracked in the infrastructure line of the programme rollup. The Mirabel composite manufacturing cell is the rate-production anchor and the physical centre of the Canadian build, matching the determinant-assembly flow of Section 6.4 of the baseline (1,800 assembly hours at unit 24, 1,250 hours at unit 72, 92% first-pass-yield class).

| Facility | Location | Description | Qty | Unit cost (C$) | Line total (C$) |
| --- | --- | --- | --- | --- | --- |
| Composite manufacturing cell | Mirabel, QC | AFP-capable composite shell and panel cell at Bombardier-scale facility | 1 | 19,800,000 | 19,800,000 |
| AFP machine cell | Mirabel, QC | Automated fiber placement for BMI wing skins | 1 | 10,300,000 | 10,300,000 |
| OOA cure ovens | Mirabel, QC | Out-of-autoclave cure capacity, two ovens | 2 | 3,400,000 | 6,800,000 |
| 5-axis machining cell | Mississauga, ON | Titanium and aluminium fitting machining, two centers | 2 | 4,700,000 | 9,400,000 |
| Ti additive manufacturing cell | Brampton, ON | Qualified recycled-titanium AM cell with lot passports | 1 | 6,200,000 | 6,200,000 |
| Final assembly hangar fit-out | Mirabel, QC | Determinant-assembly stations, two fuselage and two wing-root splice planes | 1 | 13,400,000 | 13,400,000 |
| Paint and finish booth | Mirabel, QC | Low-contrast finish and conductive coating application | 1 | 4,100,000 | 4,100,000 |
| Engine test cell time | Contract | Pooled M88 test-cell hours via Safran support network | 1 | 6,900,000 | 6,900,000 |
| RCS/pole range campaign | Contract | Coupon, panel, inlet and full-scale pole measurement time | 1 | 5,100,000 | 5,100,000 |
| Battery store and charge facility | Main operating base | LiFePO4 storage, conditioning and safety facility | 1 | 1,300,000 | 1,300,000 |
| Forward location shelter kits | 4 forward locations | Deployable shelters and environmental kits | 4 | 1,020,000 | 4,080,000 |
| Recycled-materials processing agreement | National | Closed-loop scrap return and certified remelt, 10-year term | 1 | 2,600,000 | 2,600,000 |
| Facilities total | - |  | - | - | 89,980,000 |

Facilities and infrastructure provisioning.

The recycled-materials processing agreement formalises the closed-loop streams of Section 6.2 of the baseline: segregated scrap return from Mirabel and supplier sites to EcoTitanium (titanium) and the Novelis-route rolling chain at Kingston (aluminium), with certified remelt and lot traceability back into Category B feedstock lots. This agreement is the enabling contract for the 20% rate-production recycled-content target and the material passport of Section 6.3. Smaller fleet options defer the second OOA oven and one machining centre; the Part IX infrastructure model reflects this.

# Part VII - Supplier Directory and Outreach Pathways

Every supplier code used in Parts II-VI resolves here. Pathways describe the practical route to a quotation or teaming discussion: public domain, telephone or named contact where published, procurement channel, and any government framework that applies. Canadian suppliers are engaged directly; allied and weapons items route through the government channels of Part VIII. The directory is ordered so that Canadian anchors appear first, allied non-US suppliers second; no ITAR-controlled US article supplier appears. No supplier named here has been contacted under this package; this is a planning directory, not a commitment list.

| Code | Supplier and location | Public domain | Role in programme |
| --- | --- | --- | --- |
| AHN | Aerotech Herman Nelson, Winnipeg, MB (Canada) | hermannelson.com | Portable aircraft heaters and Arctic ground support equipment |
| AVC | Avcorp Industries, Delta, BC (Canada) | avcorp.com | Composite structures and bonded assemblies, secondary structure, small assembly lines |
| BOM | Bombardier Defense, Mirabel, QC (Canada) | bombardier.com/en/defense | Large composite shells, final-assembly-scale facilities, defence special-mission integration heritage |
| CAE | CAE, Montreal, QC / Ottawa, ON (Canada) | cae.com | Training systems, full-mission simulators, constructive simulation, synthetic environments |
| CAS | Cascade Aerospace, Abbotsford, BC (Canada) | cascadeaerospace.com | Field-level MRO, fleet support, component repair |
| CCC | Canadian Commercial Corporation, Ottawa, ON (Canada) | ccc.ca | Government-to-government contracting route; Canadian content advocacy |
| CMC | CMC Electronics, Montreal, QC (Canada) | cmcelectronics.ca | Military GNSS anti-jam antennas, avionics integration, mission-computer heritage |
| CWD | Curtiss-Wright Defense Solutions, Ottawa, ON (Canada) | curtisswrightds.com | Rugged mission computing, Jetson-class module ruggedisation, data recorders, network switches |
| CYC | Cyclone Manufacturing, Mississauga, ON (Canada) | cyclonemfg.com | Precision machining of titanium and aluminium fittings, 5-axis work |
| GDC | General Dynamics Mission Systems-Canada, Ottawa, ON (Canada) | gd-ms.ca | Secure communications, cryptographic integration, cross-domain guards, ground stations |
| HER | Heroux-Devtek, Longueuil, QC / Cambridge, ON (Canada) | herouxdevtek.com | Landing gear systems, actuation, gear machining, aftermarket support |
| IMP | IMP Aerospace and Defence, Enfield, NS (Canada) | impaerospace.com | Depot-level maintenance, repair and overhaul; structural and avionics depot lines |
| MAG | Magellan Aerospace, Winnipeg, MB / Mississauga, ON (Canada) | magellan.aero | Composite and metallic aerostructures, machined assemblies, engine-adjacent structure |
| MDA | MDA Space / 49North, Brampton, ON / Sainte-Anne-de-Bellevue, QC (Canada) | mda.space | Conformal antenna structures, secure multi-domain C4ISR, ESM integration, timing and RF assemblies |
| NOV | Novelis (Canadian operations), Kingston, ON (Canada) | novelis.com | Closed-loop aluminium; recycled aerospace plate feedstock with rolling partners |
| NOVT | NovAtel / Hexagon, Calgary, AB (Canada) | novatel.com | GNSS receivers, anti-jam controlled-reception antennas, PNT boards |
| SHI | Shimco, Markham, ON (Canada) | shimco.com | Precision shims, washers, spacers, small stamped and machined parts |
| STA | StandardAero, Winnipeg, MB / Richmond, BC (Canada) | standardaero.com | Engine and component MRO, military engine support lines, accessory overhaul |
| WES | L3Harris WESCAM, Burlington, ON (Canada) | l3harris.com | MX-series EO/IR sensor heritage; flush EO/IR snapshot module development and manufacture, Canadian-manufactured |
| AKZ | AkzoNobel Aerospace Coatings, Sassenheim (Netherlands) | aerospace.akzonobel.com | Chromate-free primers and low-contrast polyurethane finish systems |
| CLA | Clarendon Specialty Fasteners, Alcester (United Kingdom) | clarendonsf.com | AS9120/EN9120 aerospace fastener distribution, kitting by tail number |
| CON | Constellium, Paris / Issoire (France) | constellium.com | Al-Li 2099/2195 plate and sheet; closed-loop aluminium recycling partner |
| DHL | Diehl Defence, Ueberlingen (Germany) | diehl.com | IRIS-T short-range air-to-air missile; Gripen E integration heritage |
| ECO | EcoTitanium, Saint-Georges-de-Mons (France) | ecotitanium.com | Closed-loop recycled titanium ingot from aviation scrap, up to 75% recycled content |
| EJT | EUROJET Turbo, Munich (Germany / EU consortium) | eurojet.de | EJ200 fallback engine class, 60/90 kN, consortium of Rolls-Royce, MTU, ITP and Avio |
| LIS | LISI Aerospace, Paris (France) | lisiaerospace.com | HI-LITE-class structural fasteners, lockbolts, blind fasteners, nuts and collars |
| MBD | MBDA, Paris / Stevenage / Rome (France / EU) | mbda-systems.com | SmartGlider Light compact glide weapon for KIN-1 module |
| MNK | Mankiewicz Coatings, Hamburg (Germany) | mankiewicz.com | Aerospace coatings, conductive and low-observable finish systems, sealants |
| NVD | NVIDIA (via ruggedisation integrator), Santa Clara, CA (United States) | nvidia.com | Jetson AGX Orin 64GB production modules; public distribution price anchor US$3,199; EAR99-class commercial module - sole residual US-origin line |
| SAA | Saab, Linkoping (Sweden) | saab.com | Gripen E teaming interface control, cooperative track manager integration support |
| SAE | Safran Aircraft Engines, Paris (France) | safran-group.com | M88-2 engine, FADEC, accessory gearbox, starter-generator; Safran Canada footprint anchors in-country support |
| SED | Safran Electronics and Defense, Paris (France) | safran-group.com | FOG inertial units, precision timing, fuel and inerting components, filtration |
| SFT | Saft, Bagnolet (France) | saft.com | Aviation LiFePO4 battery systems with AOG support partners |
| SYQ | Syensqo, Brussels (Belgium) | syensqo.com | BMI and epoxy carbon prepreg, adhesive films (former Solvay aerospace materials) |
| SYS | SYSGO, Klein-Winternheim (Germany) | sysgo.com | PikeOS safety-critical partitioning operating system |
| TOR | Toray Advanced Composites (Japan / global) | toraytac.com | BMI and epoxy carbon prepreg for primary structure |

Supplier directory. Codes are used throughout Parts II-VI. Canadian suppliers: 19 anchors. Allied non-US suppliers: 16. Residual US-origin line: one (NVD), EAR99-class, mitigated in Part XI.

## 7.1 Outreach pathways

Contact and engagement notes per supplier, in directory order:

- AHN - Aerotech Herman Nelson: 100 Eagle Drive, Winnipeg, MB, 1 800 486-4328 or +1 204 633-1999. Existing Canadian Armed Forces supplier.

- AVC - Avcorp Industries: Supplier quotation route via avcorp.com sales contact. AS9100-registered; accepts AS9102 first-article flow-down.

- BOM - Bombardier Defense, Mirabel: Official contact page at bombardier.com/en/defense/contact-us. Programme engagement letter through the programme office with ITB credits registered against the national content plan.

- CAE - CAE: CAE Defence and Security; CAE Canada, 350 Legget Drive, Suite 200, Ottawa, ON, +1 613 247-0342. Simulator work packages align with the Gripen-teamed syllabus of baseline Section 12.

- CAS - Cascade Aerospace: Fleet support services via cascadeaerospace.com; alternate depot and western-region field support node.

- CCC - Canadian Commercial Corporation: 350 Albert Street, Suite 1100, Ottawa, ON, +1 613 996-0034. G2G vehicle for allied-government purchases of the Sprinter system.

- CMC - CMC Electronics: Sales and engineering liaison via cmcelectronics.ca; existing DND supplier standing offers shorten onboarding.

- CWD - Curtiss-Wright Defense Solutions: Ottawa facility, 333 Palladium Drive, Ottawa, ON K2V 1A6, +1 613 599-9199. Ruggedises NVIDIA Jetson AGX Orin-class modules for the mission computer of Category H.

- CYC - Cyclone Manufacturing: Quotation route via cyclonemfg.com with AS9102 FAI flow-down; candidate for recycled-titanium fitting machining.

- GDC - General Dynamics Mission Systems-Canada: Defence programmes intake via gd-ms.ca. COMSEC-controlled items route through the Canadian Cryptographic Modernization Program account; no commercial substitution.

- HER - Heroux-Devtek: Defence sales via herouxdevtek.com; head office 1111 Saint-Charles Street West, Suite 600, Longueuil, QC, +1 450 679-5450. Canadian anchor for Category D with Gripen E landing-gear design authority; spares through its aftermarket division.

- IMP - IMP Aerospace and Defence: MRO services quotation via impaerospace.com; Enfield, NS, +1 902 873-2250. Depot activation at programme month M40.

- MAG - Magellan Aerospace: Business development and quotation route through magellan.aero corporate contact. Defence work packages flow under the ITB framework via PSPC and the Defence Investment Agency.

- MDA - MDA Space / 49North: MDA Brampton, 7500 Financial Drive, Brampton, ON, +1 905 790-2800. Classified-adjacent work under the Contract Security Program.

- NOV - Novelis (Canadian operations): Industrial account route via novelis.com; closed-loop agreements reference the Airbus-Constellium-Novelis recycling model cited in the baseline (Section 6.2).

- NOVT - NovAtel / Hexagon: Hexagon Calgary Campus, 10921 14th Street NE, Calgary, AB, +1 403 295-4500; Canada/US toll-free 1-800-668-2835.

- SHI - Shimco: Catalogue and custom quotation via shimco.com; aerospace quality registration for flight hardware.

- STA - StandardAero: Winnipeg +1 204 318 7544. Candidate for M88 sustainment partnership under Safran licence, leveraging Safran’s established Canadian support footprint; no current M88 capability is claimed pending OEM agreement.

- WES - L3Harris WESCAM: WESCAM sales, 649 North Service Road West, Burlington, ON, +1 905 633 4000. Canadian manufacture; government-to-government route available through the Canadian Commercial Corporation.

- AKZ - AkzoNobel Aerospace Coatings: Aerospace coatings sales via aerospace.akzonobel.com; EU export, no US content in finish-system lines.

- CLA - Clarendon Specialty Fasteners: Quotation route via clarendonsf.com contact page; UK operation supports the per-aircraft fastener packs of Category B.

- CON - Constellium: Aerospace sales via constellium.com; recycled-content lots coordinated with the NOV closed-loop stream.

- DHL - Diehl Defence: IRIS-T export sales via diehl.com; government-to-government arrangement through DND/PSPC with German export authorisation; IRIS-T already integrated on Gripen E.

- ECO - EcoTitanium: Supply agreement route via ecotitanium.com; framework references the Airbus closed-loop programme cited in the baseline (Section 6.2).

- EJT - EUROJET Turbo: Consortium programme office via eurojet.de; EJ200 fallback engine, 60/90 kN and about 1,000 kg per public consortium material; four-nation (UK/Germany/Italy/Spain) export arrangement.

- LIS - LISI Aerospace: OEM specials via lisiaerospace.com; French manufacture with full lot traceability.

- MBD - MBDA: MBDA export sales via mbda-systems.com; government-to-government arrangement through DND and PSPC with ITB offsets.

- MNK - Mankiewicz: Aerospace division via mankiewicz.com; qualification support for conductive and low-observable finish systems.

- NVD - NVIDIA (via ruggedisation integrator): Production modules through the NVIDIA embedded channel and distribution; ruggedisation by CWD Ottawa. Export classification EAR99-class module; end-use statement with order. Mitigation and second-source study per Part XI.

- SAA - Saab: Saab Canada office (Ottawa) as entry point; technical interface control through Gripen programme channels per Section 12 of the baseline.

- SAE - Safran Aircraft Engines: Safran defence sales via safran-group.com; M88 procurement routes as Direct Commercial Sale with French government export authorisation - the programme’s controlling export-control action (Part XI). Safran’s Canadian footprint (landing systems, electrical systems, engine support) anchors the in-country support model.

- SED - Safran Electronics and Defense: Avionics and optronics sales via safran-group.com; FOG IMU, timing and fuel-system quotations against Categories E, G and H.

- SFT - Saft: Aviation battery inquiries via saft.com; AOG and service partners include Boeing Distribution, Proponent and Satair. Chemistry and qualification alignment to the LiFePO4 baseline confirmed at RFQ.

- SYQ - Syensqo: Aerospace materials sales via syensqo.com; BMI prepreg qualification aligned to the wing-skin tooling schedule.

- SYS - SYSGO: PikeOS licensing via sysgo.com sales; EU export classification with Canadian end-use statement; DO-178C DAL-A evidence package partnership with CMC.

- TOR - Toray Advanced Composites: Official contact form path published on toraytac.com; Japanese-source BMI prepreg for primary structure.

## 7.2 Standard engagement sequence

For each register line the programme office executes the same five-step sequence: (1) issue request for information through the supplier portal or published contact; (2) execute a non-disclosure agreement and exchange the controlled baseline extract; (3) issue request for quotation against the register line with AS9102 first-article and AS9100 flow-downs; (4) qualify the source (site audit, special-process NADCAP check where applicable, Controlled Goods Program registration check); (5) place the line on contract under the acquisition strategy of Part VIII. Canadian lines additionally register ITB credit; export-controlled lines additionally open the licence track of Part XI before step 3. Government routes: PSPC and the Defence Investment Agency for Canadian contracting, the Canadian Commercial Corporation for government-to-government export of the system, and the French, German, Swedish, Belgian, Dutch, British and Japanese embassy defence sections for allied-government authorisations.

# Part VIII - Acquisition Strategy and Procurement Routes

## 8.1 Route by line type

| Line type | Route | Contract form |
| --- | --- | --- |
| Canadian-sourced structure, gear, GSE, training | PSPC/DIA competitive or directed source under ITB framework | Firm fixed price, production lots |
| Engine and engine accessories (M88-2) | Direct commercial sale with Safran, French government export authorisation; Canadian government endorsement | FFP with options; pooling agreement as annex |
| Fallback engine (EJ200) | EUROJET consortium, four-nation export arrangement | FFP; exercised only on fallback trigger |
| German weapons (IRIS-T) | Government-to-government via DND/PSPC; Diehl export sales with German export authorisation | G2G arrangement with ITB offsets |
| European weapons (SmartGlider Light) | Government-to-government via DND/PSPC; MBDA export sales | G2G arrangement with ITB offsets |
| COMSEC-controlled items (crypto, fill interface) | Canadian Cryptographic Modernization Program account; no commercial substitution | Government account transfer |
| Standard hardware and consumables (Category B, connectors, seals) | AS9120-registered distributors (Clarendon UK, LISI France), kitting by tail number | Blanket purchase agreements |
| Recycled feedstock streams | Direct commercial agreements with EcoTitanium and the Novelis-route chain; closed-loop scrap return annex | Long-term agreement, 10-year |
| Software licences (RTOS, FCS loads, mission software) | Direct licence (SYSGO PikeOS, CMC, Curtiss-Wright Ottawa) with export classification review | Licence agreement, per-tail |
| Allied-government export of the Sprinter system | Canadian Commercial Corporation government-to-government vehicle | G2G arrangement |

## 8.2 Industrial and Technological Benefits

The package is structured so that Canadian content leads wherever capability exists: Bombardier Mirabel, Avcorp and Magellan (structure), Heroux-Devtek (gear and actuation), Cyclone and Shimco (machining and small parts), WESCAM (EO/IR), MDA/49North (ESM and RF), CMC (avionics integration), General Dynamics Mission Systems-Canada (comms and crypto), Curtiss-Wright Ottawa (rugged computing), NovAtel/Hexagon Calgary (PNT), CAE (training), IMP and Cascade (MRO), StandardAero (engine sustainment candidate), Aerotech Herman Nelson (GSE) and the Novelis-route feedstock chain in Kingston. Estimated Canadian content is 48% of recurring flyaway value at first article, rising toward 56% at rate production. Part XII carries the full workshare plan; ITB credits are registered line-by-line at contract placement.

## 8.3 Competition posture

One genuine competition remains open at CDR: the depot MRO lead (IMP Enfield vs Cascade Abbotsford, decided by M30). The engine line is a directed OEM procurement to Safran pending the Section 13.2 validation items, with the EJ200 fallback carried as a priced risk, not a competition. All other lines are directed to the Canadian anchor, routed through government channels, or placed with the qualified distributor channel.

# Part IX - Cost Analysis and Fleet Economics (CAD)

## 9.1 Currency basis

All values in this package are 2026 Canadian dollars. Where a public anchor is in US dollars (the Jetson module distribution price), the controlled planning conversion of 1.35 CAD/USD applies. This is a controlled planning conversion, not a market exchange-rate quote; procurement-time conversion uses the contract rate. The v6.1 supply chain is deliberately structured so that no material line depends on a US-dollar-denominated controlled article.

## 9.2 Per-unit cost and flyaway reconciliation

The per-unit recurring cost of one Sprinter is C$11.85M at the 72-aircraft production rate (C$14.25M ceiling). Line-level recurring costs from Part II reconcile to the build-up below. The register-sum column is computed directly from the Part II registers; residuals against the controlled lines are installation labour, integration and acceptance shares, and are stated openly rather than forced to zero.

| Build-up line | Value (C$M) | Register mapping |
| --- | --- | --- |
| Airframe structure and assembly | 2.05 | A + B + L + bay racks/liners of K + assembly labour (1,250 h at unit 72) |
| M88-2 engine and installation | 3.50 | C; residual is installation labour at cassette mate |
| Avionics and flight controls | 1.38 | E (minus actuation share) + H + J |
| Baseline ESM and EO/IR | 0.98 | I |
| Landing gear and actuators | 0.77 | D + EHA/actuation share of E |
| Power, thermal, and fuel systems | 0.65 | F + G |
| Integration and acceptance test | 1.14 | K remainder + system integration, acceptance flight test, first-article qualification support |
| Overhead and profit | 1.05 | 15%-class programme allowance |
| Installation, integration and acceptance residual | 0.33 | CALCULATED closing value: harness and module installation labour, module mate, acceptance flight test and first-article qualification support not carried in register unit prices |
| Recurring flyaway total | 11.85 | Register sums (C$8.98M) plus the two allowance lines plus the stated residual close to the controlled target |

Flyaway reconciliation, 2026 CAD, 72-aircraft rate. The ceiling of C$14.25M absorbs the long-lead and engine-price risks of Parts XIII and XIV; a cost breach above the ceiling triggers payload or coating scope reduction before speed or fuel is traded away, consistent with the baseline’s closure-first design rule.

The recurring unit cost follows a 92% unit learning curve against quantity: smaller fleets pay more per aircraft because tooling, qualification and first-article costs amortise over fewer tails. The curve is anchored at C$11.85M for unit 72; the formula applied is U(Q) = 11.85 x (Q/72)^b with b = ln(0.92)/ln(2).

## 9.3 Fleet-size economics - seventeen options

The table below prices all seventeen candidate fleet sizes on identical rules: the learning-curve unit cost of Section 9.2; non-recurring engineering of C$1,320M held constant across options (the aircraft must be developed once regardless of fleet size); and initial spares/support modelled as C$27.0M fixed base plus C$1.36M per aircraft, which reproduces the reference-fleet composition of Part X (spares pool C$61.2M + GSE C$17.9M + training C$45.7M = C$124.8M at 72 aircraft) within 0.1%. All figures are 2026 CAD planning values.

| Fleet size (aircraft) | Estimated recurring unit cost (C$M) | Total aircraft procurement cost (C$M) | Allocated non-recurring cost per aircraft (C$M) | Initial spares and support (C$M) | Total program acquisition cost (C$M) |
| --- | --- | --- | --- | --- | --- |
| 10 | 15.03 | 150.3 | 132.00 | 40.6 | 1,510.9 |
| 15 | 14.31 | 214.7 | 88.00 | 47.4 | 1,582.1 |
| 20 | 13.82 | 276.5 | 66.00 | 54.2 | 1,650.7 |
| 25 | 13.46 | 336.5 | 52.80 | 61.0 | 1,717.5 |
| 28 | 13.28 | 371.7 | 47.14 | 65.1 | 1,756.8 |
| 30 | 13.17 | 395.0 | 44.00 | 67.8 | 1,782.8 |
| 35 | 12.92 | 452.3 | 37.71 | 74.6 | 1,846.9 |
| 38 | 12.80 | 486.3 | 34.74 | 78.7 | 1,885.0 |
| 40 | 12.72 | 508.7 | 33.00 | 81.4 | 1,910.1 |
| 45 | 12.54 | 564.3 | 29.33 | 88.2 | 1,972.5 |
| 50 | 12.38 | 619.1 | 26.40 | 95.0 | 2,034.1 |
| 55 | 12.24 | 673.2 | 24.00 | 101.8 | 2,095.0 |
| 58 | 12.16 | 705.4 | 22.76 | 105.9 | 2,131.3 |
| 60 | 12.11 | 726.8 | 22.00 | 108.6 | 2,155.4 |
| 65 | 12.00 | 779.8 | 20.31 | 115.4 | 2,215.2 |
| 70 | 11.89 | 832.3 | 18.86 | 122.2 | 2,274.5 |
| 72 | 11.85 | 853.2 | 18.33 | 124.9 | 2,298.1 |

Fleet-size economics, 2026 CAD planning values. Total program acquisition cost = aircraft procurement + non-recurring engineering + initial spares/support; weapons, infrastructure, contingency and lifecycle are added in the Part XVI decision summary.

The economics are unambiguous: non-recurring cost dominates small fleets (C$132.0M per aircraft at a 10-aircraft buy) and dilutes steadily to C$18.3M per aircraft at 72. Each step up in quantity buys down both the unit recurring cost and the NRE allocation; the curve flattens beyond about 40 aircraft, which is the practical minimum for an economically defensible programme.

## 9.4 Programme rollup (reference 72-aircraft fleet)

| Programme line | CAD | Source |
| --- | --- | --- |
| Recurring production (72 x C$11.85M) | 853.2 M | Part II rollup, learning-curve anchor |
| Non-recurring engineering (planning allowance) | 1,320.0 M | ESTIMATE - outside the controlled baseline |
| Initial spares pool | 61.2 M | Part II section 2.13 |
| Training system | 45.7 M | Part V |
| Ground support equipment | 17.9 M | Part IV |
| Programme total | 2,298.1 M | Planning total, weapons and facilities excluded |
| Weapons initial provisioning (separate account) | 50.8 M | Part III |
| Facilities (programme capital) | 90.0 M | Part VI |
| Contingency (10% of acquisition) | 243.9 M | Part XVI rule |
| Total acquisition, reference fleet | 2,682.7 M | Part XVI decision summary |

Programme rollup reconciliation, 2026 CAD.

## 9.5 Operating cost anchors

- Engine pooling: the M88 remains a recoverable programme asset with life well beyond the 1,000 h airframe; the pooled 138-engine inventory with the Striker fleet materially reduces the cost of losing an airframe (baseline Section 4.1). Safran’s public record of more than one million Rafale fleet flight hours and on-condition modular maintenance anchors the sustainment model of Part X.

- Material recovery credit: US$40k-80k-equivalent per aircraft at production maturity, excluding engine pooling; the 72-aircraft run recovers about 28.9 tonnes of recycled-content material (402 kg per aircraft).

- Spares consumption: the Part II pool is sized for the first depot cycle; steady-state re-stock runs at roughly 11% of pool value per year (ESTIMATE).

- Maintenance burden: 35-minute turnaround, 20-minute module swap, 90-minute engine cassette removal and 15-minute post-flight download bound the line-maintenance manpower of Part V and Part X.

# Part X - Sustainment Package

This part translates the v6.1 reliability and maintainability requirements - 0.97 mission reliability, 1,000 h airframe life, 250 dash events, 35-minute turnaround, 20-minute module swap, 90-minute engine cassette removal, 15-minute post-flight download, -40 C Arctic start (baseline Sections 11 and 11.1) - into provisioned resources and money. The reference quantities below support the 72-aircraft fleet; the Part IX support model (C$27.0M fixed + C$1.36M per aircraft initial, then C$1.35M per aircraft per year recurring plus C$22.0M per year fixed) scales them to any fleet size in Part XVI.

## 10.1 Spares provisioning

| Spares element | Basis | Provision (reference fleet) | Cost (C$M) |
| --- | --- | --- | --- |
| Airframe and LRU spares pool | Part II register spares columns, first depot cycle | Categories A-L as tabled | 26.3 |
| Engine spares (pooled) | 15% float on installed engines; Sprinter share 11 engines of the 138-engine pool | 11 x M88-2 plus engine sensor/mount pools | 34.9 |
| Mission module spares | Two module sets per three aircraft (baseline Section 12.1) | 48 shipsets across ESM-1, EW-1, DEC-1, ISR-1, AAM-1, REL-1 | Included above |
| Recycled feedstock qualified lots | Lot-qualified, not generic scrap (baseline Section 12.1) | First-year re-stock lots, all five streams | 1.9 |
| Initial spares pool total |  |  | 61.2 |

## 10.2 Depot equipment and repair pipeline

| Element | Description | Quantity / value | Anchor |
| --- | --- | --- | --- |
| Depot line activation | Structural, avionics and mission-module depot lines | C$14.5M, activation M40 | IMP Enfield or Cascade Abbotsford (competition, Part VIII) |
| Engine depot and test | M88 modular maintenance under Safran licence; test-cell hours pooled with Striker | C$6.9M test-cell block (Part VI) plus C$4.2M/yr support | Safran / StandardAero Winnipeg candidate |
| Sensor test benches | ESM/EO/IR depot benches | 2 x C$410k (Part IV) | MDA / WESCAM |
| Repair pipeline | 90-minute cassette swap at line; engine modules flow to depot; 62% of OEW recoverable at end of life | Pipeline stock C$3.1M | Safran modular architecture |
| Composite repair | Field hot-bonder kits and depot autoclave access | 5 field kits (Part IV); depot via Magellan/Bombardier | MAG / AVC |

## 10.3 Engine maintenance model

The M88 is maintained on-condition under its public modular architecture: modules swap at line or depot without full engine strip, supporting the one-hour removal and reinstallation capability cited in the baseline. Engines are programme assets, not airframe assets: on airframe loss or life expiry, the engine returns to the pool. Planning values: scheduled module intervention every 400 engine hours (ESTIMATE), dash-event tracking against the 250-event allocation, health-monitoring data review after every flight via the 15-minute download, and fleet engine-usage balancing through the pool. Engine sustainment runs at approximately C$0.42M per installed engine per year (ESTIMATE, on-condition analogue class).

## 10.4 Line-replaceable units and software support

| Sustainment line | Scope | Annual cost, reference fleet (C$M) |
| --- | --- | --- |
| LRU re-stock and repair | EHA units, FCCs, SSPCs, batteries, sensor LRUs, bay modules | 14.8 |
| Engine maintenance | On-condition module pool, test cell, OEM support | 30.2 |
| Software support | FCS loads (DO-178C DAL-A), mission software, autonomy behaviour-node updates, cyber patching | 9.6 |
| Consumables | Sealants, coatings, tires, brakes, filters, safety wire, RAM strip replacement | 6.4 |
| Training device upkeep | Simulator concurrency, courseware updates, AR aid refresh | 5.8 |
| Depot fixed costs | Depot line, quality and standards, material passports, closed-loop return | 22.0 |
| Support manpower | See Section 10.6 | 30.5 |
| Steady-state sustainment | Reference 72-aircraft fleet | 119.3 / yr |

## 10.5 Consumables and repairables

Consumables provision the 250-dash-event life: RAM leading-edge strips are replaceable sacrificial items (Category L); brake heat-packs are sized against the 12 MJ RTO case; tires are winter-friction rated with a 25-minute change task; PAO coolant and battery modules are conditioned stocks. Repairables loop through the depot pipeline with a target 45-day turnaround time at line level and 120 days at depot; condemned articles feed the closed-loop recycled-material streams of Section 6.2 of the baseline, with mass recovery logged against the material passport.

## 10.6 Expected support manpower

| Cadre | Reference fleet | Scaling rule | Note |
| --- | --- | --- | --- |
| Line maintainers | 70 | 14 per squadron of 12 | Turnaround, module swap, refuel, download |
| Depot artisans | 24 | Fixed for fleets of 25+; 12 below | IMP/Cascade depot line |
| Engine shop technicians | 16 | 1 per 4-5 pooled engines | M88 module work under Safran licence |
| Software/mission support | 12 | Fixed | CMC/CWD/MDA in-service support contracts |
| COMSEC custodians | 10 | 2 per squadron | Crypto account control |
| Supply and provisioning | 9 | Fixed | Catalogue, codification, kitting oversight |
| Total support manpower | 141 |  | Excludes operational cadre of Part V |

# Part XI - ITAR and Export-Control Strategy

The v6.1 baseline deliberately excludes export-controlled US articles from the aircraft supply chain (baseline Sections 1.2 and 4); this part converts that design decision into an executable sourcing and compliance strategy.

## 11.1 Sourcing architecture

| Exposure class | v6.1 position | Mechanism |
| --- | --- | --- |
| Propulsion | French Safran M88-2; zero US content claim path | French export authorisation (Part 11.2); EJ200 four-nation fallback |
| Weapons | German IRIS-T; European SmartGlider Light | G2G via DND/PSPC; German and EU export authorisations |
| Avionics and computing | Canadian integration; French IMU/timing; German RTOS | Commercial export classification; one EAR99 residual (11.4) |
| Materials and feedstock | Canadian, French, Belgian, Japanese | Commercial contracts; closed-loop agreements |
| COMSEC | Canadian national cryptographic account | Cryptographic Modernization Program; no commercial substitution |

## 11.2 The controlling export action: M88 French authorisation

The single controlling export-control action of the programme is French government authorisation for M88-2 supply to Canada and for the programme-modified accessory gearbox, inlet-control schedule, health-monitoring interface and nozzle-edge treatment (baseline Section 4). Status: PENDING OEM VALIDATION per the baseline pending-validation register. The action plan: (1) programme office issues the Safran RFI at M0-M6; (2) Canadian government endorsement letter via PSPC/DIA at M6; (3) French export-authorisation request lodged by M12; (4) authorisation and OEM quotation target M18, protecting the M40 first-flight gate; (5) if authorisation stalls beyond M18, the EJ200 fallback triggers with fuselage length and MTOW recheck before contract commitment (baseline Section 13.2).

## 11.3 Canadian export-control compliance

Canadian-side items fall under the Controlled Goods Program and the Contract Security Program; organizations examining, possessing or transferring controlled goods in Canada require CGP registration with a Designated Official, personnel security assessments and breach reporting. Military goods and technology - military aircraft, targeting and guidance systems, military communications, EW equipment and related technical data - are controlled under the Export and Import Permits Act and Export Control List, with Global Affairs Canada permits applying regardless of destination. Export of the complete Sprinter system to allied governments routes through the Canadian Commercial Corporation as a government-to-government sale, preserving Canadian control of the configuration and of the IRIS-T/SmartGlider third-party transfer conditions.

## 11.4 Residual US-origin exposure and mitigation

One register line retains US origin: the Jetson AGX Orin 64GB computing module (Category H, SPRN6-H-001). It is a commercial EAR99-class module, not ITAR-controlled; procurement risk is therefore supply and policy, not licence denial. Mitigations: (1) Canadian ruggedisation, chassis, qualification and support by Curtiss-Wright Ottawa, with module buffer stock of two production lots; (2) a funded second-source study (M12-M30) qualifying a European or allied AI compute module against the 40 TOPS mission-computer requirement; (3) software portability requirement in the mission-computer contract so the autonomy stack re-hosts without redesign. No other register line carries US content of consequence; earlier baselines’ US dependencies (engine, weapons, RTOS, atomic clock, coatings, fuel components) are all re-sourced in v6.1.

## 11.5 ITAR-free verification rule

Every RFQ and contract carries an ITAR-free flow-down: the supplier certifies that no ITAR-controlled US article, technical data or defence service enters its deliverable, and the programme office audits the certification at source qualification and annually thereafter. A line that cannot make the certification is re-sourced or descoped before award - the supply-chain veto risk that eliminated the US engine option at design level is not re-admitted at contract level.

# Part XII - Industrial and Canadian Workshare Plan

## 12.1 Prime and integrator structure

| Role | Candidate | Scope |
| --- | --- | --- |
| Prime integrator and final assembly | Bombardier Defense, Mirabel, QC | Final mate at two fuselage splices and two wing-root splice planes; acceptance flight test; programme delivery |
| Airframe major modules | Magellan Aerospace (Winnipeg/Mississauga) and Avcorp (Delta) | Nose, forward fuselage, centerbody with Bombardier; wings; fins; DSI assemblies |
| Landing gear and actuation | Heroux-Devtek (Longueuil/Cambridge) | Full Category D plus EHA actuation of Category E |
| Mission systems integration | MDA Space / 49North, CMC Electronics, Curtiss-Wright Ottawa | ESM, computing, navigation, mission network |
| EO/IR | L3Harris WESCAM, Burlington | Flush snapshot sensor and window |
| Secure communications | General Dynamics Mission Systems-Canada, Ottawa | Datalinks, crypto integration, ground stations |
| Training system | CAE, Montreal/Ottawa | Part V full category |
| Sustainment | IMP or Cascade (depot); StandardAero (engine, under Safran licence) | Part X full scope |

## 12.2 Canadian assembly, component manufacture and supply-chain participation

Final assembly, composite manufacturing, machining, gear, actuation, mission computing, EO/IR, ESM, training and depot support are all Canadian-located. The Mirabel determinant-assembly flow (baseline Section 6.4) eliminates large monolithic jigs and lets Canadian module suppliers ship complete, tested sections. Domestic-content objective: 48% of recurring flyaway value Canadian at first article, 56% at rate production, with the growth coming from recycled-feedstock localisation (Kingston aluminium, Canadian machining of EcoTitanium ingot), engine-support indigenisation under Safran licence, and the compute-module second-source study.

## 12.3 Technology-transfer assumptions

| Transfer | From | To | Gate |
| --- | --- | --- | --- |
| M88 maintenance and module repair | Safran | StandardAero Winnipeg | French export authorisation plus licence agreement |
| Flush EO/IR build | WESCAM product line | Burlington, ON manufacturing | Development contract M18 |
| Conformal ESM array manufacture | MDA heritage | Brampton/Montreal lines | CDR-100 drawings |
| Recycled Ti AM qualification | EcoTitanium feedstock | Cyclone/Magellan machining and AM cells | Lot passport closure (baseline 6.3) |
| Gripen teaming interface | Saab | CAE/MDA mission-planning and link integration | Interface control document |

## 12.4 Industrial benefits registration

All eligible major contracts register ITB obligations equal to contract value, with credits weighted toward Canadian supplier content, R&D in Canada, IP retention and facility investment (Mirabel cells, Brampton AM cell, Mississauga machining). The Defence Drone Initiative provides the sovereign-capability engagement channel; the Build-Partner-Buy posture of the Defence Investment Agency favours this Canadian-build structure over an offshore purchase. Estimated industrial-benefit value at the reference fleet: approximately C$1.28B of Canadian business activity across production and first-depot-cycle sustainment (ESTIMATE, 48% of acquisition value excluding government accounts).

# Part XIII - Schedule and Long-Lead Plan

Programme milestones per the baseline timeline: CDR procurement release M0; first flight M40; flight-test block M40-M78; initial operational capability M84. Procurement actions are phased against these milestones:

| Month | Procurement action |
| --- | --- |
| M0-M6 | CDR release; RFI wave to all Part VII suppliers; NDA execution; CGP registration checks; French export-authorisation track opened for the M88 |
| M6-M12 | Long-lead placement: M88-2 engine (18 mo, PENDING OEM), FCS software evidence (18 mo), crypto account (14 mo), ESM receiver (14 mo), starter-generator (14 mo) |
| M12-M18 | Engine pooling agreement executed with the Striker fleet (120-aircraft inventory, 15% float); recycled-feedstock long-term agreements signed (EcoTitanium, Novelis route); French export authorisation target |
| M18-M24 | Category A/B production contracts; Mirabel cell activation begins; COMSEC account opens; IRIS-T and SmartGlider Light G2G arrangements submitted |
| M24-M30 | Category C-L production contracts; depot lead decision (IMP vs Cascade); training system contract (CAE) |
| M30-M40 | First-article fabrication and assembly; AS9102 FAI wave; qualification test articles delivered; reflectometry coupon acceptance |
| M40-M48 | First flight M40; LRIP-1 lot (6 aircraft) placed; depot line activation |
| M48-M72 | LRIP-2 (24) and LRIP-3 (42) lots; rate ramp toward 1.5 aircraft/month; weapons deliveries begin |
| M72-M84 | Fleet delivery completes; IOC declaration M84; spares pool at full depth |

## 13.1 Long-lead register

| Item | Lead | Order-by | Mitigation |
| --- | --- | --- | --- |
| M88-2 engine (Category C) | 18 mo | M12 | Pooled 120-aircraft inventory with 15% float; EJ200 fallback per baseline Section 13.2; French authorisation lodged M12 |
| FCS software certification evidence (Category E) | 18 mo | M6 | DO-178C DAL-A package started at CDR; CMC and SYSGO co-located team |
| COMSEC crypto account (Category J) | 14 mo | M12 | Government account opened at M18; no commercial substitution |
| ESM receiver/processor (Category I) | 14 mo | M12 | MDA Brampton line; 49North secure-programme channel |
| Starter-generator (Category F) | 14 mo | M12 | Safran gearbox review early; RAT fallback adds a Category F line if extraction does not close |
| Ku directional terminal (Category J) | 12 mo | M12 | MDA/GDC dual-source strategy |
| EO/IR snapshot sensor (Category I) | 12 mo | M12 | WESCAM Burlington line; Canadian manufacture |
| Recycled feedstock qualification lots (Category B) | 15 mo | M12 | Secondary-first insertion per baseline Section 6.3; virgin fallback holds OML, mass and performance |

Long-lead items and mitigations. Order-by dates protect the M40 first-flight gate.

# Part XIV - Supply Chain and Procurement Risk

| Risk | Class | Exposure | Mitigation |
| --- | --- | --- | --- |
| R-P1 M88 supply, price or French export approval fails | Schedule/Cost | First-flight gate M40; largest single cost line | Pooled inventory float; EJ200 fallback repriced before contract commitment (baseline Section 13.2); authorisation lodged M12 |
| R-P2 Inlet-engine compatibility test slip | Schedule | Ground-engine runs before high-speed taxi | CFD and subscale tunnel work funded at M0; spill-door schedule adjustable without OML change |
| R-P3 Recycled-feedstock lot variability | Quality | Category B feedstock | EcoTitanium certified remelt with lot chemistry traceability; 15% machining yield margin at Cyclone; secondary-first insertion |
| R-P4 Allied export-authorisation delay (France, Germany, EU) | Schedule | Engine, IRIS-T, SmartGlider Light | G2G cases opened at M18; Canadian government endorsement; EJ200 and EW-only bay fallbacks |
| R-P5 Store separation envelope fails to close | Technical | KIN-1 and AAM-1 mission options | EW-only bay fallback per baseline Section 10.2; subsonic release envelope first; weapons account held until clearance |
| R-P6 Distributor stock-out on standard hardware | Supply | Category B/F consumables | Blanket purchase agreements with Clarendon UK and LISI France; 6-month buffer stock; kitting by tail number |
| R-P7 Single-source Canadian anchors | Industrial | Bombardier, Heroux-Devtek, WESCAM, CAE | Named second sources carried per line; tooling data packages owned by the Crown |
| R-P8 Residual US policy shift on compute module | Supply | Category H mission computer | EAR99 buffer stock; allied second-source study M12-M30; software portability requirement |
| R-P9 Price escalation beyond C$14.25M ceiling | Cost | Programme margin | Ceiling reserves about 20% over target; payload or coating scope reduces before speed or fuel |

Procurement risk register. Technical risks internal to the design remain in the baseline register; this register covers supply and procurement only.

# Part XV - Quality, Standards and Compliance

## 15.1 Quality flow-downs

- AS9100D: quality management system, all production suppliers.

- AS9102: first-article inspection, every register line at first delivery.

- AS9120 / EN9120: distributor quality for Category B/F standard hardware and connectors.

- AS5553: counterfeit prevention, all electronic and standard parts.

- NADCAP: special processes - composites, heat treat, welding, NDT, coatings - for all structural and engine-adjacent lines.

- ISO 17025: calibration and test laboratories supporting acceptance.

- MIL-STD-2073: military packaging for all spares and deployable stocks.

- DO-178C DAL-A / DO-254: flight-control software and complex electronic hardware evidence packages.

## 15.2 Export control and security

Canadian-side compliance per Part XI Section 11.3; allied export authorisations per Section 11.2 and 11.4; the ITAR-free verification rule of Section 11.5 applies to every register line. No export-controlled technical data is released to any supplier before the applicable licence or agreement is in force.

## 15.3 Canadian procurement framework

Defence procurement engages the Defence Investment Agency as contracting authority for large defence contracts, with PSPC, DND, ISED, Global Affairs Canada, the Canadian Commercial Corporation and Defence Construction Canada in their distinct roles. The ITB policy applies to eligible major procurements, with business activity in Canada equal to contract value and incentives for Canadian suppliers, R&D, IP and facility investment. The Defence Drone Initiative provides a sovereign-capability engagement channel for this class of system. The Build-Partner-Buy approach favours the Canadian build posture of this package.

## 15.4 Traceability and records

Full lot traceability applies from feedstock (including recycled remelt lots) through machining, assembly and installation. Every Category A structural part and every Category C rotating part carries a serialised record; recycled-content mass is logged per tail number against the 402 kg first-article and 20% rate targets of the baseline. The material passport of Section 6.3 accompanies every recycled-content article. Health and maintenance records close the loop into the 1,000 h life-management programme.

# Part XVI - Procurement Decision Summary

This closing table is the decision-maker’s set of numbers. Every row prices one fleet option on the identical rules of Part IX: 92% learning curve on the C$11.85M rate unit cost; C$1,320M non-recurring program cost held constant; initial support at C$27.0M fixed plus C$1.36M per aircraft; payload/weapons at C$0.706M per aircraft (IRIS-T and SmartGlider Light loadouts, Part III); infrastructure at C$57.5M fixed plus C$0.45M per aircraft (Part VI); contingency at 10% of the acquisition subtotal; and the lifecycle estimate adds ten years of sustainment at C$1.35M per aircraft per year plus C$22.0M per year fixed program support (Part X). All figures are 2026 CAD planning values in millions.

| Aircraft quantity | Recurring aircraft cost (C$M) | Non-recurring program cost (C$M) | Initial support cost (C$M) | Payload / weapons cost (C$M) | Infrastructure cost (C$M) | Contingency 10% (C$M) | Total acquisition cost (C$M) | Lifecycle estimate, 10 yr (C$M) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 150.3 | 1,320.0 | 40.6 | 7.1 | 62.0 | 158.0 | 1,737.9 | 2,092.9 |
| 15 | 214.7 | 1,320.0 | 47.4 | 10.6 | 64.2 | 165.7 | 1,822.6 | 2,245.1 |
| 20 | 276.5 | 1,320.0 | 54.2 | 14.1 | 66.5 | 173.1 | 1,904.4 | 2,394.4 |
| 25 | 336.5 | 1,320.0 | 61.0 | 17.6 | 68.8 | 180.4 | 1,984.2 | 2,541.7 |
| 28 | 371.7 | 1,320.0 | 65.1 | 19.8 | 70.1 | 184.7 | 2,031.3 | 2,629.3 |
| 30 | 395.0 | 1,320.0 | 67.8 | 21.2 | 71.0 | 187.5 | 2,062.5 | 2,687.5 |
| 35 | 452.3 | 1,320.0 | 74.6 | 24.7 | 73.2 | 194.5 | 2,139.4 | 2,831.9 |
| 38 | 486.3 | 1,320.0 | 78.7 | 26.8 | 74.6 | 198.6 | 2,185.0 | 2,918.0 |
| 40 | 508.7 | 1,320.0 | 81.4 | 28.2 | 75.5 | 201.4 | 2,215.3 | 2,975.3 |
| 45 | 564.3 | 1,320.0 | 88.2 | 31.8 | 77.8 | 208.2 | 2,290.2 | 3,117.7 |
| 50 | 619.1 | 1,320.0 | 95.0 | 35.3 | 80.0 | 214.9 | 2,364.3 | 3,259.3 |
| 55 | 673.2 | 1,320.0 | 101.8 | 38.8 | 82.2 | 221.6 | 2,437.7 | 3,400.2 |
| 58 | 705.4 | 1,320.0 | 105.9 | 40.9 | 83.6 | 225.6 | 2,481.4 | 3,484.4 |
| 60 | 726.8 | 1,320.0 | 108.6 | 42.4 | 84.5 | 228.2 | 2,510.4 | 3,540.4 |
| 65 | 779.8 | 1,320.0 | 115.4 | 45.9 | 86.8 | 234.8 | 2,582.6 | 3,680.1 |
| 70 | 832.3 | 1,320.0 | 122.2 | 49.4 | 89.0 | 241.3 | 2,654.2 | 3,819.2 |
| 72 | 853.2 | 1,320.0 | 124.9 | 50.8 | 89.9 | 243.9 | 2,682.7 | 3,874.7 |

Procurement decision summary, seventeen fleet options, 2026 CAD.

## How to read the decision space

| Decision point | Reading |
| --- | --- |
| Cheapest total program | 10 aircraft at C$1,741.7M acquisition - but C$174.2M all-in per aircraft |
| Best value inflection | 40 aircraft: unit cost C$12.72M, all-in C$55.5M per aircraft; NRE dilution flattens beyond this point |
| Reference programme | 72 aircraft: unit cost C$11.85M, all-in C$37.3M per aircraft, lifecycle C$3,877.8M over ten years |
| Marginal cost of growth | Each additional squadron of 12 beyond 40 adds about C$155M of recurring procurement while reducing unit cost by only 2-4% |
| Attrition margin | The 2-3% per-sortie planning loss range (baseline Section 12.1) argues for the attrition reserve embedded in fleets of 60+ |

Single-aircraft reference card: one Sprinter costs C$11.85M recurring at rate; carries one Safran M88-2, a 250 kg internal payload including one IRIS-T, 402 kg of certified recycled content; is supported by a 138-engine pool, a C$61.2M spares pool and a 141-person support cadre; and delivers Mach 1.60 for five minutes at 40-45,000 ft under Gripen E command with no ITAR-controlled article anywhere in its supply chain.

# Appendix A - Traceability to SPRINTER-v6.1-BL

| Package part | Baseline sections | Content traced |
| --- | --- | --- |
| Part I | Sections 1, 2.3, 2.4, 4, 5, 6.2, 10, 14 | Platform summary, mission set, programme quantities |
| Part II Cat. A/B | Sections 6.1, 6.2, 6.3, 6.4; recycled-content article table | Structure, materials, fastener model, recycled content |
| Part II Cat. C | Sections 4, 4.1, 4.2, 4.3, 13.2 | M88 engine, inlet, nozzle, pooling, pending OEM items |
| Part II Cat. D | Sections 5.2, 6.1, 11.1 | Gear, brakes, RTO energy (7.2 MJ vs 12 MJ), Arctic operation |
| Part II Cat. E | Sections 8, 8.1, 8.2 | Flight controls, actuation, autonomy policy, navigation |
| Part II Cat. F | Section 7.2 | Electrical architecture, batteries, load table (38.2 kW) |
| Part II Cat. G | Sections 7, 7.1 | Fuel system (990 kg usable), thermal sink budget (14.5 vs 12.6 MJ) |
| Part II Cat. H | Sections 8.2, 10 | Mission computer, recorder, timing, GNSS |
| Part II Cat. I | Sections 9, 10 | ESM arrays, EO/IR snapshot, aperture policy |
| Part II Cat. J | Section 8.2 | Datalinks, crypto, SATCOM exclusion |
| Part II Cat. K | Sections 2.4, 10.1, 10.2, 8.1 | 3.05 m bay, stations, IRIS-T dual-ejector fit, authorization chain |
| Part II Cat. L | Section 9 | Signature materials and claim boundary |
| Part III | Sections 10.2, 8.1 | IRIS-T and SmartGlider Light only, separation status, role separation |
| Parts IV/V/VI | Sections 2.4, 6.4, 11.1, 12.1 | GSE, training, facilities, closed-loop agreement |
| Part IX | Sections 6.4, 12.1; programme cost model | Unit cost, learning curve, fleet economics |
| Part X | Sections 4.1, 11, 11.1, 12.1 | Sustainment, engine pooling, turnaround targets, support manpower |
| Part XI | Sections 1.2, 4, 13.2 | ITAR-free sourcing, French export authorisation, EJ200 fallback |
| Part XII | Sections 6.2, 6.4 | Canadian workshare, recycled-feedstock localisation |
| Part XIII | Section 13; baseline timeline | Schedule, long-lead plan, pending validation register |

Traceability map. The parent specification (SPRINTER-v6.1-BL) remains the controlling document for all technical content.

# Appendix B - Referenced Standards

AS9100D, AS9102, AS9120/EN9120, AS5553 (counterfeit prevention), NADCAP special-process accreditation, ISO 9001, ISO 17025, MIL-STD-2073 (packaging), NATO-pattern 14-inch suspension, DO-178C (software), DO-254 (hardware), ARINC 653 (partitioning), Export and Import Permits Act, Export Control List, Controlled Goods Program, Contract Security Program, Industrial and Technological Benefits Policy, EAR (residual EAR99 line only). No ITAR-controlled standard or article applies to any register line. Copies are maintained by the programme office standards library; suppliers receive the applicable extracts with each RFQ.

# Appendix C - Price Basis and Verification Status

Every register line carries one of four price-status tags:

| Tag | Meaning | Count in this package |
| --- | --- | --- |
| VENDOR | A public distribution price or procurement record anchors the value | Mission computer line (Jetson public distribution anchor) |
| CALCULATED | Transparent arithmetic from controlled inputs | Washer, coupon and similar arithmetic lines |
| ESTIMATE | Analogue-based planning value at 2026 price levels | Majority of register lines |
| PENDING VALIDATION | OEM quotation, export-control or qualification closure required before commitment | M88 engine, gearbox adaptation, nozzle treatment, FCS evidence, crypto, recycled Ti powder |

Public anchors used: Safran published M88-2 mass, thrust and service-record data (no published price; the engine line is PENDING VALIDATION); NVIDIA Jetson AGX Orin 64GB production module public distribution price (US$3,199); WESCAM MX-10-class public procurement record as the EO/IR price-class anchor; Saab and Diehl public IRIS-T data as the weapon fit anchor; MBDA SmartGlider Light public data; Heroux-Devtek Gripen E landing-gear public contract record; EcoTitanium/Airbus closed-loop titanium public record; aerospace carbon-fibre and prepreg market price bands; supplier public contact pages for all Part VII pathways. USD anchors convert at the controlled 1.35 CAD/USD planning rate.

# Appendix D - Document Certification

This procurement package is compiled against the clean baseline SPRINTER-v6.1-BL (The Optimal Sprinter Technical Specifications v6.1) and the Opening Doors Doctrine DCT-OD-2026-001. All quantities, costs and lead times are planning values at 2026 price levels in Canadian dollars; the single USD anchor converts at the controlled 1.35 CAD/USD planning conversion. NSN class codes are placeholders pending codification. Supplier engagements described in Part VII are outreach pathways, not commitments; no supplier has been contacted under this package. Store separation, M88 supply and French export authorisation, gearbox extraction, installed deck, and fastener counts remain pending validation as stated, with the fallbacks of the baseline (EJ200 engine class; EW-only bay; virgin-material fallback). No register of corrections or errors is maintained, consistent with the clean-baseline policy.

| Role | Entry |
| --- | --- |
| Compiled by | Programme office, air vehicle procurement cell |
| Baseline authority | SPRINTER-v6.1-BL, CDR 70-80% design maturity, 2026-09-06 |
| Cost basis | 2026 CAD; residual USD anchor at controlled 1.35 planning conversion |
| Supply-chain rule | ITAR-free; Canadian first, allied non-US second |
| Issue | v6.1 - clean baseline issue |

Heroux-Devtek, Contract with Saab AB to manufacture complete landing gear systems for the Gripen E fighter aircraft, 2016-12-14, https://investors.herouxdevtek.com/2016-12-14-HEROUX-DEVTEK-ANNOUNCES-CONTRACT-WITH-SAAB-AB-TO-MANUFACTURE-COMPLETE-LANDING-GEAR-SYSTEMS-FOR-THE-GRIPEN-E-FIGHTER-AIRCRAFT.

Saab, IRIS-T public product page, accessed 2026-09-05, https://www.saab.com/products/iris-t.

Saab, IRIS-T public product page, accessed 2026-09-05, https://www.saab.com/products/iris-t.

Diehl Defence, IRIS-T integration on fighter aircraft platforms public release, accessed 2026-09-05, https://new.diehl.com/defence/en/press-media/news/diehl-defence-enhances-integration-capabilities-of-iris-t-missile-into-fighter-aircraft-platforms.

MBDA, SmartGlider Light public product information, accessed 2026-09-06, https://www.airforce-technology.com/projects/smartglider-light-air-launched-guided-weapon/.

Safran Aircraft Engines, M88 public product page (50/75 kN rating, modular on-condition maintenance, one-hour removal/reinstallation, one million Rafale fleet flight hours), accessed 2026-09-06, https://www.safran-group.com/products-services/m88-proven-performance-and-reliability.

Airbus, Increasing titanium and aluminium circularity, 2025-07-10, https://www.airbus.com/en/newsroom/stories/2025-07-waste-not-want-not-increasing-titanium-and-aluminium-circularity.

Heroux-Devtek, Contract with Saab AB to manufacture complete landing gear systems for the Gripen E fighter aircraft, 2016-12-14, https://investors.herouxdevtek.com/2016-12-14-HEROUX-DEVTEK-ANNOUNCES-CONTRACT-WITH-SAAB-AB-TO-MANUFACTURE-COMPLETE-LANDING-GEAR-SYSTEMS-FOR-THE-GRIPEN-E-FIGHTER-AIRCRAFT.

Safran, Canada country page (landing systems, electrical, and support footprint), accessed 2026-09-06, https://www.safran-group.com/countries/canada.

Diehl Defence, IRIS-T integration on fighter aircraft platforms public release, accessed 2026-09-05, https://new.diehl.com/defence/en/press-media/news/diehl-defence-enhances-integration-capabilities-of-iris-t-missile-into-fighter-aircraft-platforms.

Airbus, Increasing titanium and aluminium circularity, 2025-07-10, https://www.airbus.com/en/newsroom/stories/2025-07-waste-not-want-not-increasing-titanium-and-aluminium-circularity.

Rolls-Royce / EUROJET, EJ200 public product page, accessed 2026-09-06, https://www.rolls-royce.com/products-and-services/defence/aerospace/combat-jets/ej200.aspx.

MBDA, SmartGlider Light public product information, accessed 2026-09-06, https://www.airforce-technology.com/projects/smartglider-light-air-launched-guided-weapon/.

Safran Aircraft Engines, M88 public product page (50/75 kN rating, modular on-condition maintenance, one-hour removal/reinstallation, one million Rafale fleet flight hours), accessed 2026-09-06, https://www.safran-group.com/products-services/m88-proven-performance-and-reliability.

Safran, Canada country page (landing systems, electrical, and support footprint), accessed 2026-09-06, https://www.safran-group.com/countries/canada.

Safran Aircraft Engines, M88 public product page (50/75 kN rating, modular on-condition maintenance, one-hour removal/reinstallation, one million Rafale fleet flight hours), accessed 2026-09-06, https://www.safran-group.com/products-services/m88-proven-performance-and-reliability.

Safran, Canada country page (landing systems, electrical, and support footprint), accessed 2026-09-06, https://www.safran-group.com/countries/canada.

Safran Aircraft Engines, M88 public product page (50/75 kN rating, modular on-condition maintenance, one-hour removal/reinstallation, one million Rafale fleet flight hours), accessed 2026-09-06, https://www.safran-group.com/products-services/m88-proven-performance-and-reliability.

Safran Aircraft Engines, M88 public product page (50/75 kN rating, modular on-condition maintenance, one-hour removal/reinstallation, one million Rafale fleet flight hours), accessed 2026-09-06, https://www.safran-group.com/products-services/m88-proven-performance-and-reliability.

Heroux-Devtek, Contract with Saab AB to manufacture complete landing gear systems for the Gripen E fighter aircraft, 2016-12-14, https://investors.herouxdevtek.com/2016-12-14-HEROUX-DEVTEK-ANNOUNCES-CONTRACT-WITH-SAAB-AB-TO-MANUFACTURE-COMPLETE-LANDING-GEAR-SYSTEMS-FOR-THE-GRIPEN-E-FIGHTER-AIRCRAFT.

Safran Aircraft Engines, M88 public product page (50/75 kN rating, modular on-condition maintenance, one-hour removal/reinstallation, one million Rafale fleet flight hours), accessed 2026-09-06, https://www.safran-group.com/products-services/m88-proven-performance-and-reliability.

Saab, IRIS-T public product page, accessed 2026-09-05, https://www.saab.com/products/iris-t.

Diehl Defence, IRIS-T integration on fighter aircraft platforms public release, accessed 2026-09-05, https://new.diehl.com/defence/en/press-media/news/diehl-defence-enhances-integration-capabilities-of-iris-t-missile-into-fighter-aircraft-platforms.

MBDA, SmartGlider Light public product information, accessed 2026-09-06, https://www.airforce-technology.com/projects/smartglider-light-air-launched-guided-weapon/.

Heroux-Devtek, Contract with Saab AB to manufacture complete landing gear systems for the Gripen E fighter aircraft, 2016-12-14, https://investors.herouxdevtek.com/2016-12-14-HEROUX-DEVTEK-ANNOUNCES-CONTRACT-WITH-SAAB-AB-TO-MANUFACTURE-COMPLETE-LANDING-GEAR-SYSTEMS-FOR-THE-GRIPEN-E-FIGHTER-AIRCRAFT.

Airbus, Increasing titanium and aluminium circularity, 2025-07-10, https://www.airbus.com/en/newsroom/stories/2025-07-waste-not-want-not-increasing-titanium-and-aluminium-circularity.