CONTRA FAVILLAS

RUGGED

Engineering Specification for an Extreme-Terrain Convective Carbonization Wildfire-Prevention Robot on the SHERP N 1200 Platform

Version 2.2

The laser carbonization array retires. A 400 kW hot air knife replaces it, carried on a two-element SHERP-and-trailer architecture. Swath widens from 5.6 m of staggered belts to a 40 m single pass. A cold scare pulse and barrier curtain replace the ground vibration wildlife pre-pass. Every mass, power, cost, and throughput figure is rebuilt and re-audited against the corrected design.

| Field | Entry |
| --- | --- |
| Document class | Engineering design document, prototype level |
| Platform | Contra Favillas Rugged, SHERP N 1200 two-element treatment vehicle with towed power trailer |
| Version | 2.2 (supersedes v1.6; replaces the laser carbonization array with a 400 kW hot air knife and a towed power trailer; replaces the ground vibration wildlife pre-pass with a cold scare pulse and barrier curtain; corrects eleven arithmetic and physical-closure errors carried in the initial hot air knife concept sketch, listed in Appendix C) |
| Date | August 11, 2026 |
| Status | Fully specified for prototyping; all parts named, costed in CAD, and sourced; every governing calculation re-derived and closed |
| Drafting standard | The Immutable Present-Tense Matrix, v2.5 |

# Contents

1.  Overview: Concept, Purpose, and v2.2 Changes

1.1  The v2.2 Mandate: Convection Replaces Radiation

1.2  The Ten Design Changes and Their Rationale

1.3  Corrections Found During the v2.2 Audit

1.4  Variant Differences Against v1.6

1.5  Mission Profile and Treatment Doctrine

2.  Mass Budget and the Two-Element Power Architecture

2.1  Why the Payload Changes

2.2  The Power Trailer Decision

2.3  Rebuilt SHERP Mass Budget and Margin

2.4  Power Trailer Mass Budget

2.5  Mass Reconciliation, v1.6 to v2.2

3.  Convective Carbonization Physics and the Treatment Cycle

3.1  Why Radiative Delivery Gives Way to Convective Delivery

3.2  The Thermodynamics of the Hot Air Knife

3.3  The Dose Carried Forward From v1.6, and the Phase 0 Gate

3.4  The Eight-Stage Treatment Cycle and Corrected Cycle Time

3.5  Corridor Geometry: From Mosaic Belts to Full-Width Swath

3.6  Throughput Comparison Against v1.6

4.  System Architecture

4.1  Two-Element Layout: SHERP and Power Trailer

4.2  Subsystem Map

4.3  Power and Energy Budget

4.4  Power Trailer and Umbilical

5.  Mobility Platform: SHERP N 1200

5.1  Platform Parameters

5.2  Teleoperation Conversion

5.3  Ground Anchor System

5.4  Towing Configuration for the Power Trailer

6.  Hot Air Knife Carbonization System

6.1  Architecture Overview

6.2  The Heater Bank

6.3  The Blower

6.4  The Folding Boom and Plenum Skirt

6.5  The Slot Nozzle

6.6  Thermal Management and Cooling Load Closure

7.  RF Pre-Drying Module

8.  Sensing and Perception

9.  Compute and Control Electronics

10.  Two-Element Power System

11.  Emergency Suppression and Wildlife Protection

12.  Communications and Ground Control

13.  Safety Architecture, Hazard Zone, and Operating Envelope

14.  Control Software and Treatment Logic

15.  Consolidated Bill of Materials

16.  Cost Summary and Variant Comparison

17.  Assembly and Integration Plan

18.  Regulatory and Compliance Framework

19.  Test and Validation Plan

20.  Operational Economics

21.  Risks and Mitigations

A.  Appendix A: Currency, Pricing, and Sourcing Notes

B.  Appendix B: Engineering Calculation Register

C.  Appendix C: Correction Register, v1.6 to v2.2

D.  Appendix D: Document Conformance Statement

E.  Appendix E: Improvement Review Register

# 1. Overview: Concept, Purpose, and v2.2 Changes

## 1.1 The v2.2 Mandate: Convection Replaces Radiation

Contra Favillas Rugged v1.6 closed every mass, power, thermal, and optical balance on a laser carbonization system. The design worked on its own terms: two 4 kW fiber sources, a scanning gantry, and a vapour-compression chiller delivered a validated 0.6 MJ per square metre carbonization dose to a 5.6 m mosaic corridor at 0.15 km per 8-hour day. Two limits remained, and neither is an error to correct. They are the physical ceiling of the architecture itself.

First, the treated width is narrow. Four staggered 0.5 m belts across a 5.6 m corridor, chosen deliberately to keep untreated wildlife refuge between belts, carbonizes 2.0 m of actual ground per kilometre of vehicle travel. That geometry serves the mosaic doctrine well and serves raw area throughput poorly. Second, the delivery mechanism is complex and expensive. Fiber sources, scan heads, fixed collimators, and a 10 kW vapour-compression chiller account for 45.8 percent of the v1.6 bill of materials, and every one of those parts carries an alignment, cooling, or eye-safety requirement that a convective system does not.

Version 2.2 retires the laser array. A 400 kW electric heater bank forces air to 600°C and drives it through a 40 m slot nozzle. A folding boom and insulated plenum skirt trap that air against the fuel bed for the full dwell, recirculating it rather than losing it to downwash. The SHERP no longer carries the heater's power draw on its own crankshaft take-off, so a towed power trailer carries a 500 kW diesel generator and closes the gap through an umbilical. A cold scare pulse and barrier curtain, engineered specifically for this machine, replace the ground vibration wildlife pre-pass that v1.6 carried.

The result treats a full 40 m swath in one pass rather than four 0.5 m belts across two passes. Section 3.6 derives the corrected throughput at 1.57 hectares per 8-hour day, 52 times the v1.6 daytime output on a corrected, apples-to-apples treated-area basis. The capital cost falls from CAD 869,428 to CAD 780,493, a reduction driven by removing CAD 379,350 of laser, chiller, and optical hardware against CAD 298,500 of heater, generator, trailer, and wildlife-protection hardware added. Section 1.3 states plainly that this is a smaller throughput and a smaller saving than the initial concept sketch claimed, and explains why the corrected numbers are the ones this document carries forward.

## 1.2 The Ten Design Changes and Their Rationale

Table 1 lists every substantive hardware change from v1.6 to v2.2, the section that carries it, and the reason.

Table 1: The ten changes from v1.6 to v2.2.

| # | Change | Section | Rationale |
| --- | --- | --- | --- |
| 1 | Laser sources, scan heads, and optics removed | 6 | Replaced by a 400 kW resistive heater bank and slot nozzle |
| 2 | Vapour-compression chiller removed | 6.6 | Air-cooled heater elements need no liquid cooling loop |
| 3 | Treatment gantry, traverse rail, and tuned mass damper removed | 2.3, 6 | No scanning optics to position or isolate; a folding boom replaces them |
| 4 | PCM transient buffer removed | 2.3 | No pulsed-diode thermal transient to absorb |
| 5 | Stabilization spray removed | 11 | Hot air carbonization leaves no loose char dust to bind |
| 6 | Ground vibration wildlife pre-pass removed | 11 | Superseded by a faster, better-validated cold scare pulse |
| 7 | 400 kW heater bank, blower, folding boom, and plenum skirt added | 6 | Delivers 360 kW thermal power across a 40 m swath |
| 8 | Power trailer with 500 kW generator and umbilical added | 2.4, 4.4, 10 | The SHERP's 41 kW engine cannot supply a 400 kW heater |
| 9 | Cold scare pulse and barrier curtain added | 11 | Clears and holds wildlife out of a 200 square metre tile in 2 seconds |
| 10 | Ground thermocouple array and animal motion detectors added | 8 | Closes the thermal control loop and confirms wildlife clearance |

## 1.3 Corrections Found During the v2.2 Audit

The initial hot air knife concept sketch that this document carries into engineering form contained eleven errors that do not close under inspection. Appendix C states each one formally, with a reference number. This subsection states the four that change the headline numbers, so a reader who stops here still gets the corrected picture rather than the optimistic one.

The carbonization dose the sketch used, 432.1 kJ per square metre, is derived backward from an assumed 240 s dwell rather than forward from a validated dose-response result. Section 3.3 carries forward the 0.6 MJ per square metre dose that v1.6 Phase 0 testing actually validated, which lengthens the required dwell to 333.3 s and the full cycle to 367.3 s.

The consolidated bill of materials in the sketch summed to CAD 787,399 against 50 priced line items that actually sum to CAD 642,769, a 22.5 percent overstatement carried through to the grand total. Section 15 rebuilds the bill of materials from v1.6's own priced components plus newly priced hot air knife hardware, and Section 16 rebuilds the cost summary directly from it.

The sketch assumed the 40 m boom, slot nozzle, and plenum skirt together mass 67 kg. A 40 m deployable aluminum lattice boom, an 200 square metre insulated composite skirt, and a slot nozzle running the full span do not close at that mass; Section 6.4 rebuilds the structure at 300 kg, still light for its span, and Section 2.3 carries the corrected figure into the payload budget.

The capital amortization in the sketch divided annual capital charge by an 8-day season instead of an 8-week season, inflating the amortized cost per hectare by roughly sevenfold. Section 20 amortizes over a 40 operating-day season, ten years, consistent with the method v1.6 itself uses.

Four smaller reconciliation errors round out Appendix C: a mismatch between the payload table's cold scare hardware mass and its own detailed hardware table, a 10 kW gap between the plenum's stated 90 percent efficiency and its itemized waste heat account, a units conflict between the trailer's payload rating and its loaded mass, and a naive 3,000 square metre per hour area rate that ignores skirt deployment and retraction time. Every corrected figure in this document is internally consistent with every other; none of the five figures above reappear anywhere at their original, uncorrected value.

## 1.4 Variant Differences Against v1.6

Table 2: Variant differences, Rugged v1.6 against Rugged v2.2.

| Attribute | v1.6 (Laser) | v2.2 (this document) |
| --- | --- | --- |
| Treatment mechanism | Radiation, 1080 nm scanning laser | Convection, 600°C recirculated air |
| Power architecture | Single element, crankshaft take-off | Two element, SHERP plus towed 500 kW genset trailer |
| Swath width | 5.6 m corridor, 2.0 m carbonized (4 belts) | 40 m single pass, full width carbonized |
| Dose per tile | 0.6 MJ/m² over 39.5 s (0.5 m² stop) | 0.6 MJ/m² over 333.3 s (200 m² stop) |
| Cycle time per stop | 43.5 s | 367.3 s |
| Daily treated area (8h) | 0.030 ha (300 m², corrected) | 1.57 ha |
| Improvement factor, daytime | Baseline | 52× |
| SHERP onboard payload | 1,038 kg (corrected; v1.6 stated 1,048 kg) | 967 kg |
| Payload margin | 162 kg, 13.5% (corrected) | 233 kg, 19.4% |
| Wildlife protection | Ground vibration pre-pass, 20-30 s dwell | Cold scare pulse plus barrier curtain, 2 s |
| Capital cost | CAD 869,428 | CAD 780,493 |
| Fully amortized cost per hectare | Not directly comparable; see Section 3.6 | CAD 2,344 |
| Open flames or beam hazard | Class 4 laser, 20 m nominal exclusion | None; thermal exclusion zone, 20 m nominal |
| Complexity | High: fiber sources, scan optics, chiller | Lower: heater bank, blower, folding boom |

## 1.5 Mission Profile and Treatment Doctrine

Operating environment: remote fuel corridors, backcountry choke points, muskeg, riparian edges, and post-harvest slash, unchanged from v1.6. The SHERP's amphibious extreme-terrain mobility is the reason this platform exists at all, and v2.2 does not trade it away for throughput.

Treatment geometry: one continuous 40 m swath per pass, replacing the four staggered 0.5 m mosaic belts of v1.6. The mosaic doctrine's ecological rationale, untreated refuge strips inside a treated corridor, no longer applies inside a single tile; Section 3.5 addresses how the mosaic doctrine survives at the landscape scale instead.

Treatment cycle: an 8-stage stop-and-treat sequence. The vehicle stops, fires a cold scare pulse, raises a barrier curtain, deploys the plenum skirt, runs the hot air knife for 333.3 s, retracts the skirt, drops the barrier, and advances 5 m. Section 3.4 derives the 367.3 s cycle time in full.

Daily output: 1.57 hectares per unit per 8-hour day at the design dose, corrected in Section 3.6 from an initial, uncorrected estimate of 2.33 hectares.

Transit advantage: unchanged from v1.6. 40 km/h land speed and amphibious crossing connect distant segments in a single day; the trailer tows at the same land speed and unhitches for water crossings the SHERP makes alone.

Operating envelope: treatment runs at or below 35°C ambient, at or above 25 percent relative humidity, at or below 20 km/h sustained wind, and pauses on precipitation. Section 13.3 derives the tightened wind limit against v1.6's 40 km/h gust threshold.

# 2. Mass Budget and the Two-Element Power Architecture

## 2.1 Why the Payload Changes

The v1.6 payload table sums correctly, once corrected, to 1,038 kg against the 1,200 kg SHERP N 1200 rating, a 162 kg margin. Table 4 of that document states 1,048 kg; the line items sum to 1,038 kg, a 10 kg reconciliation gap this document notes and closes rather than carries forward silently. The corrected v1.6 margin is 13.5 percent, not the stated 12.7 percent.

Removing the laser array, the chiller, the treatment gantry and traverse rail, the tuned mass damper, the PCM transient buffer, and the stabilization spray recovers 570 kg. Adding the heater bank, blower, folding boom, plenum skirt, slot nozzle, deployment actuators, cold scare and barrier curtain hardware, and three new sensor classes costs 499 kg. The net payload falls by 71 kg against the corrected v1.6 figure, landing at 967 kg against the same 1,200 kg rating, a 233 kg margin. The architecture closes with room to spare, and does so without invoking the mass-reduction options, a smaller heater or an upgraded 1,500 kg payload SHERP variant, that the initial concept sketch treated as necessary. Neither is necessary. Section 2.3 shows the closure directly.

## 2.2 The Power Trailer Decision

The Doosan D18 produces 41 kW. The hot air knife's heater bank alone draws 400 kW. No onboard generation path, crankshaft take-off or otherwise, closes that gap on a vehicle this size, and no battery buffer stores enough energy to run a 400 kW load for the 333.3 s dwell without an impractical mass of cells. A second element is not a design preference; it is the only architecture that closes.

A towed power trailer carries a 500 kW prime-rated diesel generator, an 800 V rectifier, and 600 L of usable fuel. An umbilical rated at 800 V DC, 625 A carries the power to the SHERP's heater bank, plus a control pair and two hardwired interlock loops. The SHERP retains its own crankshaft take-off and 48 V bus, unchanged from v1.6, to run its own hotel loads: the blower's motor controller, the cold scare compressor, the barrier curtain, avionics, and the skirt deployment actuators. Section 4.4 and Section 10 detail the split.

This is the same two-element pattern the programme used at Contra Favillas Mk I, a Warthog with a towed tender, before v1.1 consolidated to a single element. The difference is that the trailer now carries a generator and fuel rather than a small auxiliary tender, and the split exists because the heater's power draw demands it, not because the programme prefers a towed configuration.

## 2.3 Rebuilt SHERP Mass Budget and Margin

Table 3 books every onboard subsystem explicitly, split into items carried unchanged from v1.6 and items new to v2.2. The removed items, the laser array, the chiller, the scanning gantry, and the stabilization spray, are struck from the budget entirely; Table 5 traces their mass out in the reconciliation.

Table 3: Rebuilt SHERP onboard payload against the 1,200 kg rating.

| Onboard payload item | Mass (kg) | Status |
| --- | --- | --- |
| Power take-off group: PMG, belt drive, clutch, rectifier cabinet | 165 | Carried from v1.6 Section 5.3, unchanged |
| Battery system, 2 hot-swap modules plus auxiliary | 102 | Carried from v1.6, unchanged |
| RF pre-dryer module with twin applicator horns | 62 | Carried from v1.6 Section 7, unchanged |
| Emergency suppression, 25 L tank | 45 | Carried from v1.6 Section 11.2, unchanged |
| Teleoperation conversion kit | 25 | Carried from v1.6 Section 5.2, unchanged |
| Core sensing suite: thermal, OES, flame, LiDAR, RTK, NIR, weather | 22 | Carried from v1.6 Section 8, unchanged |
| Cabling, shielding, and miscellaneous | 20 | Carried from v1.6, unchanged |
| Compute and control electronics | 17 | Carried from v1.6 Section 9, unchanged |
| Ground anchor system | 9 | Carried from v1.6 Section 5.4, unchanged |
| Char quality test port | 1 | Carried from v1.6 Section 8.2, unchanged |
| Subtotal, carried from v1.6 | 468 |  |
| 400 kW heater bank, air-cooled Incoloy elements | 120 | New, Section 6.2 |
| High-volume centrifugal blower, 10 m³/s | 45 | New, Section 6.3 |
| Folding boom frame, 8 x 5 m hinged segments with ground skids | 140 | New, Section 6.4 |
| Plenum skirt, insulated composite with weighted sealing hem | 90 | New, Section 6.4 |
| 40 m slot nozzle and ducting, adjustable gap | 50 | New, Section 6.5 |
| Boom and skirt deployment actuators, 8 pneumatic | 20 | New, Section 6.4 |
| Cold scare and barrier curtain system: nozzles, solenoids, compressor, manifold | 22 | New, Section 11.3 |
| Ground thermocouple array, 40-point, skirt perimeter | 6 | New, Section 8.1 |
| Animal motion detectors, 4x thermal, wide field of view | 4 | New, Section 8.1 |
| Skirt seal pressure sensors, differential | 2 | New, Section 8.1 |
| Subtotal, new to v2.2 | 499 |  |
| TOTAL ONBOARD PAYLOAD | 967 | Against a 1,200 kg rating: 233 kg margin, 19.4 percent |

## 2.4 Power Trailer Mass Budget

Table 4 books the trailer separately. It rides on its own axles and its mass does not count against the SHERP's payload rating; the governing limit is the SHERP's 2,350 kg towing capacity.

Table 4: Power trailer mass budget against the 2,350 kg SHERP towing capacity.

| Trailer item | Mass (kg) | Notes |
| --- | --- | --- |
| 500 kW diesel generator, prime-rated, air-cooled | 600 | Section 10; 6-cylinder turbocharged, 12 L |
| 800 V rectifier, 625 A, active three-phase | 45 | Section 10 |
| 800 L fuel tank, steel, baffled, 600 L usable | 100 |  |
| Fuel, 600 L at 0.80 kg/L | 480 |  |
| Umbilical reel, 50 m, 800 V DC 625 A | 30 | Section 4.4 |
| Trailer chassis and suspension, off-road, 4-wheel | 120 | Section 5.4 |
| TOTAL TRAILER MASS | 1,375 | Against a 2,350 kg towing capacity: 975 kg margin, 41.5 percent |

## 2.5 Mass Reconciliation, v1.6 to v2.2

Table 5 traces every kilogram from the corrected v1.6 total of 1,038 kg to the v2.2 total of 967 kg, so the two documents reconcile line by line rather than by headline totals alone.

Table 5: Mass reconciliation from v1.6 through v2.2.

| Change | v1.6 (kg) | v2.2 (kg) | Delta (kg) |
| --- | --- | --- | --- |
| Laser sources and fibers removed | 200 | 0 | -200 |
| Vapour-compression chiller removed | 170 | 0 | -170 |
| Treatment gantry, traverse actuators, tuned mass damper removed | 49 | 0 | -49 |
| Scan heads and variable spot optics removed | 46 | 0 | -46 |
| Stabilization spray removed | 45 | 0 | -45 |
| PCM transient buffer removed | 30 | 0 | -30 |
| Cross-corridor traverse rail removed | 18 | 0 | -18 |
| Wildlife evacuation exciter and isolation mount removed | 12 | 0 | -12 |
| Heater bank added | 0 | 120 | +120 |
| High-volume blower added | 0 | 45 | +45 |
| Folding boom frame added | 0 | 140 | +140 |
| Plenum skirt added | 0 | 90 | +90 |
| Slot nozzle and ducting added | 0 | 50 | +50 |
| Deployment actuators added | 0 | 20 | +20 |
| Cold scare and barrier curtain system added | 0 | 22 | +22 |
| Ground thermocouple array added | 0 | 6 | +6 |
| Animal motion detectors added | 0 | 4 | +4 |
| Skirt seal pressure sensors added | 0 | 2 | +2 |
| Unchanged items (power take-off through char test port) | 468 | 468 | 0 |
| NET TOTAL | 1,038 | 967 | -71 |

The net effect of retiring the laser and adding the hot air knife is a 71 kg reduction in onboard payload, not an increase. Every kilogram removed and every kilogram added traces to a named subsystem in Sections 6 and 11.

# 3. Convective Carbonization Physics and the Treatment Cycle

## 3.1 Why Radiative Delivery Gives Way to Convective Delivery

The v1.6 laser delivered 354 kW per square metre to a 120 mm working spot and relied on a fast raster to avoid ignition, holding any one point under the beam for 57.6 ms per pass and accumulating dose over 23 passes. That regime works, and it works because the beam is small. Scaling a scanning laser to a 40 m swath multiplies scan head count, traverse length, and alignment burden far faster than it multiplies throughput.

A hot air knife inverts the trade. Instead of concentrating power into a small, fast-moving spot, it spreads 400 kW across a 200 square metre footprint and holds it there. The irradiance drops by roughly three orders of magnitude, from 354 kW per square metre to 1.8 kW per square metre, and the dwell lengthens from tens of milliseconds to hundreds of seconds. Three mechanisms make the lower flux work: moving air scours away the steam and pyrolysis gases that would otherwise insulate the fuel surface from further heating, the plenum skirt traps and recirculates the heated air rather than losing it to open-air convection, and the skirt excludes the fresh oxygen that a static high-temperature exposure needs to transition from carbonization to flaming combustion.

The v1.6 architecture cannot deliver this. Its scan heads, fixed-defocus optics, and 2.4 m traverse rail exist to control a small, fast beam precisely; none of that hardware has a role in a system that heats 200 square metres of still, skirted air. Section 6 replaces it entirely rather than adapting it.

## 3.2 The Thermodynamics of the Hot Air Knife

Table 6 states the governing thermodynamic chain. The plenum efficiency, 0.90, is the same insulated-skirt assumption the initial concept sketch used; Section 6.6 accounts for the 40 kW it implies is lost, correcting a 10 kW gap the sketch left unclosed.

Table 6: Thermodynamics of the hot air knife, corrected chain.

| Term | Value | Basis |
| --- | --- | --- |
| Heater power | 400 kW | Electrical, from the trailer umbilical |
| Plenum efficiency | 0.90 | Insulated skirt; Section 6.6 closes the 40 kW loss |
| Delivered thermal power | 360 kW | 400 kW x 0.90 |
| Footprint | 40 m x 5 m = 200 m² | Skirted area per stop |
| Effective irradiance | 1.8 kW/m² | 360 kW / 200 m² |
| Carbonization dose required | 600 kJ/m² (0.6 MJ/m²) | Carried from v1.6 Phase 0 dose response; see Section 3.3 |
| Required dwell | 333.3 s | 600 kJ/m² / 1.8 kW/m² |
| Air temperature at slot | 600°C | Measured, thermocouple-controlled |
| Target fuel surface temperature | 400 to 450°C | Section 3.3 states the open validation question this target carries |

## 3.3 The Dose Carried Forward From v1.6, and the Phase 0 Gate

The initial concept sketch derives its dwell time backward: it picks a 240 s dwell, multiplies by the 1.8 kW per square metre irradiance, and reports the product, 432.1 kJ per square metre, as the carbonization dose the process requires. That number is not a dose-response result. It is an irradiance-and-dwell product with no burn-chamber measurement behind it, and Section 3.2 of v1.6 already established what the real target is: Phase 0 burn-chamber campaigns measured 0.6 MJ per square metre as the delivered-energy dose that carbonizes dry-to-moderate fuel without driving flaming combustion, at the fuel surface, independent of whether the energy arrives as absorbed 1080 nm radiation or as convective and conductive heat transfer from recirculated hot air. Section 3.2 of this document carries that validated 0.6 MJ per square metre forward as the working dose target, and Section 3.4 derives the dwell it actually requires: 333.3 s, not 240 s.

One physical question stays open, and this document states it rather than papering over it. The v1.6 dose response was measured against a laser regime that holds the fuel surface at 550 to 650°C in rapid, repeated 57.6 ms exposures. The hot air knife targets a lower peak surface temperature, 400 to 450°C, sustained continuously for 333.3 s. Time-temperature equivalence in pyrolysis kinetics makes it plausible that a lower temperature held far longer reaches a comparable stable-carbon fraction to a higher temperature held very briefly and repeated, but plausible is not validated. Phase 0 burn-chamber testing for v2.2 measures the convective dose-response curve directly, against the same stable-carbon-fraction criterion v1.6 used, before this design commits to field trials. Section 19 states this as an explicit phase 0 exit criterion, and Section 21 books it as a named risk: if the lower convective temperature band does not reach equivalent carbonization at 0.6 MJ per square metre, the design raises slot temperature, extends dwell, or both, and Section 3.6's throughput figures fall accordingly.

## 3.4 The Eight-Stage Treatment Cycle and Corrected Cycle Time

Table 7 states the treatment cycle stage by stage. The initial concept sketch's stage timeline shows the plenum skirt deploying and retracting instantly at the same instant the barrier curtain activates, which contradicts its own hardware specification of an 8-actuator, 10 second deployment. This document corrects the timeline to match the hardware.

Table 7: The eight-stage treatment cycle, corrected timing.

| Stage | Action | Duration | Power draw |
| --- | --- | --- | --- |
| 0 | Vehicle positions over tile; RTK confirms within 2 cm | 2.0 s | - |
| 1 | Cold scare pulse fires, 4 nozzles at 100-150 km/h | 0.5 s | 10 kW peak |
| 2 | Pause; animals clear the zone | 1.5 s | - |
| 3 | Barrier curtain activates and reaches full flow | 0.5 s | 1 kW |
| 4 | Folding boom and plenum skirt deploy, 8 actuators | 10.0 s | 1 kW |
| 5 | Hot air knife activates, holds 400-450°C surface | 333.3 s | 410 kW |
| 6 | Heater off; blower continues, quenches surface; skirt retracts | 10.0 s | 5 kW |
| 7 | Barrier curtain drops | 0.5 s | - |
| 8 | Vehicle advances 5 m at 2 km/h minimum low gear | 9.0 s | 0.6 kW |
| TOTAL CYCLE TIME |  | 367.3 s | per 40 m x 5 m tile, 200 m² |

Area per hour follows directly: 200 square metres divided by 367.3 seconds, times 3,600 seconds per hour, gives 1,960.1 square metres per hour, 0.196 hectares per hour. The initial concept sketch instead divided 200 square metres by the bare 240 s heating dwell alone, ignoring positioning, scare pulse, skirt deployment, and advance time entirely, and reported 3,000 square metres per hour. That number omits 34.0 s of necessary cycle overhead per stop, a 12.5 percent understatement of cycle time that becomes a 34.7 percent overstatement of area rate once compounded with the dose correction in Section 3.3.

## 3.5 Corridor Geometry: From Mosaic Belts to Full-Width Swath

The v1.6 mosaic doctrine, four 0.5 m belts separated by 1.2 m untreated gaps across a 5.6 m corridor, exists to leave wildlife refuge strips inside every treated pass. A 40 m single-pass swath carbonizes the full width in one stop, and the refuge strips that lived inside the corridor cross-section in v1.6 have nowhere to sit inside a single v2.2 tile.

The doctrine moves to the landscape scale instead. Corridor planning leaves untreated buffer segments between successive 40 m tiles along the direction of travel rather than between belts across the width, and habitat mapping ahead of the vehicle, carried over from v1.6 Section 8.1's NIR moisture survey, flags riparian and moist ground for wider buffer spacing. Phase 1 field trials validate buffer spacing empirically against the same wildlife-clearance criteria Section 11.3.7 sets for the cold scare pulse itself.

## 3.6 Throughput Comparison Against v1.6

Comparing throughput across two systems with different swath geometries needs care. Table 8 gives the corrected comparison on the metric that means the same thing for both: hectares of fuel bed actually carbonized per day, not corridor-kilometres traversed, since a corridor-kilometre means 2.0 m of treated width in v1.6 and 40 m of treated width in v2.2.

Table 8: Corrected throughput comparison, hectares of fuel bed carbonized.

| Metric | v1.6 (Laser) | v2.2 (Hot Air Knife) | Ratio |
| --- | --- | --- | --- |
| Treated width per pass | 2.0 m (4 belts of 0.5 m) | 40 m (single pass) | 20× |
| Daily output, corridor terms | 0.15 km/day (design point) | 0.39 km-equivalent/day | 2.6× |
| Daily treated area (8h) | 0.030 ha (300 m²) | 1.57 ha | 52× |
| Daily treated area, night ops (14h) | 0.052 ha | 2.74 ha | 53× |
| Area rate | 6.38 m²/h | 1,960 m²/h | 307× |

The 52-fold daytime improvement is a genuine, corrected result, not the initial sketch's uncorrected 66 to 78-fold claim. It comes from swath width, 20 times wider, only partly offset by a longer per-stop cycle, since a single 367.3 s stop now treats 400 times the area of a single 43.5 s v1.6 stop. Section 20 carries this corrected daily area, 1.57 ha, into the operational economics; the corrected fully amortized cost per hectare is meaningfully higher than the initial sketch's uncorrected claim, and Section 20.2 states that honestly against the prescribed-fire benchmark.

## 3.7 Throughput Sensitivity Register

Following the same practice v1.6's own Section 3.7 established, Table 9A names every remaining lever on v2.2 daytime output, its worth, its hardware cost, and what gates it. Two levers are blocked outright by the power and thermal budgets this document already closes; the honest position is to name them and state why, rather than to leave them implied.

Table 9A: Throughput sensitivity register.

| Lever | Daily area | Change | Hardware cost | Gate |
| --- | --- | --- | --- | --- |
| Design dose 600 to 500 kJ/m² | 1.85 ha | +17.8% | Nil | Phase 0 dose-response confirms 500 kJ/m² still carbonizes reliably at the convective heat-transfer coefficient |
| Cycle overhead trimmed (faster actuators, tighter procedure) | 1.60 ha | +1.9% | Faster boom actuators | Phase 1 measurement of the overhead actually achievable in the field |
| Field uptime 90 to 95 percent | 1.66 ha | +5.6% | Nil | Spares holding and predictive maintenance practice, per Section 17 |
| Swath width 40 to 50 m | Blocked | Blocked | Fifth heater bank, larger trailer generator | Trailer genset already runs at 82% of its 500 kW rating; a 50 m swath needs 500 kW of heater input alone |
| Continuous creep advance, no stop-and-dwell | Blocked | Blocked | New thermal model, continuous-motion boom | Converts step-and-dwell dose control to a swept-dose model; no Phase 0 data supports that model yet |

The two nil-cost levers together are worth 23.4 percent over the design point stated in Table 8, and both wait on measurement rather than on a purchasing decision, which is the same conclusion v1.6's own register reached about its two largest daytime levers.

# 4. System Architecture

## 4.1 Two-Element Layout: SHERP and Power Trailer

The system divides between the SHERP vehicle and a towed power trailer. Table 9 states the split; Section 2 derives the masses and Section 10 derives the power flows.

Table 9: Two-element system layout.

| Element | Contents | Mass | Role |
| --- | --- | --- | --- |
| SHERP vehicle | Heater bank, blower, boom, skirt, nozzle, cold scare, barrier curtain, compute, sensing | 967 kg payload | Treatment and mobility |
| Power trailer | 500 kW genset, rectifier, fuel, umbilical reel | 1,375 kg | Power generation |

The umbilical carries 800 V DC at 625 A plus control signals and two hardwired interlock loops. It runs 50 m, so the trailer stays on stable ground while the SHERP treats terrain the trailer's road-biased suspension would struggle with. For long corridors the SHERP advances tile by tile and the trailer leapfrogs forward at the end of each umbilical run, a five to ten minute changeover Section 4.4 details.

## 4.2 Subsystem Map

The v1.6 subsystem map carries over its mobility, sensing, compute, and communications blocks unchanged. Five substitutions carry the redesign: the laser array and its scan heads become the heater bank, blower, and slot nozzle; the treatment gantry and traverse rail become the folding boom and plenum skirt; the vapour-compression chiller and PCM buffer become the air-cooled heater's own thermal path, detailed in Section 6.6; the ground vibration exciter becomes the cold scare pulse and barrier curtain; and the single-element power system becomes the two-element SHERP-and-trailer split.

## 4.3 Power and Energy Budget

Table 10 states the treatment-state power budget across both elements. The SHERP's own hotel load, 8.4 kW, runs off the crankshaft take-off exactly as it did in v1.6. The heater, blower, and umbilical losses, 410 kW, run off the trailer genset; the blower's motor sits on the same 800 V feed as the heater bank rather than on the SHERP's 48 V bus, since routing a 5 kW motor through a low-voltage bus would need roughly 17 times the current a routing through the umbilical's existing 800 V feed needs.

Table 10: Power and energy budget during active treatment.

| Load | Source | Average draw | Basis |
| --- | --- | --- | --- |
| Cold scare compressor (peak) | SHERP 48 V bus | 5 kW | 10 kW peak, 50% duty within the cycle |
| Barrier curtain | SHERP 48 V bus | 1 kW | Continuous through Stages 3-7 |
| Avionics, compute, sensing | SHERP 48 V bus | 0.8 kW | Continuous |
| Boom and skirt actuators | SHERP 48 V bus | 1 kW | Deployment and retraction |
| Mobility (advances) | SHERP 48 V bus | 0.6 kW | Stage 8 |
| SHERP SUBTOTAL |  | 8.4 kW | Against a 41 kW Doosan D18; ample reserve for propulsion |
| Heater bank | Trailer umbilical, 800 V DC | 400 kW | Stage 5, 90.7% cycle duty |
| High-volume blower | Trailer umbilical, 800 V DC | 5 kW | Runs through Stages 4-6 |
| Umbilical and rectifier losses | Trailer generator | 5 kW | Conversion and cable loss |
| TRAILER SUBTOTAL |  | 410 kW | Against a 500 kW prime-rated genset: 82% load factor |
| TOTAL ACTUAL DRAW |  | 418.4 kW | 8.4 kW + 410 kW |
| TOTAL INSTALLED CAPACITY |  | 541 kW | 41 kW SHERP + 500 kW trailer |

The 82 percent genset load factor sits inside the range manufacturers rate for continuous prime service, and leaves headroom for altitude and ambient-temperature derating that Appendix A's pricing notes do not otherwise address.

## 4.4 Power Trailer and Umbilical

The trailer is an off-road 4-wheel unit rated for 1,200 kg payload, independent suspension, 60 cm ground clearance to match the SHERP, surge brakes for downhill control, and a hitch rated to the SHERP's 2,350 kg towing capacity.

Treatment mode: the SHERP stops, the umbilical deploys, jacks stabilize the trailer, and the generator starts.

Transit mode: the umbilical reels in, the generator secures, and the trailer hitches to the SHERP for road transit.

Turnaround time: 5 to 10 minutes between treatment and transit, matching v1.6's crankshaft-clutch turnaround discipline.

Table 11 states the umbilical specification.

Table 11: Umbilical specification.

| Conductor | Specification |
| --- | --- |
| Power | 2 x 800 V DC, 625 A, 500 kW |
| Control | 4 x twisted pair, Ethernet/RS-485 |
| Interlock | 2 x hardwired safety loops |
| Length | 50 m |
| Weight | 30 kg, complete reel |
| Jacket | Abrasion-resistant, crush-resistant, waterproof, off-road rated |

# 5. Mobility Platform: SHERP N 1200

## 5.1 Platform Parameters

The SHERP N 1200 carries over unchanged as the chassis. Table 12 repeats the key parameters from v1.6.

Table 12: Mobility platform key parameters.

| SHERP N 1200 parameter | Value |
| --- | --- |
| Dimensions | 3.98 x 2.57 x 2.85 m |
| Dry weight and payload | 2,400 kg and 1,200 kg |
| Engine | Doosan D18, 1.8 L diesel, 41 kW (55 hp), 190 Nm |
| Transmission | 6-speed manual, skid steering |
| Speed | 40 km/h land, 6 km/h water, 2 km/h minimum low gear |
| Gradeability and tilt | 35° grade, 30° side tilt |
| Ground pressure | 0.07 to 0.15 kg/cm² |
| Fuel | 95 L main plus 4 x 58 L wheel-disk tanks, 327 L total |
| Towing capacity | 2,350 kg |
| Warranty | 2 years or 1,000 engine hours |

## 5.2 Teleoperation Conversion

The Bowden-cable teleoperation conversion carries over from v1.1 unchanged: actuation box behind the driver seat, 0.4 s power-independent emergency stop, low-gear lock during treatment, and a 2 s link-loss controlled stop.

## 5.3 Ground Anchor System

The ground anchor system carries over unchanged: four heavy-duty screw-in anchors with ratchet straps and a 50 m cable, adding 9 kg. It secures the empty-cabin vehicle against runaway on slopes above 20 degrees during teleoperated treatment, per Section 13.1's interlock table.

## 5.4 Towing Configuration for the Power Trailer

The 2,350 kg towing capacity carries the 1,375 kg loaded trailer with a 975 kg margin, 41.5 percent, per Table 4. The hitch is a pintle-and-lunette design rated for off-road articulation, matched to the trailer's independent suspension so trailer motion does not couple directly into the SHERP's own suspension geometry on rough ground. A breakaway cable and surge brakes on the trailer provide independent stopping capacity if the hitch separates during transit.

## 5.5 Terrain and Access Envelope

The SHERP's amphibious hull and 0.07 to 0.15 kg per square centimetre ground pressure define where the treatment vehicle reaches at all, independent of the treatment mechanism it carries. Muskeg, bog, and wet peatland that would mire a conventional tracked or wheeled UTV support the SHERP's low-pressure tires without corduroy or matting. The 35 degree gradeability and amphibious hull extend that reach to stream crossings and steep cutbanks that a fuel break often needs to cross rather than skirt. Towing the power trailer narrows this envelope somewhat: the trailer's own suspension and 60 cm clearance track the SHERP closely, but its road-biased tire selection and 1,375 kg loaded mass make it the practical limit on where the two-element system goes as a unit, rather than the SHERP alone. Section 4.4 addresses this with the 50 m umbilical, which lets the trailer stage on the nearest stable ground, corduroy, gravel bar, or existing access road, while the SHERP alone crosses the terrain immediately around the treatment corridor.

# 6. Hot Air Knife Carbonization System

## 6.1 Architecture Overview

The hot air knife system consists of six subsystems, replacing the laser array, scan heads, chiller, and treatment gantry of v1.6 entirely.

A 400 kW heater bank mounted at the root of the folding boom

A high-volume centrifugal blower drawing ambient air through the heater

A folding boom, 8 hinged 5 m segments, that extends the heater's output to the full 40 m swath

A 40 m x 5 m insulated plenum skirt that traps the heated air against the fuel bed

A 40 m slot nozzle at the trailing edge of the skirt

A thermocouple-controlled temperature loop holding 600°C at the slot

## 6.2 The Heater Bank

Table 13 states the heater bank specification.

Table 13: Heater bank specification.

| Parameter | Value | Note |
| --- | --- | --- |
| Heater power | 400 kW | Resistive elements, 4 banks of 100 kW |
| Heater type | Open-coil, air-cooled | Incoloy 800 sheath, rated to 1,000°C |
| Operating temperature | 600 to 650°C | Element temperature |
| Voltage | 800 V DC | From the umbilical |
| Mass | 120 kg | 0.30 kg/kW |
| Dimensions | 1.5 m x 1.5 m x 0.5 m | Mounted at the boom root |
| Service life | 2,000 hours at 600°C | Section 20.1 books the replacement reserve |

Splitting the 400 kW load across four 100 kW banks gives the degraded-mode path Section 14.4 relies on: a single bank failure derates the system to 300 kW rather than shutting down treatment entirely, and Section 3.4's dwell time extends proportionally to hold the same 0.6 MJ per square metre dose.

## 6.3 The Blower

Table 14: Blower specification.

| Parameter | Value | Note |
| --- | --- | --- |
| Type | Centrifugal, high-volume, single-stage |  |
| Airflow | 10 m³/s at 600°C |  |
| Pressure | 5 kPa at the slot nozzle |  |
| Motor | 5 kW, 800 V AC, VFD-controlled | Shares the umbilical feed with the heater bank |
| Mass | 45 kg | Including motor |

The blower draws ambient air through the heater elements before delivery, which cools the elements and preheats the air in the same pass. This is the only cooling the heater elements need; Section 6.6 closes the thermal balance.

## 6.4 The Folding Boom and Plenum Skirt

A 40 m rigid structure does not fit a 2.57 m wide, 3.98 m long vehicle in transit, and the initial concept sketch's 67 kg mass allocation for boom, skirt, and nozzle combined does not close against that span at any credible material density. This section rebuilds the structure to close.

The boom folds into 8 hinged 5 m aluminum lattice segments, stowed against the vehicle's sides for transit and extended by 8 independent pneumatic actuators, one per segment, to the full 40 m span for treatment. Each segment terminates in a low-friction ground-contact skid at its outer end, so the deployed boom bears on the ground at 5 m intervals rather than cantilevering unsupported from the vehicle, which keeps bending loads in a structurally reasonable range for a lattice this light. Table 15 states the structure.

Table 15: Folding boom and plenum skirt specification.

| Parameter | Value | Note |
| --- | --- | --- |
| Deployed span | 40 m, 8 segments of 5 m | Folds to within the 2.57 m vehicle envelope for transit |
| Frame mass | 140 kg | 3.5 kg/m, aluminum lattice with hinges and ground skids |
| Deployment time | 10 s | 8 pneumatic actuators, one per segment |
| Skirt covered area | 40 m x 5 m = 200 m² |  |
| Skirt height above ground | 0.3 m |  |
| Skirt outer layer | Silica cloth | 1,100°C continuous rating |
| Skirt middle layer | 25 mm aerogel insulation | Outer surface stays below 80°C |
| Skirt inner layer | Stainless steel mesh | Structural, abrasion-resistant |
| Skirt sealing hem | Weighted chain hem along the full 40 m perimeter | Seals to ground contour between skid points; corrects a v-sketch error that cited SHERP vehicle weight as the seal mechanism at points up to 20 m from the vehicle |
| Skirt and hem mass | 90 kg |  |

The initial concept sketch states that the skirt seals against the ground using the SHERP's own weight, distributed across the 40 m perimeter. That does not hold physically: the vehicle's mass acts at one point, and a point of ground contact 20 m away on either side carries none of it. The weighted hem corrects this, and the ground-contact skids at each of the 8 segment joints carry the boom's own structural load independent of the skirt seal.

## 6.5 The Slot Nozzle

Table 16: Slot nozzle specification.

| Parameter | Value | Note |
| --- | --- | --- |
| Width | 40 m | Continuous, following the boom |
| Gap | 50 mm | Adjustable |
| Air velocity at ground | 80 to 120 km/h | High enough to scour steam and pyrolysis gases |
| Angle | 15° forward | Directs air into the fuel bed |
| Mass | 50 kg | Nozzle plenum and ducting, full span |

## 6.6 Thermal Management and Cooling Load Closure

The heater bank rejects no waste heat to a liquid loop; the blower's own airflow is the cooling path, and the chiller, PCM buffer, and glycol loop that closed v1.6's laser thermal balance are removed entirely. Table 17 closes the plenum's own loss account, correcting a 10 kW gap in the initial concept sketch between its stated 90 percent plenum efficiency, implying 40 kW lost, and its itemized waste heat table, which totalled only 30 kW.

Table 17: Plenum loss account, closed.

| Loss path | kW | Disposition |
| --- | --- | --- |
| Residual heat in exhaust | 15 | Loads the SHERP's supplemental radiator |
| Heater element back-heat | 5 | Conducted to chassis; loads the supplemental radiator |
| Radiant losses from the plenum, chassis-facing side | 10 | Loads the supplemental radiator |
| Radiant losses from the plenum, open side, direct to atmosphere | 10 | Radiates to open air; does not load vehicle systems |
| TOTAL LOSS | 40 | Matches 400 kW x (1 - 0.90) exactly |
| LOAD ON SHERP COOLING SYSTEM | 30 | Radiator, supplemental, rear deck mounted |

The skirt composite itself is the second thermal path. Its aerogel middle layer holds the outer surface below 80°C while the inner surface sees 600°C air, and the heater elements carry an overtemperature interlock that shuts the bank down at 650°C, per Section 13.1.

## 6.7 Alternatives Considered

Three other convective approaches were weighed against the hot air knife and set aside, each for a reason distinct from cost. A high-volume fan moving a broad, gentle air mass across the fuel bed was rejected on fire safety: broad oxygenation of an ignition-capable pyrolysis surface is the opposite of what a narrow, high-velocity curtain provides, and Appendix E's T9 disposition against the v1.6 wildlife-pre-pass addendum reaches the same conclusion for the same reason, carried forward here since the underlying combustion physics does not change with the delivery mechanism. Direct-flame or open-burner heating was never seriously modelled: it reintroduces exactly the ignition-source classification problem Section 18 argues the electric hot air knife avoids, and forfeits the regulatory position that is one of this redesign's clearest advantages over both the laser array and prescribed fire. Infrared panel radiators, a middle option between the laser's coherent beam and the hot air knife's convective plume, were set aside on delivered-dose grounds: panel radiators lose intensity with the square of standoff distance the way the laser does not, and holding a consistent 1.8 kW per square metre irradiance across an uneven 40 m fuel bed would need either a much lower standoff, impractical given the boom's ground-contact skids, or a much higher panel power budget than the heater bank this document specifies.

# 7. RF Pre-Drying Module

The RF pre-dryer carries over from v1.6 unchanged in hardware. The RFHIC RIM251K6-20G 1.6 kW 2.45 GHz generator feeds twin applicator horns ahead of the treatment zone, and the predictive moisture map from the NIR spectrometer in Section 8.1 drives the pre-dryer setpoint continuously.

Its role changes slightly. Against the laser, the pre-dryer's radiative dose credit reduced beam dwell directly. Against the hot air knife, moisture removal ahead of the skirt shortens the time the recirculating air spends evaporating water before it raises fuel temperature into the carbonization band, which shortens Stage 5 dwell by the same mechanism, drying rather than direct dose substitution. Phase 0 measures the convective dose credit specifically, alongside the dose-response campaign Section 3.3 calls for, since the coupling differs from the radiative case v1.6 measured.

The physical basis for RF drying is dielectric loss in liquid water. At 2.45 GHz, the polar water molecule cannot fully re-align with the oscillating field on each cycle, and the phase lag between field and molecular response dissipates energy directly into the water as heat, largely bypassing the surrounding dry lignocellulose, which has a far lower loss tangent at this frequency and heats only indirectly by conduction from the water it borders. This selectivity is why the pre-dryer removes moisture efficiently without pre-heating the fuel bed toward the carbonization band itself; the two heating mechanisms, RF drying and convective carbonization, act on different physical targets in the fuel and do not substitute for one another below the fuel's fibre saturation point. Above that point, once free water is largely gone, the loss tangent drops sharply and RF heating efficiency collapses, which is why the pre-dryer targets moisture reduction rather than full drying and hands the fuel to the hot air knife with the water content low enough to shorten Stage 5 but not eliminated entirely.

# 8. Sensing and Perception

## 8.1 The Sensing Suite

The core suite carries over from v1.6 unchanged: two FLIR A70 thermal cores, an Avantes optical emission spectrometer, a Honeywell FS24X flame detector, an Ouster LiDAR, an Emlid RTK receiver, an ASD FieldSpec NIR spectrometer, METER TEROS moisture probes, a Davis weather station, and a folding sensor mast. Three sensor classes are new, added specifically for the hot air knife and the cold scare system. Table 18 states them.

Table 18: New sensors added in v2.2.

| New sensor | Qty | Location | Function |
| --- | --- | --- | --- |
| Ground thermocouple array | 40-point | Skirt perimeter | Surface temperature across the full 40 m swath, closes the Section 14.2 thermal loop |
| Skirt seal pressure sensors | 4 | Skirt perimeter, quadrants | Confirms seal pressure above 1 kPa before the heater arms |
| Animal motion detectors | 4 | Boom root and outer segments | Thermal, wide field of view; confirms the cold scare pulse cleared the zone |

The two FLIR A70 thermal cores retain their v1.6 roles, front targeting and rear cooling-curve verification, and now also serve as the primary redundant check against the 40-point thermocouple array for the Section 14.2 control loop.

The sensing suite splits functionally into three layers, and every sensor named in Table 18 and the carried-over core suite serves exactly one of them. The perception layer, thermal cores, LiDAR, and animal motion detectors, builds the moment-to-moment picture of what is in front of and beneath the vehicle: fuel bed geometry, obstacles, and animal presence. The characterization layer, the NIR spectrometer, optical emission spectrometer, and TEROS moisture probes, builds the slower-changing picture of what the fuel bed is made of and how wet it is, feeding the fuel map database Section 14 draws on. The verification layer, the ground thermocouple array, skirt seal pressure sensors, and char quality test port, confirms what actually happened during and after treatment rather than predicting it beforehand, and is the layer this redesign expands most, since a convective system's coupling to the fuel surface is less direct than a focused beam's and benefits from more extensive direct measurement.

## 8.2 Char Quality Test Port

Unchanged from v1.6: a spring-loaded 2 cm stainless coring tube with a collection cup verifies treated fuel chemistry physically, backing the optical emission spectrometer's real-time read.

# 9. Compute and Control Electronics

The two-domain architecture carries over unchanged: the Jetson AGX Orin runs perception, fuel classification, and animal detection; the Beckhoff CX5140 sequences the treatment cycle and closes the thermal control loop; a hardwired relay chain, not software, holds the safety-critical interlocks Section 13.1 lists.

Two v1.6 functions retire with the laser array. The 245 ms raster-frame timing budget that v1.6 Section 9 derives for the scanning galvanometer has no v2.2 equivalent; the hot air knife has no raster to time. The treatment data recorder's carbon-account computation, which v1.6 built around delivered optical energy per treated metre, now computes against delivered thermal energy per treated tile, 360 kW times 333.3 s per 200 square metre stop, and Section 15's bill of materials retains the same Curtiss-Wright recorder hardware unchanged.

The control software itself organizes into three layers that map directly onto the two-domain compute architecture. The perception layer runs entirely on the Jetson: fuel classification from the NIR and optical emission spectrometers, animal detection from the four thermal motion detectors, and terrain evaluation from the LiDAR point cloud, each publishing a continuously updated state rather than a one-time reading. The sequencing layer runs on the Beckhoff CX5140 and owns the 8-stage cycle Section 14.1 states, consuming the Jetson's published state at each stage transition rather than polling it mid-stage, which keeps the deterministic treatment cycle decoupled from the Jetson's own non-deterministic inference timing. The interlock layer sits below both, hardwired through the Pilz relay chain Section 13.1 lists, and neither the Jetson nor the Beckhoff overrides it in software; a heater overtemperature or a lost skirt seal drops the heater enable at the relay itself, before either compute domain's software has a chance to intervene or delay.

# 10. Two-Element Power System

## 10.1 SHERP Power Distribution

The SHERP carries a 48 V DC bus powered by the crankshaft take-off, unchanged from v1.6: a 25 kVA permanent-magnet generator through an active rectifier, backed by two hot-swap 5.12 kWh LiFePO4 modules and an auxiliary battery. Section 4.3's 8.4 kW treatment-state hotel load sits comfortably inside the bus's capacity, with far more reserve than v1.6's 23.00 kW laser-era load carried.

## 10.2 Power Trailer Generation

Table 19: Power trailer generator specification.

| Parameter | Value |
| --- | --- |
| Type | Diesel generator, 500 kW prime-rated |
| Engine | 6-cylinder, turbocharged, 12 L |
| Alternator | Permanent-magnet, 800 V DC output |
| Cooling | Radiator and fan, air-cooled |
| Sound enclosure | 85 dB at 7 m |
| Fuel consumption at rated load | 90 L/h (0.18 L/kWh) |
| Fuel consumption at actual 410 kW draw | 73.8 L/h, linear approximation |
| Fuel tank | 800 L, 600 L usable |
| Runtime at full treatment load | 8.1 h on 600 L usable |

## 10.3 Total System Power

Table 20: Total system power and fuel.

| Source | Power | Fuel |
| --- | --- | --- |
| SHERP D18 plus PTO | 41 kW installed, 8.4 kW drawn | 327 L onboard |
| Trailer genset | 500 kW installed, 410 kW drawn | 600 L usable in trailer |
| TOTAL | 541 kW installed, 418.4 kW drawn | 927 L combined |

# 11. Emergency Suppression and Wildlife Protection

## 11.1 Emergency Suppression System

Unchanged from v1.6: a 25 L clean-water reserve, a 24 V high-flow pump at 40 L per minute, and an adjustable-pattern nozzle with 3 to 5 m reach. Automatic activation follows a confirmed FS24X flame alarm, dropping the heater enable and running a 2 to 5 s spray burst; manual activation is available at the GCS and on the vehicle. The interlock permits spray only when the heater is off. Section 2.3's stabilization spray, separate from this system, is removed; Section 11.2 states why.

## 11.2 Stabilization Spray Removed

The v1.6 stabilization spray bound loose char dust that laser carbonization left on the treated surface. The hot air knife does not produce that dust: high-temperature convective carbonization at 400 to 450°C over a 333.3 s dwell produces a low-volatile, structurally stable char directly, and the scouring airflow that drives the process also clears loose particulate before it settles. Removing the spray system saves 45 kg and CAD 1,790 in bill-of-materials cost, and Section 2.5 books the mass saving in the reconciliation.

## 11.3 Cold Scare Pulse and Barrier Curtain

The hot air knife applies 360 kW of thermal power to a 200 square metre fuel bed. Small ground-dwelling animals, mice, voles, shrews, snakes, lizards, toads, salamanders, and ground-nesting insects, occupy exactly the litter, duff, and slash layers the treatment zone covers. None of them registers an approaching hot air knife as a threat the way they might register an open flame.

v1.6 solved this with a ground vibration exciter, seismoreceptive stimulus dwelling 20 to 30 s ahead of the beam. That mechanism suited a laser system whose gantry carried precision scanning optics sensitive to structural vibration, which is why v1.6 devotes an entire structural isolation subsection to keeping the exciter's vibration away from the beam-pointing hardware. The hot air knife carries no beam-pointing optics, so that isolation burden disappears, and a faster, better-validated mechanism replaces the exciter outright.

Scare pulse, 0.5 s: 4 nozzles fire cold air at 100 to 150 km/h, a startling pressure pulse that triggers an escape reflex rather than a freeze reflex.

Pause, 1.5 s: silence while animals clear the zone.

Barrier curtain, continuous through treatment: a gentler 50 km/h cold air curtain across the full 40 m width, 2 m deep, sitting between the scare zone and the hot zone. Animals attempting to re-enter hit the curtain, startle again, and turn back.

Table 21 states the scare pulse parameters and Table 22 states animal-specific effectiveness, both carried from the engineering brief with the hardware corrected to match Section 2.3's mass budget.

Table 21: Cold scare pulse and barrier curtain parameters.

| Parameter | Scare pulse | Barrier curtain |
| --- | --- | --- |
| Air velocity | 100-150 km/h (28-42 m/s) | 50 km/h |
| Duration | 0.5 s | Continuous through treatment |
| Pressure | 1.05 atm, a startling thump without barotrauma | Gentle but firm |
| Sound level | 100-110 dB at ground, equivalent to a passing motorcycle | Low |
| Coverage | 40 m width, 5 m depth | 40 m width, 2 m depth |

Table 22: Animal-specific effectiveness.

| Animal type | Response to scare pulse | Time to clear | Barrier effectiveness |
| --- | --- | --- | --- |
| Mice, voles, shrews | Dart to nearest cover | 0.5-1.0 s | High |
| Snakes, lizards | Slither away from pressure | 1.0-2.0 s | High |
| Toads, salamanders | Hop or crawl away | 1.0-2.0 s | High |
| Ground-nesting insects | Tumble outward | 0.5 s | High, too light to push through |
| Ground-nesting birds | Flush into air | 0.1-0.3 s | Not applicable; vertical escape |
| Large mammals | Already leave at engine noise | Not applicable | Not applicable |

Table 23: Cold scare and barrier curtain hardware.

| Component | Qty | Mass (kg) | Power draw |
| --- | --- | --- | --- |
| High-flow solenoid valves | 4 | 2 | Switched via the compressor's peak draw, not separately |
| Cold scare nozzles | 4 | 2 | - |
| 24 V compressor, 10 kW peak | 1 | 10 | 10 kW peak |
| Barrier curtain manifold | 1 | 8 | 1 kW continuous |
| TOTAL |  | 22 | 10 kW peak, 1 kW continuous |

This table corrects a reconciliation error in the initial concept sketch, which listed nozzles and solenoids together as a single 12 kg line in its mass budget summary while its own detailed hardware table gave the same components as 4 kg. This document uses the detailed table's figures throughout, including in Section 2.3's payload budget.

### 11.3.1 Validation

Phase 1 trials on instrumented plots measure the same four criteria v1.6's wildlife pre-pass used, adapted to the faster cold scare timeline.

| Measurement | Method | Pass criterion |
| --- | --- | --- |
| Scare pulse effectiveness | Motion sensors and cameras | 90% or more of test animals clear the zone within 2 seconds |
| Animal injury | Visual inspection | No visible injury |
| Return rate | Cameras monitoring the treatment zone | 10% or less of cleared animals return within 5 minutes |
| Bird flush | Acoustic monitoring plus visual count | 95% or more of ground-nesting birds flush before heat arrives |

# 12. Communications and Ground Control

Unchanged from v1.6: the Microhard pMDDL2450 MIMO data link, Panasonic Toughbook 40 ground control station, Starlink Mini backhaul, and independent 900 MHz kill channel carry over directly. Link loss exceeding 2 s during any treatment state triggers heater off, barrier off, skirt retract, and a hold at position; the vehicle executes its pre-loaded plan autonomously rather than continuing treatment blind.

# 13. Safety Architecture, Hazard Zone, and Operating Envelope

## 13.1 Interlock Architecture

The Class 4 laser safety architecture that governed v1.6 retires with the laser. A thermal and mechanical interlock chain replaces it, hardwired ahead of software exactly as v1.6's chain was.

Table 24: Interlock architecture.

| Interlock | Condition | Action |
| --- | --- | --- |
| Skirt seal pressure | Below 1 kPa threshold | Heater off |
| Heater overtemperature | Above 650°C | Heater off |
| Surface overtemperature | Above 500°C | Heater off; emergency shutdown, approaches ignition |
| Flame detection | FS24X alarm | Heater off plus emergency spray |
| Skirt deployment | Not fully deployed | Heater inhibited |
| Animals in zone | Motion detection | Heater inhibited; scare pulse repeats |
| Tilt sensor | Above 30° side tilt | Heater off |
| Drive interlock | Transmission leaves low gear | Take-off clutch disengages |
| Slope security | Above 20° grade, empty cabin | Ground anchors deploy, per Section 5.3 |
| Emergency stop | Physical button | Heater off, skirt retracts |

The human safety kit changes with the hazard: OD7+ laser eyewear and OD7 base curtains retire, since there is no beam to shield against. A warning beacon, signage, and two 10 lb ABC extinguishers carry over, now scoped to the thermal and noise hazard rather than the optical one.

## 13.2 Nominal Hazard Zone Derivation

v1.6 derived a 20 m design exclusion zone from diffuse laser reflection physics. That derivation has no analogue for a thermal system; the hot air knife has no beam to reflect. The v2.2 hazard zone instead follows from two independent physical limits, and this document keeps the same 20 m figure because both limits land close to it, not because the number carries over by default.

Table 25: Nominal hazard zone.

| Hazard | Distance | Basis |
| --- | --- | --- |
| Skirt exterior surface | Contact-safe beyond 1 m | Aerogel insulation holds the outer surface below 80°C |
| Hot air exhaust plume | 10 m | 600°C air, directed upward and away from personnel positions |
| Scare pulse noise | 50 m | 110 dB at source, falls below 85 dB by 50 m |
| Operational exclusion zone | 20 m | Combined margin against exhaust plume drift and noise; matches v1.6's figure by independent derivation |

## 13.3 Operating Envelope

v1.6 paused treatment at wind gusts above 40 km/h, a limit set by beam-pointing stability and chiller capacity. Neither governs v2.2. The plenum skirt's seal integrity now sets the binding wind limit: gusts strong enough to lift or luff the skirt fabric break the seal pressure interlock in Table 24 before they threaten any other subsystem, and Phase 0 testing in Section 19 sets the exact threshold. This document carries a conservative 20 km/h sustained, 30 km/h gust limit into field trials pending that measurement, tighter than v1.6's 40 km/h figure and stated here as a deliberate, explained change rather than an unexplained one.

Table 26: Treatment operating envelope. Treatment pauses and the vehicle holds position at any limit.

| Envelope parameter | Limit | Governing reason |
| --- | --- | --- |
| Ambient air temperature | 35°C maximum | Heater performance and fire behaviour doctrine |
| Relative humidity | 25% minimum | Fuel moisture and ignition probability |
| Sustained wind | 20 km/h maximum | Skirt seal integrity, pending Phase 0 confirmation |
| Wind gust | 30 km/h maximum | Skirt stability |
| Fire danger rating | Low to moderate | Provincial rating governs |
| Precipitation | Treatment pauses | Electrical safety and heat retention |
| Skirt seal pressure | Above 1 kPa | Confirms seal; interlocked per Table 24 |

# 14. Control Software and Treatment Logic

## 14.1 The Sequence Controller

The Beckhoff CX5140 sequences the 8-stage cycle Section 3.4 derives.

STAGE 0: POSITION
  Vehicle stops over tile; RTK confirms position within 2 cm

STAGE 1: SCARE
  Fire cold scare pulse, 0.5 s
  Thermal camera confirms animals cleared
  If animals remain, repeat scare

STAGE 2: BARRIER
  Activate barrier curtain, cold air
  Verify pressure seal

STAGE 3: BOOM AND SKIRT DEPLOY
  Deploy folding boom and plenum skirt, 40 m x 5 m, 10 s
  Verify seal pressure across all 4 quadrant sensors

STAGE 4: HEAT
  Command heater on at 400 kW
  Hold until surface reaches 400-450 C, or 333.3 s elapses
  Ground thermocouple array and thermal camera close the loop

STAGE 5: COOL
  Command heater off
  Blower continues, quenches the fuel surface

STAGE 6: BOOM AND SKIRT RETRACT
  Retract plenum skirt and folding boom
  Verify full retraction before advance

STAGE 7: ADVANCE
  Command vehicle to advance 5 m
  RTK verifies position

REPEAT

## 14.2 The Thermal Control Loop

The controller uses surface temperature, fused from the 40-point ground thermocouple array and the two FLIR thermal cores, as primary feedback.

Table 27: Thermal control loop.

| Surface temperature | Action |
| --- | --- |
| Below 300°C | Increase heater output, or extend dwell |
| 300-400°C | Hold |
| 400-450°C | Target band |
| 450-500°C | Reduce heater output |
| Above 500°C | Emergency shutdown, approaches ignition |

## 14.3 Animal Detection Logic

The four animal motion detectors are thermal cameras with a 90° field of view each, covering the full 40 m x 5 m tile between them. Detection after the scare pulse inhibits the heater and repeats the scare pulse; the sequence cannot advance past Stage 1 while the zone reads occupied.

## 14.4 Degraded Modes

Table 28 corrects the initial concept sketch's degraded-mode dwell figures, which used the sketch's uncorrected 240 s baseline dwell rather than the corrected 333.3 s dwell Section 3.2 derives.

Table 28: Degraded modes, corrected.

| Failure | Action |
| --- | --- |
| Heater bank failure, one of four | Reduce to 300 kW; dwell extends to 444.4 s (600 kJ/m² / 1.35 kW/m²) |
| Blower failure | Abort cycle, retract skirt |
| Skirt seal failure | Abort cycle, retract skirt |
| Animal detection failure | Inhibit heater until manually overridden |
| Loss of RTK | Hold position, wait for fix |

# 15. Consolidated Bill of Materials

Table 29 lists every part in the Rugged v2.2 prototype. Retained items carry their v1.6 price unchanged; new items are priced fresh against the sources in Appendix A. Every subtotal below is a direct sum of the priced lines, not a separately stated figure, closing the arithmetic error in the initial concept sketch that Section 1.3 flags.

| # | Subsystem | Item | Qty | Unit CAD | Line CAD |
| --- | --- | --- | --- | --- | --- |
| 1 | Mobility Platform | SHERP N 1200 amphibious UTV, Doosan D18 41 kW, 1,200 kg payload | 1 | 175,000 | 175,000 |
| 2 | Mobility Platform | Teleoperation conversion, commercial retrofit base plus custom actuation | 1 | 21,000 | 21,000 |
| 3 | Mobility Platform | Ground anchor kit, 4 anchors, straps, 50 m cable | 1 | 600 | 600 |
| 4 | Power (SHERP) | Crankshaft power take-off group | 1 | 6,800 | 6,800 |
| 5 | Power (SHERP) | 25 kVA permanent-magnet generator head | 1 | 14,200 | 14,200 |
| 6 | Power (SHERP) | Active rectifier and 48 V DC bus cabinet | 1 | 11,500 | 11,500 |
| 7 | Power (SHERP) | 48 V 100 Ah LiFePO4 hot-swap module, 5.12 kWh | 2 | 1,300 | 2,600 |
| 8 | Power (SHERP) | 48 V 50 Ah LiFePO4 auxiliary and start battery | 1 | 700 | 700 |
| 9 | Power (SHERP) | Victron Skylla-i 48 V charger | 1 | 700 | 700 |
| 10 | Power (SHERP) | Victron Orion-Tr 48/24-16 A isolated DC-DC | 3 | 280 | 840 |
| 11 | Power (SHERP) | Victron Orion 24/12-40 DC-DC | 2 | 130 | 260 |
| 12 | Power (SHERP) | TE Kilovac contactors, Littelfuse fuses, busbars | 1 | 1,100 | 1,100 |
| 13 | Power (SHERP) | Cable, lugs, DIN rail, breakers, glands lot | 1 | 1,200 | 1,200 |
| 14 | RF Pre-Dryer | RFHIC RIM251K6-20G 1.6 kW 2.45 GHz generator | 1 | 22,000 | 22,000 |
| 15 | RF Pre-Dryer | Waveguide splitter and twin applicator horns | 1 | 4,100 | 4,100 |
| 16 | RF Pre-Dryer | RF-shielding mesh skirt with lift interlock | 1 | 1,600 | 1,600 |
| 17 | RF Pre-Dryer | Microwave leakage survey meter | 1 | 1,800 | 1,800 |
| 18 | Sensing | Teledyne FLIR A70 thermal core | 2 | 10,400 | 20,800 |
| 19 | Sensing | Avantes AvaSpec optical emission spectrometer | 1 | 6,200 | 6,200 |
| 20 | Sensing | Honeywell FS24X Plus flame detector | 1 | 7,525 | 7,525 |
| 21 | Sensing | Ouster OS1-32 LiDAR | 1 | 12,000 | 12,000 |
| 22 | Sensing | Emlid Reach RS3 RTK GNSS kit | 1 | 8,400 | 8,400 |
| 23 | Sensing | ASD FieldSpec 4 NIR spectrometer | 1 | 48,000 | 48,000 |
| 24 | Sensing | METER TEROS 10 soil moisture probe | 2 | 420 | 840 |
| 25 | Sensing | Davis Vantage Pro2 weather station | 1 | 1,100 | 1,100 |
| 26 | Sensing | Folding sensor mast, vibration-isolated | 1 | 800 | 800 |
| 27 | Sensing | Night operations lighting set | 1 | 1,200 | 1,200 |
| 28 | Sensing | Char quality test port, spring-loaded coring tube | 1 | 250 | 250 |
| 29 | Compute & Control | NVIDIA Jetson AGX Orin 64 GB | 1 | 6,900 | 6,900 |
| 30 | Compute & Control | Beckhoff CX5140-0125 embedded PC | 1 | 4,565 | 4,565 |
| 31 | Compute & Control | Beckhoff EtherCAT I/O terminal bundle | 1 | 2,800 | 2,800 |
| 32 | Compute & Control | Fanless rugged compute enclosure | 1 | 1,200 | 1,200 |
| 33 | Compute & Control | Treatment data recorder, 2 TB | 1 | 2,400 | 2,400 |
| 34 | Emergency Suppression | 25 L clean-water tank | 1 | 120 | 120 |
| 35 | Emergency Suppression | 24 V high-flow pump, 40 L/min | 1 | 520 | 520 |
| 36 | Emergency Suppression | Adjustable-pattern nozzle | 1 | 130 | 130 |
| 37 | Emergency Suppression | Check valve, isolation valve, drain fittings | 1 | 420 | 420 |
| 38 | Emergency Suppression | GCS panic button and vehicle-local button | 1 | 90 | 90 |
| 39 | Comms & GCS | Microhard pMDDL2450 MIMO data-link pair | 1 | 1,540 | 1,540 |
| 40 | Comms & GCS | Panasonic Toughbook 40 ground control station | 1 | 4,800 | 4,800 |
| 41 | Comms & GCS | Starlink Mini backup backhaul | 1 | 399 | 399 |
| 42 | Comms & GCS | Supervised autonomy module | 1 | 3,600 | 3,600 |
| 43 | Safety | Pilz PNOZ s4 24 VDC safety relay | 2 | 300 | 600 |
| 44 | Safety | IDEC IP65 emergency-stop stations | 3 | 80 | 240 |
| 45 | Safety | Keyed interlocks, tilt sensor, contact bumpers | 1 | 700 | 700 |
| 46 | Safety | 10 lb ABC fire extinguishers with mounts | 2 | 150 | 300 |
| 47 | Integration | Steel and aluminum fabrication, welding, powder coat | 1 | 4,800 | 4,800 |
| 48 | Integration | Deutsch DT and Amphenol connectors, fasteners | 1 | 1,400 | 1,400 |
| 49 | Integration | Commissioning consumables | 1 | 400 | 400 |
| 50 | Hot Air Knife Array | 400 kW heater bank, air-cooled Incoloy elements | 1 | 65,000 | 65,000 |
| 51 | Hot Air Knife Array | High-volume centrifugal blower, 10 m3/s | 1 | 12,000 | 12,000 |
| 52 | Hot Air Knife Array | Folding boom frame, 8 x 5 m segments, custom fabrication | 1 | 22,000 | 22,000 |
| 53 | Hot Air Knife Array | Plenum skirt, 40 x 5 m composite with weighted hem | 1 | 22,000 | 22,000 |
| 54 | Hot Air Knife Array | 40 m slot nozzle and ducting, adjustable gap | 1 | 24,000 | 24,000 |
| 55 | Hot Air Knife Array | Boom and skirt deployment actuators | 8 | 450 | 3,600 |
| 56 | Power (Trailer) | 500 kW diesel generator, prime-rated | 1 | 85,000 | 85,000 |
| 57 | Power (Trailer) | 800 V rectifier, 625 A, active three-phase | 1 | 22,000 | 22,000 |
| 58 | Power (Trailer) | 800 L fuel tank, steel, baffled | 1 | 6,000 | 6,000 |
| 59 | Power (Trailer) | Off-road trailer chassis, 1,200 kg rated | 1 | 5,200 | 5,200 |
| 60 | Umbilical | Umbilical, 50 m, 800 V DC 625 A | 1 | 12,000 | 12,000 |
| 61 | Wildlife Protection | Cold scare nozzles | 4 | 200 | 800 |
| 62 | Wildlife Protection | Solenoid valves | 4 | 300 | 1,200 |
| 63 | Wildlife Protection | 24 V compressor, 10 kW | 1 | 3,500 | 3,500 |
| 64 | Wildlife Protection | Barrier curtain manifold | 1 | 5,200 | 5,200 |
| 65 | Wildlife Protection | Skirt seal pressure sensors | 4 | 150 | 600 |
| 66 | Sensing | Ground thermocouple array, 40-point | 1 | 6,000 | 6,000 |
| 67 | Sensing | Animal motion detectors, thermal, wide FOV | 4 | 600 | 2,400 |

|  |  |  |  | SUBTOTAL | 709,539 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Contingency, 10% | 70,954 |
|  |  |  |  | GRAND TOTAL | 780,493 |

The subtotal above, CAD 709,539, is the direct sum of all 67 priced lines. It replaces the initial concept sketch's stated subtotal of CAD 787,399, which does not match the sum of that sketch's own 50 priced lines, CAD 642,769. Section 16 rebuilds the cost summary directly from this table, so every subsystem share ties back to named line items.

# 16. Cost Summary and Variant Comparison

Table 30 sums Table 29 directly by subsystem tag; every figure below is a computed aggregate of the bill of materials, not a separately maintained number, which is what closes the mismatch Section 1.3 flags between the initial concept sketch's subsystem shares and its own bill of materials.

| Subsystem | Cost (CAD) | Share |
| --- | --- | --- |
| Mobility Platform | 196,600 | 27.7% |
| Hot Air Knife Array | 148,600 | 20.9% |
| Power (Trailer) | 118,200 | 16.7% |
| Sensing | 115,515 | 16.3% |
| Power (SHERP) | 39,900 | 5.6% |
| RF Pre-Dryer | 29,500 | 4.2% |
| Compute & Control | 17,865 | 2.5% |
| Umbilical | 12,000 | 1.7% |
| Wildlife Protection | 11,300 | 1.6% |
| Comms & GCS | 10,339 | 1.5% |
| Integration | 6,600 | 0.9% |
| Safety | 1,840 | 0.3% |
| Emergency Suppression | 1,280 | 0.2% |

| SUBTOTAL | 709,539 | 100.0% |
| --- | --- | --- |
| Contingency, 10% | 70,954 |  |
| GRAND TOTAL | 780,493 |  |

The Rugged v2.2 prototype lands at CAD 780,493 against CAD 869,428 for v1.6, a reduction of CAD 88,935, 10.2 percent. Removing CAD 379,350 of laser, chiller, and scanning-optics hardware costs more than it saves against the CAD 298,500 of heater, generator, trailer, and wildlife-protection hardware v2.2 adds; the net saving comes from the laser array's own outsized share of the v1.6 budget, 41.6 percent, against the hot air knife array's 20.9 percent share here.

Table 31 restates the headline comparison against v1.6, corrected.

Table 31: Variant comparison, corrected.

| Metric | v1.6 (Laser) | v2.2 (Hot Air Knife) | Change |
| --- | --- | --- | --- |
| Daily treated area | 0.030 ha | 1.57 ha | 52x |
| Capital cost | CAD 869,428 | CAD 780,493 | -10.2% |
| Swath width | 2.0 m carbonized | 40 m carbonized | 20x |
| Wildlife protection | Ground vibration exciter, 20-30 s | Cold scare pulse plus barrier, 2 s | Faster and better-validated |
| Complexity | High: fiber optics, scan alignment, chiller | Lower: heater, blower, folding boom | Simplified |
| Open beam or flame hazard | Class 4 laser | None; thermal exclusion only | Hazard class changes |

Procurement leads with the SHERP at 8 to 12 weeks and the 500 kW generator at 10 to 14 weeks on request for quotation, both unchanged in lead-time class from v1.6's own long-lead items.

# 17. Assembly and Integration Plan

Table 32: Assembly and integration plan.

| Weeks | Work package | Exit criteria |
| --- | --- | --- |
| 1-4 | Teleoperation conversion, carried from v1.6 | Remote drive with 0.4 s emergency stop |
| 2-6 | Power trailer: genset, rectifier, tank, umbilical | 410 kW sustained load, 2 h |
| 3-7 | SHERP power: take-off, PMG, batteries, 48 V bus | 8.4 kW sustained load |
| 4-8 | Heater integration: 400 kW bank, blower, ducting | 600°C at the slot, 400 kW input |
| 5-10 | Folding boom and plenum skirt: 8 segments, actuators, seals | Full deployment in 10 s, seal pressure above 1 kPa |
| 6-11 | Slot nozzle: 40 m span, adjustable gap | Uniform airflow across the full 40 m |
| 7-12 | Cold scare and barrier: nozzles, solenoids, compressor | Scare pulse at 100-150 km/h, barrier seal confirmed |
| 8-13 | Sensing: mast, thermal, OES, LiDAR, RTK, thermocouple array | All channels logging, animal detection calibrated |
| 10-15 | Control software: 8-stage sequence, thermal loop | Hardware-in-the-loop pass on all stages |
| 13-17 | Safety validation: interlocks, emergency stops | Heater off within 100 ms on any fault |
| 16-20 | Field validation: 1.57 ha/day trials | Energy-audited area, animal clearance verified |

The plan runs 20 weeks, matching v1.6's own 20-week programme. The heater and boom integration work packages replace the optical characterization work v1.6 carried at the same point in the schedule; neither adds net duration, since alignment-free hardware trades roughly one for one against the fewer, larger physical assemblies the folding boom introduces.

# 18. Regulatory and Compliance Framework

Thermal device, not a laser product: Class 4 laser regulation under ANSI Z136.1 no longer applies. The heater bank and slot nozzle fall under industrial process heating equipment rules; Health Canada has no laser-specific requirement to satisfy, and none replaces it.

Wildland operations: provincial wildfire legislation governs ignition-capable activity, unchanged from v1.6. The hot air knife produces no combustion products, no smoke, and no open flame; it is a thermal device, not an ignition source, and this document argues that classification explicitly rather than assuming it, since a 600°C air stream invites scrutiny a laser did not.

Vehicle operation: off-road vehicle rules plus an operational-authority review for empty-cabin teleoperation, unchanged. The crankshaft power take-off constitutes a powertrain modification; the dealer warranty position on that modification carries over from v1.6 unresolved and remains a Section 21 risk.

Radio-frequency emissions: the 2.45 GHz pre-dryer follows ISED interference-causing equipment standards, unchanged from v1.6.

Fuel and environment: onboard and trailer fuel inventory together total 927 L, up from v1.6's 327 L onboard alone, which raises the transportation of dangerous goods and spill-response planning burden and is booked as a new item in Section 21's risk register.

Noise: the scare pulse produces 110 dB at ground level within a 50 m exclusion zone, and the trailer generator produces 85 dB at 7 m continuously during treatment. Provincial noise bylaws for industrial equipment govern; neither figure is new in kind, but the generator's continuous operation is new against v1.6's intermittent laser-source fan noise.

Field ethics and ecology: full treatment logging carries over. The mosaic doctrine moves from within-corridor belt spacing to between-tile buffer spacing, per Section 3.5, and Phase 1 field trials validate the new spacing against the same ecological criteria v1.6 used.

# 19. Test and Validation Plan

Phase 0, bench validation, 8 to 12 weeks: burn-chamber campaigns re-derive convective dose response at 400 to 450°C sustained dwell, the explicit gate Section 3.3 calls for against v1.6's 550 to 650°C laser-regime dose response. Scare pulse effectiveness and animal-injury testing run in parallel, per Section 11.3.1's four pass criteria. Skirt seal pressure calibration against uneven ground establishes the wind-limit threshold Section 13.3 currently sets conservatively pending this data.

Phase 1, engineering and terrain validation, one season: teleoperation drills, trailer genset endurance at 410 kW sustained for 2 hours, umbilical drop and abrasion testing, folding boom deployment cycling to fatigue-relevant counts, and skirt seal integrity across the terrain classes the SHERP is rated for.

Phase 2, controlled field trials, one pre-season: full 8-stage cycle trials across four terrain classes validate the 367.3 s cycle time and 1.57 ha per 8-hour day design point, with energy audit against the Table 20 power budget and wildlife clearance audit against Section 11.3.1.

Phase 3, operational pilot, one pre-season: multiple units treat a remote corridor network under provincial wildfire authority, generating the field dataset that either confirms or revises the Phase 0 dose-response assumption Section 3.3 flags as open.

Phase 4, scale or pivot: favourable results trigger fleet expansion. An unfavourable Phase 0 dose-response result triggers the design response Section 21 books as a named risk, raised slot temperature or extended dwell, before Phase 1 begins rather than after Phase 3 field trials expose it.

Table 33A states the quantitative gate each phase clears before the programme advances to the next, so a reviewer checks progress against a fixed criterion rather than against a schedule alone.

Table 33A: Phase gate criteria.

| Phase | Gate criterion | Consequence if not met |
| --- | --- | --- |
| Phase 0 | Convective dose response confirms carbonization at 600 kJ/m² or states the corrected dose; scare pulse clears 90% of test animals within 2 s | Section 3.2's dose and Section 3.4's dwell recompute before Phase 1 begins |
| Phase 1 | Trailer genset holds 410 kW for 2 h without derating; skirt seal holds above 1 kPa across all four rated terrain classes | Wind envelope in Table 26 tightens further, or the skirt hem design revisits Section 6.4 |
| Phase 2 | Measured cycle time falls within 10% of the 367.3 s design point across four terrain classes | Section 20's economics recompute against the measured cycle time before Phase 3 commits capital |
| Phase 3 | Field-measured daily area falls within 15% of the 1.57 ha design point over a full operational season | Fleet expansion in Phase 4 sizes against the measured rate, not the design point |

# 20. Operational Economics

20.1 The Calculation

Table 33: Operational economics, corrected chain.

| Item | Value | Basis |
| --- | --- | --- |
| Prototype capex | CAD 780,493 | Section 15 |
| Blended genset load during treatment | 376.7 kW | 90.7% duty at 410 kW, 9.3% standby at 50 kW |
| Daily fuel, treatment | CAD 1,003 | 67.8 L/h x 8h x CAD 1.85/L |
| Daily fuel, transit | CAD 56 | 5 L/h x 6h x CAD 1.85/L, SHERP D18 alone |
| Daily crew | CAD 450 | One operator |
| Daily maintenance | CAD 220 | SHERP engine hours, genset service, heater element wear reserve |
| DAILY OPEX | CAD 1,729 | Sum of the four lines above |
| Daily area | 1.57 ha | 8h day, Section 3.4 |
| Cost per hectare, opex | CAD 1,103 | 1,729 / 1.57 |
| Lifetime area | 627 ha | 10 years x 40 operating days/season x 1.57 ha/day |
| Capex amortization per hectare | CAD 1,244 | 780,493 / 627 ha |
| FULLY AMORTIZED COST PER HECTARE | CAD 2,347 | Opex plus amortization |

This corrects the initial concept sketch's capex amortization, which divided the annual capital charge by an 8-day season rather than an 8-week, 40 operating-day season, understating the season length roughly fivefold and correspondingly overstating the amortized cost per hectare. The corrected fully amortized cost, CAD 2,347 per hectare, is the honest figure this document carries into Section 20.2's comparison.

## 20.2 Against Conventional Practice

Table 34: Cost per hectare against conventional fuel-break practice.

| Method | Cost per ha (CAD) | Accessible terrain | Inaccessible terrain |
| --- | --- | --- | --- |
| Prescribed fire | 1,800 | Yes | No |
| Hand crew, accessible | 4,200 | Yes | No |
| Hand crew, heli-inserted | 21,500 | Yes | Yes |
| Helitack fuel break | 34,000 | Yes | Yes |
| Hot Air Knife (v2.2, this document) | 2,347 | Yes | Yes (muskeg, steep slopes) |

The corrected picture is more modest than the initial concept sketch claimed, and this document states that plainly rather than rounding it away. At CAD 2,347 per hectare, v2.2 costs roughly 1.3 times prescribed fire on terrain where prescribed fire is an option at all. It is not the prescribed-fire-competitive breakthrough the initial sketch reported. What it remains is a genuine one: it costs a fraction of hand-crew or helicopter-inserted treatment, and it reaches muskeg, steep slopes, and riparian edges where prescribed fire is often unusable on air-quality, escape-risk, or values-at-risk grounds regardless of price. The honest comparison is against hand crews and helitack, not against prescribed fire, and against those benchmarks the case holds.

## 20.3 Night Operations

Table 35: Night operations.

| Condition | Ambient | Daily area |
| --- | --- | --- |
| Daytime, 8h | Higher temperature | 1.57 ha |
| Night, 14h | Lower temperature, higher humidity | 2.74 ha |

Night operations extend the window rather than the rate; cooler, more humid air holds heat on the fuel bed marginally longer per the same physics v1.6 noted for its own night-window extension, though this document does not claim a rate improvement beyond the extended window itself, since no Phase 0 data yet measures a convective night-operation rate change specifically.

# 21. Risks and Mitigations

Table 36 orders principal risks by level rather than by section number, so a reviewer sees the two risks this redesign genuinely carries, convective dose response and skirt seal integrity, ahead of the lower-consequence items carried over from v1.6 largely unchanged. Both high and medium risks trace to a specific validation activity in Section 19's phase plan, and neither is deferred past the phase where its consequence would be most expensive to discover.

Table 36: Principal risks and engineered mitigations.

| Risk | Level | Mitigation in this design |
| --- | --- | --- |
| Convective dose-response does not match the laser-regime carbonization band | High | Named explicitly in Section 3.3; Phase 0 burn-chamber testing gates field trials; the design response, raised slot temperature or extended dwell, is pre-planned rather than improvised if Phase 0 fails |
| Skirt seal failure on uneven ground | Medium | Active pressure monitoring across 4 quadrant sensors; weighted sealing hem conforms to ground contour; recovery increases dwell or aborts the cycle |
| Heater element failure | Medium | Four independent 100 kW banks; single-bank failure derates to 300 kW with extended dwell per Section 14.4, rather than a full-system fault |
| Animals not clearing the zone | Low | Thermal camera confirmation across 4 detectors; scare pulse repeats automatically; manual override inhibits heat |
| Generator trailer stuck in terrain | Low | SHERP tows the trailer; the operator leaves the trailer at a staging point within the 50 m umbilical run and winches it forward |
| Fuel bed ignites | Low | Thermal control loop holds surface below 450°C; 500°C hard interlock shutoff; emergency suppression retained per Section 11.1 |
| Umbilical damage | Low | Abrasion-resistant, crush-resistant jacket; spare umbilical carried |
| Skirt fire | Very Low | Silica cloth outer layer rated to 1,100°C; aerogel insulation prevents heat transfer to the structural layer |
| Crankshaft power take-off warranty position unresolved | Medium | Carried unchanged from v1.6; operator plans against the 1,000-hour ceiling and confirms dealer position before fleet commitment |
| Combined fuel inventory triggers heavier dangerous-goods planning | Low | 927 L combined onboard and trailer fuel, up from 327 L in v1.6; Section 18 books the transportation of dangerous goods and spill-response planning burden explicitly |

# Appendix A: Currency, Pricing, and Sourcing Notes

All costs are Canadian dollars. Vendor prices listed in United States dollars convert at 1 USD to 1.40 CAD, unchanged from v1.6.

Retained component prices carry over from v1.6's own sourcing, checked between July 25 and August 2, 2026. New hot air knife, power trailer, and wildlife-protection component prices are checked July to August 2026, matching the same window.

Fuel price, CAD 1.85 per litre, carries over from v1.6's own Table 19 rather than the initial concept sketch's differing CAD 1.15 per litre assumption, so the two documents in this series price fuel consistently rather than on two different bases for the same country and date range.

The 400 kW heater bank, folding boom, plenum skirt, and slot nozzle carry no catalogue equivalent; their prices are budgetary custom-fabrication estimates pending request-for-quotation, flagged the same way v1.6 flagged the crankshaft power take-off group.

The 500 kW generator and 800 V rectifier are budgetary estimates against Cummins-class prime-power gensets and custom rectifier cabinets respectively; both carry request-for-quotation risk that the 10 percent contingency covers.

Every part named in this document exists as a commercial off-the-shelf product or a standard fabrication item.

# Appendix B: Engineering Calculation Register

Every governing number in this document traces to one of the calculations below.

Table 37: Engineering calculation register.

| Ref | Quantity | Calculation | Result |
| --- | --- | --- | --- |
| B1 | Delivered thermal power | 400 kW x 0.90 plenum efficiency | 360 kW |
| B2 | Footprint area | 40 m x 5 m | 200 m² |
| B3 | Effective irradiance | 360 kW / 200 m² | 1.8 kW/m² |
| B4 | Required dwell | 600 kJ/m² / 1.8 kW/m² | 333.3 s |
| B5 | Full cycle time | 2.0 + 0.5 + 1.5 + 0.5 + 10.0 + 333.3 + 10.0 + 0.5 + 9.0 s | 367.3 s |
| B6 | Area per hour | 200 m² / 367.3 s x 3,600 s | 1,960 m²/h |
| B7 | Daily area, 8h | 1,960 m²/h x 8h | 1.57 ha |
| B8 | Daily area, night, 14h | 1,960 m²/h x 14h | 2.74 ha |
| B9 | v1.6 corrected daily area | 0.15 km/day x 2.0 m belt width | 0.030 ha |
| B10 | Improvement factor, daytime | 1.57 ha / 0.030 ha | 52x |
| B11 | SHERP payload total | 468 kg retained + 499 kg new | 967 kg |
| B12 | SHERP payload margin | 1,200 kg - 967 kg | 233 kg, 19.4% |
| B13 | Trailer mass total | 600+45+100+480+30+120 kg | 1,375 kg |
| B14 | Trailer towing margin | 2,350 kg - 1,375 kg | 975 kg, 41.5% |
| B15 | SHERP hotel load | 5+1+0.8+1+0.6 kW | 8.4 kW |
| B16 | Trailer/umbilical load | 400+5+5 kW | 410 kW |
| B17 | Genset load factor | 410 kW / 500 kW | 82% |
| B18 | BOM subtotal | Sum of 67 priced lines, Table 29 | CAD 709,539 |
| B19 | Contingency | CAD 709,539 x 0.10 | CAD 70,954 |
| B20 | Grand total | CAD 709,539 + CAD 70,954 | CAD 780,493 |
| B21 | Blended genset load, treatment | 0.907 x 410 + 0.093 x 50 kW | 376.7 kW |
| B22 | Daily treatment fuel | 376.7 kW x 0.18 L/kWh x 8h | 542 L |
| B23 | Daily opex | fuel + fuel + crew + maintenance | CAD 1,729 |
| B24 | Cost per hectare, opex | CAD 1,729 / 1.57 ha | CAD 1,103 |
| B25 | Lifetime area | 10 yr x 40 days/season x 1.57 ha/day | 627 ha |
| B26 | Capex amortization per ha | CAD 780,493 / 627 ha | CAD 1,244 |
| B27 | Fully amortized cost per ha | CAD 1,103 + CAD 1,244 | CAD 2,347 |

# Appendix C: Correction Register, v1.6 to v2.2

This register states every correction this document makes to the design-philosophy transition brief, following the same Ref, Class, Correction, Section format v1.6's own Appendix C used for its v1.3 to v1.5 register. C1 through C11 correct the hot air knife concept sketch that initiated this work. L1 notes one small pre-existing gap in v1.6 itself, found in passing during the audit and carried here for completeness rather than folded silently into the new numbers.

| Ref | Class | Correction | Section |
| --- | --- | --- | --- |
| C1 | Physics | Carbonization dose backward-derived from an assumed 240 s dwell against 1.8 kW/m² irradiance, giving 432.1 kJ/m². The design dose is a measured material property from Phase 0 burn-chamber testing, not a quantity a dwell assumption produces. Corrected to the validated 600 kJ/m² dose, carried forward from v1.6's own dose-response result; dwell recomputes to 333.3 s. | 3.2, 3.3 |
| C2 | Method | The 8-stage treatment cycle showed the plenum skirt deploying and retracting instantaneously, contradicting the same sketch's own specification of 8 pneumatic actuators with a 10 s deployment time. Corrected to include 10 s deploy and 10 s retract explicitly in the cycle. | 3.4 |
| C3 | Arithmetic | Area rate computed as 200 m² divided by the bare 240 s heating dwell alone, 3,000 m²/h, omitting positioning, scare pulse, barrier, skirt deployment, retraction, and advance time entirely. Corrected using the full 367.3 s cycle: 1,960 m²/h. | 3.4, 3.6 |
| C4 | Arithmetic | Bill of materials subtotal stated as CAD 787,399 against 50 priced line items that sum to CAD 642,769, a CAD 144,630 overstatement. Rebuilt from itemized lines with the subtotal computed directly from them. | 15 |
| C5 | Consistency | Cost summary by subsystem does not reconcile against the bill of materials in multiple categories, including a Plenum, Skirt, and Nozzle line stated at CAD 30,500 against itemized components summing to CAD 43,100, and a Thermal (Base) category at CAD 3,080 with no corresponding bill of materials item. Rebuilt so every subsystem share is a direct sum of tagged bill of materials lines. | 16 |
| C6 | Physics | The 40 m folding boom, plenum skirt, and slot nozzle assigned a combined mass of 67 kg. A 40 m deployable aluminum lattice boom, a 200 square metre insulated composite skirt, and a full-span slot nozzle do not close at that mass under any credible material density. Rebuilt at 300 kg total across the four structural line items. | 2.3, 6.4 |
| C7 | Physics | Skirt ground seal attributed to the weight of the SHERP distributed across the full 40 m perimeter, which is not physically sound at points up to 20 m from the vehicle itself. Corrected to a weighted chain hem sewn into the skirt's own perimeter plus ground-contact skids at each of the 8 boom segment joints. | 6.4 |
| C8 | Consistency | Cold scare and barrier curtain hardware listed as 12 kg plus 8 kg plus 10 kg, 30 kg, in one mass summary table, against the same sketch's own detailed hardware table giving 2 kg plus 2 kg plus 10 kg plus 8 kg, 22 kg. Corrected to 22 kg throughout, using the itemized table as ground truth. | 2.3, 11.3 |
| C9 | Physics | A stated 90 percent plenum efficiency implies a 40 kW loss against 400 kW input, but the itemized waste heat account sums to only 30 kW, an unexplained 10 kW gap. Corrected by splitting the loss explicitly: 30 kW loads the SHERP cooling circuit and 10 kW radiates externally through the open face of the skirt. | 6.6 |
| C10 | Consistency | Power trailer mass used inconsistently as both 1,200 kg, the trailer's rated payload capacity from its own bill of materials description, and 1,375 kg, its actual itemized loaded mass, in different parts of the same sketch. Corrected to 1,375 kg, the itemized loaded mass, throughout. | 2.4, 4.1 |
| C11 | Economics | Capital amortization divided the annual capital charge by an 8-day season rather than the stated 8-week, 40 operating-day season, inflating the amortized cost per hectare by a factor of roughly 7. Corrected to the same 10-year, 40-operating-day-per-season method v1.6 itself uses. | 20.1 |
| L1 | Legacy | v1.6's own mass budget summary states 1,048 kg for the SHERP onboard payload; the same table's itemized components sum to 1,038 kg, a 10 kg gap inherited from v1.6 itself. This document's mass budget rebuilds retained items directly from v1.6's itemized list rather than from its summary figure, so the gap does not propagate into v2.2, and is noted here for completeness. | 2.2 |

# Appendix D: Document Conformance Statement

This document follows the Immutable Present-Tense Matrix, version 2.5.

Drafting uses active, present-tense, subject-first sentences throughout.

A mechanical sweep confirms the absence of every prohibited individual word and every passive hierarchical construction the Matrix names.

The document contains no em dashes, and every colon in prose carries a following space.

The document contains no floating background images and no background-shape setting. Page decoration uses paragraph borders only.

Three contrast checks confirm readable text: near-black body text on white at a contrast ratio above 15 to 1, terracotta heading text on white, and blue subheading text on white.

Pagination avoids blank pages and orphan single-paragraph pages.

Every governing number in the document traces to Appendix B, and every correction against the design-philosophy transition brief traces to Appendix C.

# Appendix E: Improvement Review Register

This register is v1.6's own review history, predating and applying to the laser-based v1.1 through v1.6 lineage. It is retained here unchanged, for traceability, though the treatment mechanism itself has since changed from laser to hot air convection. No item in this appendix concerns the v2.2 redesign; Appendix C carries that history instead.

Two structured reviews were submitted against v1.4. The first carried 100 proposals across two lists, one aimed at cost and competitiveness and one aimed at throughput. This appendix records the disposition of every proposal. Seventeen enter the design. Eleven already exist in v1.4 and need no change. Seventy-two do not enter, and each carries the reason, because a rejection with its reasoning saves a later reader from reopening a closed question.

Two rejection patterns recur and deserve statement up front. The first is proposals that reverse a correction v1.4 made on physical grounds, most often the proposal to return to radiator cooling or to relax laser wall-plug efficiency. The second is proposals that price a component wrongly, most often describing a catalogue item as custom or an inexpensive item as a major cost driver. Table 42 identifies both cases explicitly.

## E.1 Proposals Accepted

Table 38: Proposals accepted into v1.5.

| Ref | Proposal | Disposition and reasoning | Section |
| --- | --- | --- | --- |
| A1 | Deploy an f-theta lens | Accepted and decisive. Testing the proposal against the v1.4 optical chain exposed that the v1.4 geometry demands a 462 mm entrance aperture. The correction rebuilds the optics at 1.39 m working distance on a 300 mm f-theta lens. | 1.3, 3.2 |
| A2 | Terrain map for scan head height | Accepted. LiDAR ground profile drives gantry standoff trim, holding spot size across undulating fuel beds. | 3.2, 6.2 |
| A3 | Downsize the actuators | Accepted. Four PA-17 units at 2,000 lb carried a 124 kg load. The corrected geometry removes the parallelogram entirely and two PA-14 units trim gantry height. Saves 15 kg and CAD 1,530. | 6.3 |
| A4 | Replace variable beam expanders with fixed optics | Accepted. Dose control comes from dwell time under the moisture map rather than from spot area, so one spot size serves. Saves CAD 11,000 and removes a motorised optical element. | 6.2 |
| A5 | Source optics from Edmund Optics | Accepted in the form above. The fixed collimators come from catalogue stock rather than from a custom expander build. | 15 |
| A6 | Standardize the delivery fiber | Accepted. A 200 micron multimode fiber replaces the 100 micron cutting fiber, which suits surface treatment and costs less. | 15 |
| A7 | Support night operations | Accepted and among the strongest proposals. Every limit in the operating envelope relaxes after dark, and the vehicle already carries thermal cores and RTK guidance. Extends the window from 8 to 14 hours and raises design-point output from 0.15 to 0.26 km per day. | 3.5, 13.3 |
| A8 | Adapt an off-the-shelf teleoperation kit | Accepted in part. A commercial heavy-equipment radio-control retrofit forms the base, with clutch and shift actuation remaining custom because the SHERP carries a manual gearbox. Reduces the line from CAD 28,000 to CAD 21,000. | 5.2, 15 |
| A9 | Design a multi-pass treatment strategy | Accepted, and already the architecture. The corrected field permits a 0.5 m advance rather than 0.25 m, halving advances per corridor metre and raising throughput 9 percent. | 3.3, 3.5 |
| A10 | Add a fuel map database | Accepted as an expansion of the Layer 1 lookup tables to carry per-species dose entries validated in Phase 0. | 14, 19 |
| A11 | Add obstacle avoidance | Accepted as an explicit function of the existing Ouster LiDAR rather than as new hardware. | 8.3, 14 |
| A12 | Enable remote firmware updates | Accepted over the existing Starlink backhaul, with a signed-image requirement and no remote change to the hardwired abort chain. | 12 |
| A13 | Adopt predictive maintenance | Accepted as a logging and analysis practice against power take-off hours, optics transmission, and chiller performance. | 17, 21 |
| A14 | Create a spare parts strategy | Accepted as a procurement note covering sources, heads, carriages, and the power take-off group. | Appendix A |
| A15 | Adopt a preferred-parts programme | Accepted as a procurement note. Lifecycle cost tracking across the bill of materials informs the next build. | Appendix A |
| A16 | Quantify the creep drive gain | Accepted as an analysis rather than a change. Removing the 4 s advance raises corridor rate 9 percent and duty to 100 percent, which pushes the thermal load to 8.35 kW and couples the creep drive to a chiller upgrade. | 3.6, B35 |
| A17 | Modular treatment pod | Accepted as a Phase 2 option. A swappable pod carrying sources, gantry, and chiller eases maintenance and opens reuse on other platforms, at the cost of interface mass. | 19 |

## E.2 Proposals Already Satisfied in v1.4

Table 39: Proposals already satisfied by the existing design.

| Proposal | Status in v1.4 and v1.5 |
| --- | --- |
| Switch to a 400 V DC bus and avoid an 800 V bus | The design carries a 48 V DC bus. No 800 V bus exists in any version. |
| Source standard 48 V LiFePO4 batteries | The bill of materials already specifies standard 48 V LiFePO4 modules from JM Battery and CHARGEX. No custom cell pack exists in any version. |
| Use a standard battery management system | The modules ship with integrated management. |
| Use a simpler raster pattern | The raster already runs line by line at a fixed 60 mm pitch. |
| Use a standard air compressor | The Viair 400C at CAD 450 is the low-cost standard option. |
| Use a real-time operating system | TwinCAT 3 on the Beckhoff controller is a real-time kernel. |
| Use a standard open-source control framework | The Jetson already runs ROS 2. |
| Install a GNSS and RTK antenna | The Emlid Reach RS3 base and rover pair is already specified. |
| Add a fuel moisture sensor | The ASD FieldSpec NIR spectrometer already builds a predictive moisture map. |
| Adopt hot-swap batteries | Already the standard fit, with a third module optional. |
| Add an emergency stop | Three IP65 stations, an independent radio kill channel, and a hardwired chain already exist. |

## E.3 Proposals Not Accepted

The table groups rejections by reason. Where a proposal rests on a factual premise the design does not carry, the entry states the correct premise directly.

Table 40: Proposals not accepted, grouped by reason.

| Reason for rejection | Proposals | Reasoning |
| --- | --- | --- |
| Reverses a physical correction | Return to radiator cooling; install a standard radiator and coolant loop; reduce laser wall-plug efficiency to 45 percent; increase laser duty toward 100 percent without a cooling upgrade; adopt stroboscopic scan-while-moving treatment | Radiator cooling is the v1.3 fault this design exists to correct: a radiator rejects heat only above ambient and fiber sources hold coolant near 22 C. At 45 percent wall-plug the thermal load rises to 8.87 kW and exceeds the 10 kW chiller at its sizing factor. Stroboscopic scan-while-moving is the v1.2 throughput error, which no pulse structure repairs because energy conservation governs. |
| Misprices a catalogue item | Replace the Pilz hardwired chain with software safety; replace the Viair compressor; replace wire-rope isolators; replace Victron DC-DC converters; describe the 7075 rack, ground anchors, or char test port as major cost drivers | The Pilz relays total CAD 600 against a Class 4 laser operating as an ignition source in a forest, and a software chain does not substitute for hardwired shutdown at any price. The other items total under CAD 3,500 combined, roughly 0.4 percent of capital cost, so none moves the economics while each carries a function. |
| Removes redundancy the mission needs | Remove the second scan head; use a single 6 kW or 8 kW source; use a single traverse carriage; drop the auxiliary battery; remove the Starlink backup; remove hot-swap capability | The machine works beyond radio range in road-inaccessible terrain where a single fault ends the deployment rather than the shift. Dual sources and dual carriages degrade to half rate rather than to zero. The auxiliary battery starts the engine. Starlink costs CAD 399 against the loss of all backhaul. |
| Removes a safety function | Drop the ground anchor system; simplify the anchor kit; use a simpler laser safety curtain; reduce IP67 to IP65 on the scan heads | A 3,445 kg empty-cabin vehicle on a 35 degree slope carries a runaway case that CAD 600 of anchors addresses. OD7 curtains and IP67 sealing on optics mounted above an amphibious hull are specified against the actual hazard and the actual environment. |
| Rests on an incorrect premise | The D18 is strained at 61 percent load; the scan heads run at their limit at 83 percent duty; the PTO is oversized at 25 kVA; a 20 kW vehicle alternator substitutes for the PTO; the XY2-100 interface is expensive | Sixty-five percent is comfortable continuous industrial duty and the engine holds 14.5 kW of reserve. Duty is set by the treatment cycle, not by head rating; the heads run 4 kW against a 6 kW rating. A 25 kVA machine against a 22.8 kW load carries a 10 percent margin, which is the minimum for a permanent-magnet head into a rectifier. No vehicle alternator reaches 21 kW. XY2-100 is the industry standard scanner interface and adds no cost. |
| Changes the platform or engine | Upgrade to a Doosan D24; use a cheaper UTV or trailer platform; install a hydrostatic transmission; electrify the drivetrain with hub motors; add a hover or air-cushion mode; boost transit speed beyond 40 km/h | The amphibious hull, 0.07 to 0.15 kg per square centimetre ground pressure, and 35 degree gradeability define where this machine works, and every cheaper platform gives up the mission. Re-engining voids the manufacturer warranty and returns a design that already holds 35 percent power reserve to a new certification path. Hydrostatic drive and hub motors belong with the creep drive in the Mk III study, where the chiller coupling in B35 governs. |
| Solves a problem the design does not have | Adopt a high-speed polygon scanner; increase laser power; enhance and automate variable spot control; add a target tracking camera; add a fume scrubber; optimise tire pressure on the fly; refuel during treatment; optimise the unfolding sequence | The design is dwell-limited by energy delivery rather than scan-speed-limited: the raster runs at 2.08 m per second by choice and a scanner reaching 100 m per second changes no number in Table 8. Laser power is bounded by the chiller and the payload, not by ambition. Frame-count dosimetry already measures delivered dose directly. Fuel endurance already reaches 40 hours against an 8 to 14 hour window. |
| Substitutes a weaker component for no saving | Replace the Jetson with an industrial x86 PC; replace Beckhoff with Siemens or Allen-Bradley; replace Microhard with WiFi or cellular; use cheaper enclosures; replace the air knife with a plain manifold; replace the 7075 gantry with steel; drop the folding sensor mast; remove the char test port; replace the RF stage with infrared pre-heating | The Jetson runs the spectral and moisture inference the control loop depends on. Cellular coverage does not exist in the target corridors, which is why Starlink is the backup rather than the primary. A steel gantry saves roughly CAD 5,500 and costs 50 kg and corrosion margin on a hull that crosses water; the option stays open to Phase 1 fabrication review with those figures stated. Infrared pre-heating deposits at the surface exactly as the laser does and carries no volumetric advantage, so its leverage approaches one to one at best. |
| Operational proposals outside the document scope | Fleet management system; operator training for high throughput; rapid deployment and transport handling; burn-boss interface; automated post-treatment inspection; implement predictive velocity control | Each is sound and belongs in the operations manual and the Phase 3 pilot plan rather than in a prototype engineering specification. Predictive velocity control in particular has no object in a step-and-dwell architecture, and returns as a design question only if the creep drive enters. |

The first review returns a net capital reduction of CAD 20,460, a mass reduction of 15 kg, a throughput increase of 10.1 percent on the daytime window and 74 percent on the night window, and one corrected optical fault that no other check in this programme had caught. The rejections carry equal value, because eight of them would have reintroduced an error that an earlier version already paid to find.

## E.4 Second Review, 160 Items

A second review submitted 160 items aimed at daytime output. Item-by-item comparison against Section E.1 through E.3 finds that 152 of the 160 restate proposals the first review already submitted, many appearing three or four times across the four category headings of the second list. Twenty-one distinct proposals appear at least twice within the second list itself. The dispositions in Section E.1 through E.3 therefore carry across unchanged, and this section records only what the second review adds.

Two items were new and both enter the design. The second review also submitted an impact table whose three quantitative claims do not reconcile, and checking the first of them exposed an arithmetic error in v1.5 itself.

Table 41: Second review, items adding to the first.

| Ref | Item | Disposition and reasoning | Section |
| --- | --- | --- | --- |
| S1 | Integrate a data logger | Accepted. Section 18 commits the programme to full treatment logging and no version before this one specifies the hardware that produces it. A shock-mounted 2 TB industrial recorder captures frame counts, dose, position, sensor states, and interlock transitions. Adds 2 kg and CAD 2,400. | 9, 15 |
| S2 | Quick-change cooling interface | Accepted, folded into the modular pod option. Standardised couplings on the source and optics circuits allow field exchange of the chiller and the laser module without breaking the glycol loop. | 19 |
| S3 | Impact table claim: creep drive raises speed to a continuous 0.0115 m per second | Rejected as stated, and it exposed an error in v1.5. The quoted figure equals 0.5 m divided by 43.5 s, which is the unchanged baseline speed, so the table claims a gain while quoting no change. Removing the 4 s advance gives 0.0127 m per second. The true gain is 10.1 percent, and v1.5 printed 9 percent. Section 3.6 and calculation B35 now carry 10.1 percent. | 3.6, B35 |
| S4 | Impact table claim: area rate beyond 12.59 square metres per hour | Rejected. The figure traces to no quantity in this design. Treated belt area accrues at 2.0 square metres per corridor metre times 20.7 metres per hour, or 41.4 square metres per hour. | 3.5 |
| S5 | Impact table claim: night operation raises output 73 percent to 0.26 km | Accepted and already carried. The figure agrees with Section 3.5 within rounding. | 3.5 |
| S6 | Request for daytime throughput improvement | Answered with the throughput sensitivity register in Section 3.7 rather than with a component change. The register names every remaining lever, its worth, and its gate, and it finds that the two largest daytime levers carry no hardware cost and sit outside engineering control. | 3.7 |

## E.5 Third Review, Engineering Addendum

A third review submitted an engineering addendum proposing a wildlife evacuation pre-pass, a high-throughput optical variant, a three-head configuration, and eight supporting changes. Five items enter the design and two are rejected on arithmetic that the addendum itself supplies. The review is the strongest of the three, because its accepted items add capability the design lacked rather than trimming cost from items that already work.

Table 42: Third review, engineering addendum disposition.

| Ref | Item | Disposition and reasoning | Section |
| --- | --- | --- | --- |
| T1 | Ground vibration wildlife pre-pass | Accepted, and re-sequenced. The proposal inserts 20 to 30 s of vibration into the cycle, which grows it from 43.5 to 63.5 s and costs 32 percent of throughput. Mounting the exciter 0.5 m ahead of the scan field, exactly one advance increment, lets it run on the next tile through the current tile's 39.5 s dwell. Each tile receives 39.5 s of vibration, the beam arrives 4 s later, and the cycle does not change. The feature costs 12 kg, 0.20 kW, and CAD 4,200, and no coverage at all. | 11.3 |
| T2 | Tuned mass damper on the gantry | Accepted, and now necessary rather than optional. The evacuation exciter adds a second vibration source alongside the engine firing frequency, so the damper is tuned across the combined 15 to 80 Hz band and gates concurrent operation of exciter and raster. | 6.3 |
| T3 | Phase-change material matched to the coolant setpoint | Accepted, and a correct catch against this document. RT50 paraffin melts at 50 degrees Celsius while the source circuit runs at 22 and derates at 35, so the buffer was inert through the whole operating band. RT28 replaces it for CAD 100. | 6.4 |
| T4 | Supervised autonomy | Accepted as a specified Phase 2 capability with a staged authority model. The hardware is already aboard and the cycle is deterministic. One supervisor across four units cuts daily opex from CAD 738 to CAD 401, a 46 percent reduction and the largest single economic lever in the design. The hardwired abort chain stays hardwired and the operator retains it at all times. | 14, 20 |
| T5 | Carbon audit feature | Accepted. The treatment recorder computes stable carbon per treated metre from delivered dose, treated area, and measured char quality. The design point yields 0.88 tonnes of carbon dioxide equivalent per corridor kilometre, and Phase 3 ecological cores measure the stable fraction directly. | 9, 20 |
| T6 | 100 mm working spot for +35 to +140 percent throughput | Rejected on energy conservation. Dwell equals treated area times dose divided by delivered optical power, and spot diameter appears nowhere in that relation. The proposal's areal dose per frame at the 100 mm spot reads 45.5 kJ per square metre; the correct figure is 4,000 W times 0.97 s divided by 0.25 square metres, or 15.5 kJ per square metre. That single error is the entire factor of 2.9 disagreement the addendum reports between its own two models. Corrected, the raster model returns 37.5 s and agrees with the energy model at 39.5 s exactly as it does at 120 mm. The proposed 13.6 s dwell delivers 207 kJ per square metre against a 600 kJ per square metre design dose. | B47 to B49 |
| T7 | 100 mm spot as a char-quality variant | Accepted in this reduced form. The smaller spot delivers no throughput gain and does change irradiance, interaction time, and terrain sensitivity, so Phase 0 tests it for char character and ignition margin at no hardware cost, since both collimators price identically. | 19 |
| T8 | Three heads with a 15 kW chiller | Rejected on payload and engine load rather than on cooling. The addendum sizes the chiller correctly and then counts only the chiller mass. Adding the third source at 100 kg, the third head at 23 kg, and the wider structure at 28 kg puts payload at 1,309 kg against a 1,200 kg rating. The electrical load reaches 31.8 kW and the crankshaft 36.7 kW, or 89 percent of the engine rating, which exceeds continuous duty and breaks the power take-off. The configuration belongs to a Mk II with a larger engine and a separate generator. | B50 to B52 |
| T9 | High-volume fan replacing the air knife | Rejected on fire safety. The plume geometry argument is sound, and moving a large volume of air gently across an ignition-capable pyrolysis bed oxygenates it. The air knife entrains and deflects through a narrow high-velocity curtain without broad oxygenation. Phase 0 measures plume clearing directly and reopens the question if the knife underperforms. | 6.5 |
| T10 | SWIR plume camera, second wavelength, polygon scanner, linear motor, electric creep | Rejected or deferred as previously dispositioned. The optical emission spectrometer and the triple-infrared flame detector already carry the combustion-onset function a SWIR camera would duplicate. The addendum's own verdicts place the remainder in Mk III, and this document agrees. | Appendix E.3 |

The third review returns the wildlife evacuation pre-pass, which is the most valuable single addition since the power take-off, and two rejections that matter more than most acceptances. The 100 mm optical variant would have propagated a broken energy balance into the throughput tables, which is the exact class of error that v1.3 and v1.4 each existed to correct. The three-head configuration would have exceeded payload by 109 kg and engine rating by 24 percentage points. Both proposals were internally reasoned and both failed on arithmetic their own tables contain, which is the strongest argument this programme has for keeping every governing number in Appendix B where a reviewer checks it.

The second review returns two accepted items, one corrected arithmetic error in v1.5, and a confirmation that the design had, by v1.6, absorbed the improvements available to it from proposal review alone. The programme advanced from there through Phase 0 and Phase 1 rather than through further paper revision, and it is that same v1.6 architecture Appendix C now carries forward into the hot air knife redesign.